3GPP TSG-RAN WG2 Meeting #130					   			 R2-2504109

St. Julian’s, Malta, 19 - 23 May, 2025

Title: 	Discussion on NW-sided model support

Source: 	Huawei, HiSilicon

Agenda item:	8.3.3

Document for:	Discussion/Decision

# Introduction

In the last RAN2 #129b meeting, it has been agreed that the general framework of beam management case in Rel-19 AI for PHY, should be referred as the baseline for the LCM discussion of AI mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Considering the difference between NW-sided and UE-sided model, in this paper, we focus on the NW-sided model, and discuss the framework for RRM measurement prediction case.

# Discussion

## Spatial Domain Prediction

In the historical RAN2 discussion, 6 use cases are proposed for RRM measurement prediction.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

It can be observed that cases 2/3/4 which consider both the temporal-domain and frequency-domain prediction are regarded with high priority. During the simulation discussions, most of the companies provided simulation results for temporal-domain and frequency-domain where high prediction accuracy was achieved.

For spatial domain, it is pending final decision by RAN2, due to the unclarities related to the exact scenario e.g. whether it refers to intra-cell or inter-cell prediction. Currently, in RAN1 AI for PHY, the BM sub-case 1 has thoroughly verified the feasibility of intra-cell beam prediction, which can help to accelerate the beam selection while measuring less beams, i.e. beams in set B. RAN1 research results have provided us with the theoretical basis for intra-cell-inter-beam prediction. In RAN2 AI mobility topic, since we need to consider the L3-triggered inter-cell HO procedures, the prediction should include the beam-level results from neighboring cells.

With beam-level spatial domain prediction, the UE only needs to measure a subset of the SSB beams across the serving cell and neighboring cells, while the quality of the remainder of the beams can be predicted using AIML models. In this case, the cells can be configured with less SSB beams compared to legacy deployments. Thus, both the UE and NW sides can achieve energy saving effect.

Observation 1: Spatial domain prediction is proved to be feasible and can help to realize both UE and NW-side energy saving.

For the gNB to decide which of the beams can be switched off, the gNB needs to consider the needs of the neighbouring gNBs and also the UEs present under its own coverage and the coverage of the neighbours. For RS(es) to be switched off while UE-sided model is utilized, all the UEs under the gNB’s coverage would have to be AIML capable which is unlikely to happen, at least in initial deployments. With NW-sided models on the other hand, one does not have to rely on UE capabilities and can achieve the benefits even in the presence of legacy UEs. Therefore, we believe spatial domain beam-level prediction should be supported for both UE-sided and NW-sided models.

Proposal 1: RAN2 to support spatial domain beam-level prediction for both NW-sided model and UE-sided model.

## Potential Impacts on Measurement Framework

Currently, RAN2 has agreed three cases for the cell level measurement prediction and three sub-use cases for beam level prediction, as can be seen in TR 38.744 [1]:

## 4.2 RRM measurement prediction 3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the current measurement framework, the UE can be configured to report both cell-level and beam-level L3 filtered measured results. Thus, for sub use case 2/5, we observe no impacts on the measurement framework to enable NW-sided inference for RRM measurement predictions.

Observation 2: There are no measurement framework impacts to enable NW-sided RRM measurement inference using sub-use case 2 and sub-use case 5.

For sub-use case 1, 3, 4 and 6, the prediction is based on UE-measured L1 beam-level RSRP results. AI mobility SI restricts the SI scope to L3 triggered HO where the gNB-CU is responsible for the final HO decision. In order for the gNB-CU to acquire the predicted L3 cell-level results, L1 measurement results need to be available at the CU. However, in the current specification, only DU is able to collect UE reported L1 results using a CSI measurement framework and CU can only collect L3 filtered results, which means sub-use cases 1, 3, 4 and 6 are not supported yet.

Observation 3: Changes to the current L1 or L3 measurement framework are needed to support NW-sided inference using sub-use case 1, 3, 4 and 6.

Based on the NW-sided model deployment, we observe that there could be two main candidate methods, as shown in Figure 1.

Figure 1. Candidate options for measurement framework in case 1 and 3.

In option A, the L1 measurement and report configurations are delivered to the UE via RRC signaling. Correspondingly, the UE reports L1 RSRP measurements to the CU via RRC signaling. CU can then execute prediction based on the reported L1 results. This candidate option reuses the L3 measurement framework by adding a possibility to configure the UE to report L1 measurement results. However, it brings additional burden on the UE and increases signaling overhead over SRB1.

In option B, CU can configure the DU to collect UE reported L1 results and forward them to the CU via F1 interface. Based on this, CU can execute the prediction. Moreover, for sub-use case 1 and 4, DU itself can perform L1 RSRP prediction and send the predicted results to the CU. This option brings minor/no impacts on the air-interface and the UE, but involves F1 impacts which would have to be discussed by RAN3.

It was argued before that NW-sided models could be limited to only sub-use case 2 and 5. However, we believe that similarly as for UE-sided model, the network should have a possibility to freely choose the sub-use case to utilize for model inference based on its implementation preferences. At the same time, the specifications impact to support those sub-use cases are very limited. Therefore, we propose the following:

Proposal 2: All analyzed RRM sub-use cases should be supported for NW-sided model inference.

Proposal 3: The following options should be considered to support L1 beam level measurements availability at the gNB-CU:

- Extending L3 measurement framework to support L1 beam level results reporting

- Introduce L1 beam levels forwarding from gNB-DU to gNB-CU

## Assistance information for the NW-side model

Based on the simulation discussion from RAN2#129, the generalization performance for UE speed and frequency was evaluated and the following observations were agreed.

Agreements on generalization over UE speed 1 For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds 2 Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size). 3 For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline Agreements on generalization over frequency 1. Generalization using GC#2 always outperform that of GC#1 2. Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output. 3. Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss. 4. For GC#2 case, the prediction accuracy is acceptable and close to the baseline.

Then in RAN2 #129b, the generalization performance for Uma/UMi was further evaluated and agreed as follows:

Agreements on generalization over UMa/UMi 1 For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that it is generalizable for the studied cases over cell configurations with different deployment scenarios. 2 For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline. 3 For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario than the model trained in UMa scenario when tested in UMi scenario. 4 No more generalization work is expected.

From the above agreements, we can observe that the prediction generalizes well among different UE speeds and UMa/UMi scenarios. For frequency domain prediction, the GC#2 can also achieve close accuracy comparing to the baseline. Therefore, we can observe that to achieve a required level of prediction accuracy, no information other than L1/L3 RSRP measurement results are needed.

Proposal 4: No assistance information from the UE is needed to support NW-side model inference other than L1/L3 RSRP beam/cell level measurement results.

# Conclusion

In this contribution, we have the following observations and proposals:

Spatial Domain Prediction

Observation 1: Spatial domain prediction is proved to be feasible and can help to realize both UE and NW-side energy saving.

Proposal 1: RAN2 to support spatial domain beam-level prediction for both NW-sided model and UE-sided model.

Potential Impacts on Measurement Framework

Observation 2: There are no measurement framework impacts to enable NW-sided RRM measurement inference using sub-use case 2 and sub-use case 5.

Observation 3: Changes to the current L1 or L3 measurement framework are needed to support NW-sided inference using sub-use case 1, 3, 4 and 6.

Proposal 2: All analyzed RRM sub-use cases should be supported for NW-sided model inference.

Proposal 3: The following options should be considered to support L1 beam level measurements availability at the gNB-CU:

- Extending L3 measurement framework to support L1 beam level results reporting

- Introduce L1 beam levels forwarding from gNB-DU to gNB-CU

Assistance information for the NW-side model

Proposal 4: No assistance information from the UE is needed to support NW-side model inference other than L1/L3 RSRP beam/cell level measurement results.

# References

- [1] R2-2501822	3GPP TR 38.744 V0.0.7 (2025-4)	Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR