3GPP TSG-RAN WG2 Meeting #130	R2-2504115

St. Julian's, Malta, 19th – 23rd May, 2025

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on L1 event-triggered measurement reporting

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we provide a comprehensive analysis of event-triggered L1 measurement reports, covering the semi-persistent CSI-RS measurement for LTM, measurement report MAC CE and event evaluation procedures for the key aspects.

2	Discussion

2.1	Semi-persistent CSI-RS measurement for LTM

Agreements on L1 event-triggered MR 1. Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options. • Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch. • Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch. • Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch. Will be revisited next meeting.

In the previous RAN1 meeting, it was agreed that semi-persistent CSI-RS should be supported for candidate cell L1-RSRP measurements. RAN2 also agreed to activate and deactivate the semi-persistent CSI-RS resources for the candidate cell through a MAC CE. The activation of the semi-persistent CSI-RS resource will be performed before the LTM cell switch occurs. However, a question arises regarding that how to handle the activated semi-persistent CSI-RS resources after the LTM cell switch. RAN2 has raised several options to support the subsequent LTM cell switch, and the detailed options are outlined as follows. In this meeting, RAN2 will select one of these options to be agreed.

•	Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch.

•	Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch.

•	Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells(i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

Observation 1: RAN2 needs to discuss how to handle the activated semi-persistent CSI-RS resources after LTM cell switch to support the subsequent LTM, RAN2 will then select one of the options as agreement.

First is to consider the activated semi-persistent CSI-RS resources for the candidate cells. The UE may continue to measure the activated semi-persistent CSI-RS resources after the LTM cell switch. This option could reduce the signalling overhead , as it would avoid the need to send an activation request again for the same SP CSI-RS resources during the subsequent LTM. However, if the SP CSI-RS resources are not required to be activated for the subsequent LTM, this option will introduce extra overhead for both the UE and the network. In this time, the UE would still need to detect and measure the RSRP of the SP CSI-RS, while the network would need to transmit these resources continuously.

Alternatively, if the UE stops measuring the activated semi-persistent CSI-RS resources for the candidate cells after the LTM cell switch, this issue will be resolved. For the subsequent LTM, the new serving cell would only need to activate the SP CSI-RS resources, thereby reducing unnecessary overhead for both the UE and the network.

Agreement For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration. - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2. Send an LS to RAN2 and RAN3 to inform this agreement.

In RAN1, the introduction of semi-persistent CSI-RS resources aims to support CSI acquisition. For the target cell, the UE can continue measuring resources associated with determined CSI report configuration, after the reception of LTM cell switch command, if the CSI acquisition is configured for the target cell. Thus, in alignment with RAN1 agreements, the UE should not stop measuring the activated SP CSI-RS resources for the target cell after the LTM cell switch procedure. In order to simplify the UE behaviour, the activated SP CSI-RS resources for the candidate cell other than the target cell can be stopped. Therefore, the UE should stop to measure the activated SP CSI-RS resources of the candidate cells, and continue to measure those of target cell after the cell switch.

Proposal 1: UE should continue to measure SP CSI-RS resources of target cell after the cell switch and stop to measure the other candidate cells if the target cell is configured with CSI acquisition (Option 3).

2.2	Measurement report MAC CE

Agreement Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

In the last RAN2 meeting, a working assumption was made to introduce four codepoints to represent the four types of reported beams: type 1, type 2, type 3, and type 4. The type 1 beam is the beam that satisfies the entry condition for the event based on TTT and thus triggers the MR MAC CE. The type 3 beam is the beam that has previously triggered an MR MAC CE and remains in the beamsTriggeredList, which is the beam that was reported as a type 1 beam before.

For the network, there is no need to differentiate between type 1 and type 3 beams, as the network considers both types of beams as potential target beams and treats them in the same manner. Therefore, distinguishing between type 1 and type 3 beams does not provide any additional useful information to the network and it increases complexity to the UE's behaviour, as the UE would be required to differentiate between these two beam types. To simplify the UE's behaviour, it would be more efficient to combine type 1 and type 3 beams into a single category. This unified category can be represented as a Type 3 beam, defined as a beam contained in beamsTriggeredList. As a result, three codepoints should be sufficient to represent the following: { Type 3, Type 2, Type 4}.

Proposal 2: There is no need to distinguish between Type 1 and Type 3 beams. Three codepoints should be introduced to represent the following: {Type 3, Type 2, Type 4}.

Agreement In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

Agreement When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

Additionally, it has been agreed that when the current beam is configured to be included in the MR MAC CE, only 7 bits of RSRP should be added and no RSRI is required. What’s more, it has been agreed that a uniform format should be applied to all reported beams in the MR MAC CE, which also includes the current beam. However, for reported beams other than the current beam, both the SSBRI/CRI and the beam quality (RSRP) must be included in the MAC CE. These two agreements are in conflict with each other. To resolve this issue and ensure that all reported beams follow a uniformed format, the RSRI of the current beams should also be included. In legacy L3 measurement reports, all reported beams share the same reporting structure, and the cell ID of the serving cell is explicitly included in the measurement report. A similar approach can be adopted to follow the legacy behaviour. Thus, the agreement should be updated to the following: In the MR MAC CE, a uniform format should be adopted for all reported beams, except for the current beam.

Proposal 3: In the MR MAC CE, adopt a uniform format for all reported beams, including the current beams.

2.4	Event evaluation

RAN2#127-bis meeting Agreements on L1 event triggered MR 8. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

For the agreement in RAN2#127-bis, the structure of the Rel-18 LTM-CSI-ReportConfig should be reused as much as possible. In this legacy structure, there is a parameter called spCellInclusion, which can only be configured in some specific scenarios. For example, during a subsequent LTM cell switch, if the serving cell is configured as one of the candidate cells and spCellInclusion is set to true, the measurement results for the serving cell should be included in the L1 measurement report. Thus, the configuration of the serving cell as one of the SpCells in the LTM candidate configuration can also be applied in the Rel-19 LTM-CSI-ReportConfig.

spCellInclusion This field indicates whether the UE shall include a L1 measurement report associated to the current SpCell. This field can only be configured if the current SpCell is configured as an SpCell of an LTM candidate configuration and the LTM-CSI-ResourceConfig IE associated to the LTM-CSI-ReportConfig IE includes resources for the current SpCell.

This is also aligned with the proposal above that all the beams adopt a uniform format for beam measurement reporting. Even for beam in the current cell, current beam can be reported with RSRI.

Proposal 4: Follow R18 LTM to include SpCell in the LTM candidate configuration if the UE is configured to current beam.

Additionally, if the serving cell is configured as an SpCell in the LTM candidate configuration, the LTM-CSI-SSB-ResourceSet will include SSBs/CSI-RSs from both the serving cell and the other candidate cells. Since all the candidate beams, including those from the serving cell and candidate cells, are combined into one LTM resource set, the UE needs to perform event evaluation for all these beams in the LTM-CSI-SSB-ResourceSet. Especially for EventLTM3, EventLTM4, and EventLTM5, the UE may compare the current beam with the beams from the serving cell. Furthermore, even if the serving cell’s beams satisfy the event conditions, they could also trigger the event reporting procedure for those LTM events. Apparently, these scenarios should be avoided. Therefore, the beams of the serving cell should not undergo the event evaluation procedure. This action can be easily achieved with the current LTM resource set structure, as the UE can distinguish the serving cell's beams in the RRC configuration using the ltm-CandidateIdList.

Proposal 5: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3/4/5.

2.5	Other open issue list

Agreement FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

Agreement For the RS type determination for event triggered reporting with event LTM2, - Alt 1-2: At least one candidate RS shall be configured

In the previous RAN1 meeting, it was agreed that at least one candidate RS should be configured for event-triggered reporting with event LTM2. The main reason is that, in the previous RAN1 meeting, it was also agreed that the measurement RS of the serving cell beam is determined based on the candidate beam to ensure the same RS type, i.e., the RS for the current beam of the serving cell should either be the same as or QCLed with the QCL RS of the indicated TCI state.

Observation 2: Regarding the RS type determination, RAN1 agreed that at least one candidate RS shall be configured for event triggered reporting with event LTM2.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |

According to the last RAN2 meeting, it was discussed whether the beam information of the candidate cells should be included in the MR MAC CE, particularly for the measurement report triggered by event LTM2. Since at least one candidate RS must be included in the measurement report configuration for LTM2, the UE should measure these candidate RS. Therefore, it is unnecessary to include only the current beam information in the MR MAC CE. To maintain consistency with other LTM events, it would be better for the UE report both current beam and candidate beam information in the single MR MAC CE, thus the MAC CE format can be uniform for all MR reports triggered by LTM events.

Proposal 6: (MAC-5) The measurement results of the candidate beams should also be included in the MR triggered by LTM2.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following observations and proposals:

Semi-persistent CSI-RS measurement for LTM

Proposal 1: UE should continue to measure SP CSI-RS resources of target cell after the cell switch and stop to measure the other candidate cells if the target cell is configured with CSI acquisition (Option 3).

Measurement report MAC CE

Proposal 2: There is no need to distinguish between Type 1 and Type 3 beams. Three codepoints should be introduced to represent the following: {Type 3, Type 2, Type 4}.

Proposal 3: In the MR MAC CE, adopt a uniform format for all reported beams, including the current beams.

Event evaluation

Proposal 4: Follow R18 LTM to include SpCell in the LTM candidate configuration if the UE is configured to current beam.

Proposal 5: If the serving cell is configured as an LTM candidate cell in the LTM candidate configuration, for this candidate cell, there is no need to perform event evaluation for EventLTM3/4/5.

Other open issue list

Proposal 6: (MAC-5) The measurement results of the candidate beams should also be included in the MR triggered by LTM2.

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #129-bis chairman notes;

- RAN1 #120-bis chairman notes;

- TS 3GPP 38.331 V18.5.0