3GPP TSG-RAN WG2 #130	R2-2504126

# St. Julien, Malta, 19th – 23rd May 2025

Agenda Item:	8.6.1

Source:	Ericsson

Title:	RRC open issues list for mobility

Document for:	Discussion

# 1	Introduction

Here below is provided the list of remaining FFSs according to the RRC running CR in [1] for LTM and CLTM that need to be solved

# 2	Discussion

## 2.1	RRC-1

Editor’s Note: FFS whether additional text for security key change in LTM (at the MCG and SCG) is needed.

Rapporteur comment: This issue can be solved in the discussions for the running CR and no contribution from companies is needed.

## 2.2	RRC-2

Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

Rapporteur comment: Whether this is an issue need to be clarified, but probably good to discuss this based on companies’ contributions (if companies think that current text is not okay).

## 2.3	RRC-3

Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

Rapporteur comment: Current procedural text leaves this case to the UE implementation (similar to CHO), so the proposal is to keep current text as it is and delete the FFS.

## 2.4	RRC-4

Editor’s Note: How to release ltm-ConfigNRDC is FFS.

Rapporteur comment: This is a purely stage3 issue and the plan is for the rapporteur to propose a solution on how to address this. No contribution should be needed from companies.

## 2.5	RRC-5

Editor’s Note: FFS how report configuraiton for CSI acqusition should be configured. Pending with further R1 progress

Rapporteur comment: This issue is related to how RAN1 decides to design the CSI-RS measurements for LTM. We can wait for their RAN1 parameter list.

## 2.6	RRC-6

Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution.

## 2.7	RRC-7

Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

Rapporteur comment: This is something that we never discussed. Current Running CR re-use the list of sk-counters also for the case of inter-MN LTM (to support subsequent LTM) so our suggestion is to keep current text.

## 2.8	RRC-8

FFS if fast failure recovery with different Rel-19 IDs is allowed.

Rapporteur comment: This is an open issue from last RAN2 meeting and, even if we don’t support the case mentioned in the FFS, things should work fine anyway. However, our suggestion is to discuss this based on companies’ contribution.

## 2.9	RRC-9

FFS about when to use “LTM” and “CLTM”

Rapporteur comment: A conditional LTM candidate configuration is just an LTM candidate configuration for which execution conditions have been provided. Existing procedure such as the LTM cell switch is exactly the same for LTM and CLTM. Because of this, there is no need to repeat in specs “LTM and CLTM” because this create more confusion than clarity. The term “CLTM” should only be used for something that applies explicitly to conditional LTM.

## 2.10	RRC-10

FFS whether we still need the RS type configuration for LTM2. Note that R1 has agreed on the following in R1#120bis: For the RS type determination for event triggered reporting with event LTM2, Alt 1-2: At least one candidate RS shall be configured

Rapporteur comment: About this FFS, the RAN1 agreement implies that for LTM2 we would need to configure anyway at least one candidate cell RS within the LTM CSI Resource Configuration. Since this is within the RAN1 domain, the proposal is to simply follow what RAN1 has agreed.

## 2.11	Comments on open issues

Companies are free to comment on the different open issues and whether more issues should be added to the list.

| Company | Issue | Comment |
| --- | --- | --- |
| DOCOMO | It is unclear whether the LTM-Candidate including execution condition can be also used as NW-triggered intra-CU LTM. (i.e., whether we allow to use LTM-Candidate including execution condition for both C-LTM and NW-triggered intra-CU LTM) |  |
| Nokia | RRC-3 | In case of CHO, situation is a bit different since independent of the target cell that UE selects, same switching delay is expected (since UE will anyway perform RACH). On the other hand, for CLTM both RACH and RACH-less switches are possible, and they have different requirements. In our view, UE should prioritize candidate cells for which early synchronization is performed, so both goals of robustness and fast switching are fulfilled. This would also align with the RAN4 agreement that (e.g., when early TCI state activation is performed, UE is required to performed measurements for cells for which TCI states are activated). Allowing the UE to switch to any cell, would complicate NW implementation (since NW does not expect that UE would perform condition evaluation for cells where it is not required to do so). |
| Nokia | RRC-6 | Revert back to source configuration is needed for triggering re-establishment towards cell which is not candidate cell of LTM. As this is the first step UE execute on T304 expiry prior to cell selection and UE does not know the cell for re-establishment at this time, we suggest to keep the Rel-18 behaviour related to revert back to source config. We have also provided further analysis in our contribution |
| Nokia | RRC-7 | SK-counter list is meant for Inter-SN LTM. For Inter-MN LTM RAN2 has already agreed to use the SK-counter value outside this list to be used for generation of new S-KGNB whenever cell switch is triggered across Inter-CU candidates( candidates with Rel-19 ID). So this list should not be referred for Inter MN LTM |
| Nokia | RRC-8 | Agree that the issue is under discussion in RAN2. However in our view this FFS needs clarification on whether UE should try recovery for Inter-CU candidate or not. Because current flag for Fast recovery is applicable for all candidates. Some spec changes are needed if there is restriction concluded in RAN2 meeting. |

# 3	Conclusion

According to what has what has been discussed in section 2:

RAN2 to solve the described open issues.

# References