3GPP TSG-RAN WG2 Meeting #130	R2-2504134

St. Julians, Malta, 19th – 23rd of May 2025

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues for MBS in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

At RAN2#129 meeting (February 2025) the following decisions have been made [4]:

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In addition to the aforementioned agreements, RAN2 has also sent the LS to SA2 and SA4 [14], wherein it has been asked if Intended Service Area (ISA) can be conveyed in Service Announcement:

# 2. Actions: # To SA WG 4: # RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

In RAN2#129bis (April 2025) the following agreements have been made [16]:

- RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) - If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH - If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. - The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE. - UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. - FFS whether and how the association between intended service area and frequencies is acquired.

This paper presents our further view on what remains to be addressed for MBS Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Intended Service Area (ISA) signalling

The following has been agreed at RAN2#129 [4]:

1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

The high-level structure and relationship as shown in Figure 1 has been agreed.

Figure 1. ISA and MBS service high-level structure and relationship.

As can be seen, the MBS service definition is given in MCCH (legacy behaviour) and ISA ID points to a new SIB where ISA geographical details are provided. Multiple services can be present within a single service area and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

Observation 1: Multiple services can be present within a single intended service area (ISA) and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

When inspecting the NR RRC [7], it can be found that MCCH comprises MBSMulticastConfiguration which can have a list of MBS sessions (i.e. MBS-SessionInfoListMulticast IE). Thus, under each of such sessions (i.e. MBS-SessionInfo) there needs to be an identifier pointing to a specific service area or areas defined in the new SIB.

Proposal 1: Insert a list of service area IDs under each MBS-SessionInfo provided in MCCH.

Approach suggested in Proposal 1 seems to be aligned with what NR RRC Rapporteur has included in the running CR:

MBS-SessionInfoExt-r19 ::= SEQUENCE { mbs-SessionAreaList-r19 SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19 }

## 2.2	ISA in Service Announcement

Intended Service Area in Service Announcement has been discussed in RAN2#129 based on [15]. The main motivation used by the authors of [15] was to allow the UE to skip MCCH reading if the UE determined (based on Service Announcement) that it is outside of the ISA. An LS to SA4 and SA2 has been sent [14] asking about the feasibility of such scheme in Release 19 timeframe. However, at the time of drafting this TDoc, no feedback from SA2 or SA4 has been received.

While the approach outlined in [14] could work, we do not think it is essential to involve other working groups (SA WGs in particular) in addressing this objective of RAN-centric work item.

While the approach outlined in [14] could work, other solutions were on the table which did not require other working groups. One disadvantage of the proposed approach is that it introduces new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF). As such, RAN should not rely on such solutions and instead focus on RAN-centric ones.

Observation 2: Including ISA in Service Announcement is one potential option, but not the primary solution RAN2 considers in Release 19.

Additionally to the AF and MBSF impact, the proposed solution might require a coordination between the operator and the application function if the application shall know about ISA. Eventually, AF does not need to know that certain service is transmitted via NTN RAN and not the “ordinary” TN.

Observation 3: ISA in Service Announcement will results in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Thus, in our view the gains of ISA in Service Announcement are too small compared to the pain of introducing this.

Proposal 2: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 response to R2-2501583.

## 2.3	MBS Interest Indication (MII)

At RAN2#127bis the following has been discussed and not resolved:

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. Come back in the next meeting

In section 5.9.4.2 of [7] the following text can be found:

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell.

As can be seen, the MII might be sent when UE enters or leaves the broadcast service area and this is already supported by NR RRC [7].

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Thus, agreeing to MII being sent when the UE enters or leaves the broadcast service area requires no changes MBS II in NTN. Furthermore, the behaviour described in [7] is not binding (the term used in the procedure is “UE may initiate”), so in our view this issue is not worth spending additional time. Note that the MII feature can be used by the UE in case UE cannot find the MBS broadcast service that it is expecting in a cell. A gNB can decide whether to handover the UE to a possible neighbor gNB/cell providing MBS broadcast service UE has interest in. Thus, as has been indicated at RAN2#127bis, it is a UE implementation issue, and a UE can decide when to send the MII (which could be somehow related to the intended service area).

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

## 2.4 	Changes to SIB21 for Service Continuity

Service continuity has been discussed at RAN2#129bis via offline thread (summary in [17]). The following last-minute agreement has been taken based on the aforementioned offline:

- UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. - FFS whether and how the association between intended service area and frequencies is acquired.

The discussion has been continued in the post-meeting e-mail thread [19] and the following was suggested there to clarify the applicable scenarios:

Proposal 3 For the service continuity discussion in MBS NR NTN, RAN2 takes the following scenarios as a baseline: - Multiple frequencies, served by the same satellite, illuminating the same geographical area. Each frequency provides a different MBS service. These services are not being broadcast in the serving cell. - Multiple frequencies, served by the same or different satellites, but not covering the same geographical area (they are neighbour cells). Each frequency provides a different MBS service. These services are not being broadcast in the serving cell.

During RAN2#129bis it has been argued [17] that the mapping between FSAI, MBS session ID (TMGI) and ISA should be straightforward, as RAN3 has already agreed to include those components in the AMF to NG-RAN node signalling.

Observation 6: As suggested in R2-2503057 during RAN2#129bis, RAN3 might have already defined all components in AMF to NG-RAN signalling which enable the mapping between FSAI, MBS session ID (TMGI) and ISA.

However, if intended service area is provided in SIB21 for each session or per FSAI, this would somewhat change/extend the purpose of SIB21. SIB21 is defined as follows [7]:

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). SIB21 information element -- ASN1START -- TAG-SIB21-START SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 } MBS-FSAI-r17 ::= OCTET STRING (SIZE (3)) -- TAG-SIB21-STOP -- ASN1STOP

Thus, introducing the service area information to SIB21 does not match its main purpose (i.e. frequency mapping to FSAI). FSAIs are configured per gNB or per cell. If these are to be configured per intended service area, this requires lots of changes to core and service announcements.

It also needs to be underlined that FSAI is used to improve cell reselection while it is not clear cell reselection enhancements are clearly in the scope of the WI for this objective.

Observation 7: FSAI is used to improve cell reselection while it is not clear cell reselection enhancements are clearly in the scope of the WI for this MBS objective.

Overall, the benefits of FSAI per ISA are limited, if any, while it might have a large specification impact, not only in RAN2. Thus, we suggest the following:

Proposal 4: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

# 3	Conclusion

This document has made the following observations:

Observation 1: Multiple services can be present within a single intended service area (ISA) and one service can be present in multiple ISAs. (i.e. many-to-many relationship, as agreed in RAN2#129).

Observation 2: Including ISA in Service Announcement is one potential option, but not the primary solution RAN2 considers in Release 19.

Observation 3: ISA in Service Announcement will results in new requirements to the application function (AF) and also to Multicast/Broadcast Service Function (MBSF).

Observation 4: ISA in Service Announcement might require a coordination between the operator and the application function if the application shall know about ISA.

Observation 5: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Observation 6: As suggested in R2-2503057 during RAN2#129bis, RAN3 might have already defined all components in AMF to NG-RAN signalling which enable the mapping between FSAI, MBS session ID (TMGI) and ISA.

Observation 7: FSAI is used to improve cell reselection while it is not clear cell reselection enhancements are clearly in the scope of the WI for this MBS objective.

and proposed the following:

Proposal 1: Insert a list of service area IDs under each MBS-SessionInfo provided in MCCH.

Proposal 2: RAN2 does not consider including ISA in Service Announcement, irrespective of the SA4 response to R2-2501583.

Proposal 3: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

Proposal 4: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- R2-2501333	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- S1-244857 Reply LS on requirements for ETWS primary notification 3GPP TSG SA WG WG1 Meeting #108 Orlando, FL, USA, 18th – 22nd November 2024

- S1-244753 Warning notification with geofencing for ETWS over satellite access (CR to TS 22.268) 3GPP TSG-SA1 Meeting #108	 Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

- 3GPP TS 38.331 NR RRC, v18.4.0, December 2024

- R2-2407823 LS on requirements for ETWS primary notification 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, August 19th – 23rd, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.

- R3-245844 LS on Supporting MBS broadcast service for NR NTN 3GPP TSG RAN WG3 125-bis Hefei, China, 14 - 18 October, 2024

- R2-2501583 LS on Inclusion of NTN intended service area in the Service Announcement 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- R2-2501306  Support for broadcast services in NR NTN 3GPP TSG-RAN WG2 Meeting #129 Athens, Greece, Feb 17th -21st, 2025

- R2-2502983 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #129bis	Wuhan, China, Apr 7th – 11th, 2025

- R2-2503057 Report of [AT129bis][304][R19 NR NTN] Service continuity 3GPP TSG-RAN WG2 Meeting #129bis	Wuhan, China, Apr 7th – 11th, 2025

- NR RRC Running CR for Rel-19 NTN

- Remaining RRC open issues for NR NTN Rel-19