3GPP TSG-RAN WG2 Meeting #130	R2-2504136

St. Julians, Malta, 19th – 23rd of May 2025

Agenda item:	7.0.2.22

Source:	Nokia

Title:	Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell)

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM. Our focus is predominantly on the granularity definition of pdcch-RACH-AffectedBandsList over which RAN2 has liaised with RAN4. Furthermore, we outline the issue of LTM recovery when the selected cell is the source cell.

# 2	Discussion

## 2.1 	On the granularity definition of pdcch-RACH-AffectedBandsList-r18

In February 2025 (RAN2#129) RAN2 could not conclude on the unified interpretation of pdcch-RACH-AffectedBandsList-r18 capability [1] and has sent the LS to RAN4 [2], asking RAN4 for a further guidance on this. More specifically, it has been asked which of the following understandings is correct [2]:

[…] However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. […]

In April 2025 (RAN4#114bis) RAN4 has discussed the topic and replied to RAN2 [3]. The following response has been outlined in [3]:

RAN4 would like to thank RAN2 for the LS on granularity definition of pdcch-RACH-AffectedBandsList-r18. RAN4 discussed the question from RAN2 and reached the following agreement: - "Understanding 2" in the LS (R2-2501392 / R4-2503115) is what RAN4 intended. And the band in which the PDCCH order is received has no impact on whether and on which band the interruption may occur.

As can be seen, RAN4 has confirmed Understanding 2 is what RAN4 had in mind when defining this capability.

Observation 1: RAN4 confirms the source band of the band pair represents a specific source band that encounters an interruption or does not encounter it. It has nothing to do with the band wherein the PDCCH order was received.

In order to clarify the behaviour, RAN4 has provided an example [3]:

For example, if a UE supports a PDCCH order RACH to an LTM candidate cell in bands A, B, C, D, E, and F, and supports CA with the band combinations of A+B+C and A+B+D+E, the UE may report this capability for the band combination of A+B+C as shown in the table below. Target LTM band for PDCCH order RACH; Capability for the band combination of A+B+C (x, y, z) = Interruptions on the active serving cell in bands A, B, and C due to the PDCCH order RACH. A; (No, No, No). B; (No, No, No). C; (No, No, No). D; (No, No, Yes). E; (No, No, Yes).

As can be noticed when inspecting the aforementioned table, UE supporting CA in the following band combinations:

- A+B+C

- A+B+D+E, (C, is not supported)

indicates it may cause interruptions on band C when performing the RACH attempt in the target cell which is in band D or E. The reason behind this is that bands A or B are in the carrier aggregation combination with both C (i.e. A+B+C) and D+E (i.e. A+B+D+E), as listed above. Thus, the transmission of RACH in band D or band E does not cause interruption to source band A or band B (due to the existence of A+B+D+E CA combination), but it creates an interruption in band C (as C is combined only with A and B (A+B+C), but not with D or E).

Observation 2: Interruption is indicated for the bands which are not a part of CA band combination with the band towards which RACH is attempted.

As per [4] the capability is described in the following way:

| pdcch-RACH-AffectedBandsList-r18 Indicates whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands. Each "source-target" pair indicates the band pair between the target band for RACH transmission and band under UE's current band combination. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

The intended behaviour for this capability is as follows:

- NW receives pdcch-RACH-AffectedBandsList-r18 capability, reported by the UE. This is a sequence of bands with the value interruption/noInterruption  indicated per each band

- NW is also aware of UE’s CA combinations

- As a result, it is known the UE can cause interruptions only in those bands which are not a part of any CA band combination with the band for PDCCH-ordered RACH attempt.

It could make sense to update the aforementioned field description by clarifying the following:

pdcch-RACH-AffectedBandsList-r18 Indicates whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands, as specified in TS 38.133 [5]. Each "source-target" pair indicates the band pair between the target band for RACH transmission and source band under UE's current band combination (i.e. not necessarily the band wherein PDCCH order has been received).

Annex A comprises the actual TP  to TS 38.306. We suggest to agree the changes outlined in Annex A.

Proposal 1: Adopt the changes to TS 38.306 related to pdcch-RACH-AffectedBandsList-r18 which have been shown in Annex A.

## 2.2 	On LTM Recovery to the source cell

The fast recovery was introduced for CHO to allow the UE to recover directly using the conditional configuration if the selected cell during re-establishment is one of the prepared CHO candidates. This was further extended for SCPAC and LTM in Rel-18 for fast recovery. In Rel-18 LTM this functionality is enabled through attemptLTM-Switch parameter in LTM-Config information element. In some recovery scenarios it is possible that the best cell selected for recovery can be the current/last serving cell (i.e. source cell). In this scenario, if there is a valid candidate configuration available for this cell, the LTM recovery procedure is triggered instead of re-establishment procedure.

Observation 3: In Rel-18 fast recovery procedure, the fast recovery is triggered even if the selected cell is same as the current serving cell (i.e. source cell).

Abovementioned UE’s behaviour may lead to the following issue for the recovery procedure handling at the network. The recovery procedure is triggered with the serving cell configuration prepared for subsequent LTM at the time of LTM preparation (i.e. not with the latest configuration of the source cell that was used directly before the failure). In legacy recovery procedure in this scenario UE starts the re-establishment with the latest configuration and there are no issues such as the use of earlier configuration towards the UE.

Observation 4: LTM recovery to the source cell will lead to the use of serving cell’s configuration from the candidate configuration list, not the latest source cell configuration that was used directly before the failure.

Moreover network also does not expect to handle the recovery to the same serving cell using its own candidate configuration prepared for LTM.  Hence the recovery to serving cell using LTM configuration may cause some confusion at NW for this scenario.  To avoid additional changes at NW to handle such scenario we propose that Fast recovery should not be applicable if the selected cell for recovery is same as current serving cell.

Proposal 2: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is same as current serving cell.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: RAN4 confirms the source band of the band pair represents a specific source band that encounters an interruption or does not encounter it. It has nothing to do with the band wherein the PDCCH order was received.

Observation 2: Interruption is indicated for the bands which are not a part of CA band combination with the band towards which RACH is attempted.

Observation 3: In Rel-18 fast recovery procedure, the fast recovery is triggered even if the selected cell is same as the current serving cell (i.e. source cell).

Observation 4: LTM recovery to the source cell will lead to the use of serving cell’s configuration from the candidate configuration list, not the latest source cell configuration that was used directly before the failure.

Proposal 1: Adopt the changes to TS 38.306 related to pdcch-RACH-AffectedBandsList-r18 which have been shown in Annex A.

Proposal 2: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is same as current serving cell.

# References

- R2-2501331 Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, February 17th  – 21st 2025

- R2-2501392 LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18 3GPP TSG RAN WG2 Meeting #129 Athens, Greece, 17th – 21st Feb. 2025

- R4-2504940 Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18 3GPP TSG-RAN WG4 Meeting #114bis Wuhan, China, 7th – 11th April 2025

- 3GPP TS 38.306 User Equipment (UE) radio access capabilities, V18.5.0 (2024-03)

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.5.0 (2024-03)

# Annex A – update of pdcch-RACH-AffectedBandsList-r18 capability

| pdcch-RACH-AffectedBandsList-r18 Indicates whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands, as specified in TS 38.133 [5]. Each "source-target" pair indicates the band pair between the target band for RACH transmission and the source band under UE's current band combination (i.e. not necessarily the band wherein PDCCH order has been received). The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |