3GPP TSG-RAN WG2 Meeting #130	R2-2504137

St Julians, Malta, May 19 – May 23 2025

Agenda item:	8.6.2

Source:	Nokia

Title:	On remaining issues for inter-CU LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the baseline procedure for inter-CU LTM support and identify the related key issues.

# 2	Discussion

We analyze the following key issues related to Inter-CU LTM and provide our proposals on way forward on these issues in this paper.

- Fast Recovery for Inter CU LTM

  - Detection of failure due to NCC mismatch

- Remaining issues related to non-DC LTM

  - Support for non-subsequent LTM

  - Shared usage of configured grant for Inter-CU LTM

- Remaining Inter-CU DC LTM issues

  - Analysis on MCG-LTM with SCG addition scenario

  - Inter-SN SCG LTM configuration structure and associated impacts for co-existence with MCG-LTM

## 2.1	Fast Recovery for Inter CU LTM

Issue 1: Candidate nodes for Fast recovery after LTM Switch Failure

One of the open issue related to Inter-CU Fast recovery is whether Inter-CU LTM recovery after Inter CU LTM switch failure can be supported towards all the candidate nodes.

As the starting point for this analysis RAN2 need to have common understanding of updating the security configuration of candidates after the cell-switch.  In classic conditional handover cases all the candidate nodes are prepared with the target K-GNB to be used for the candidate configuration. This early preparation is needed as the actual candidate node for handover is not known in case of CHO.  The situation is similar in LTM where switching decision is made at CU and CU only knows at the time of switching. Updating candidate configuration at this point will introduce additional delay in switching. The LTM-Config-Update procedure triggered towards other candidate cells after LTM switch (as indicated in stage-2 running CR) can also include security configuration. In this case the latest security configurations to be used for further LTM switch or recovery is available at all candidate nodes.

Observation 1: Update of security configuration at all candidate nodes can be easily integrated in current procedure and brings the benefit of recovery possibility at all candidate nodes for Inter-CU switch failure.

If the network intends to avoid the update of all candidate configurations with new security keys and selectively activate the security keys for the candidate node at the time of switching it is possible to support this case with proposed LTM Configuration Update procedure in RAN3 signaling. However, this can be an alternative option only if the NW wants to delay the configuration update for security keys until the actual switching. Considering the LTM Configuration Update as class 2 procedure, the switching from DU need to wait until the configuration update response.

Observation 2: Update of security configuration at the time of switching can be second option which involves additional delay in switching.

Based on the above observations, it is possible to update all candidates prior to LTM switching with new keys in RAN3 procedure. Hence, we propose to remove the FFS on whether Inter-CU recovery to be supported towards all the candidate gNBs.  .

Proposal 1: Remove the FFS and Confirm that Fast recovery to any candidate CU is supported after LTM switch failure is supported.

Issue 2: Reverting back to source configuration for Inter-CU Failure

For Rel-18 LTM the UE reverts back to source configuration on LTM switch failure with the exception on the PDCP state variables for SRB if the Fast recovery is enabled. This exception is introduced to avoid security re-keying issue for the RRC Reconfiguration complete message sent for recovery. During RAN2-129bis for re-establishment, companies suggested not to revert back the source configuration for Inter-CU scenarios. As the revert back to source configuration is needed for possible re-establishment to any cell this specific behaviour needs to continue with Inter-CU LTM also. However, the exception related to PDCP state variable can be maintained even for Inter-CU recovery scenario. In case of Inter-CU recovery, the target node will receive PDCP SN higher than the last PDCP SN number used at source cell and need to handle this scenario.

Proposal 2: On Inter-CU LTM switch failure UE revert back to source cell configuration except the PDCP variables for SRB2. FFS whether additional Inter node signaling needed to handle the PDCP SN difference.

Issue 3: Detection of LTM switch failure due to security issues

In RAN2-129bis RAN2 has agreed to include the NCC value used for security key generation for Inter CU LTM in the cell switch command. However, the potential risk of the NCC value alternation by fake BS resulting in handover failure still remains as indicated in SA3 LS. In case of NCC compromised due to attacks the LTM failures will look similar to any other LTM failures as the target node will not be able to identify the cause of failure in this scenario. The impact of the problem differs based on whether RRC message is only ciphered or also integrity protected.

Observation 3: The LTM switch failure due to alternation of NCC  of cell switch command is not known to the NW for Inter-CU cell-switch scenario.

For the NW to aware of the cause of the LTM failure or LTM failure recovery, the UE can include the NCC value used for key generation in the last LTM switch in the LTM Recovery message or RRC-Re-establishment complete message. This approach is similar to inclusion of used SK-counter value in the RRC Reconfiguration complete in SCPAC for the network to know the key mismatch situation.

Proposal 3: Inclusion of applied-NCC value in LTM Recovery message is supported.

Issue 4: Fast Recovery after RLF prior to first LTM switch

For the Fast link recovery for RLF, RAN2-129bis made an agreement that Fast recovery is possible if the source and target cell have same Rel-19 ID. This also mean that in case of RLF recovery is possible only for Intra-CU candidate cells. This restriction is introduced where it is not possible for UE have have NCC value for other CU prior to LTM Switch. However if the RLF occurs prior to the first LTM switch all the candidate nodes are prepared with K-GNB value based on current NCC value.

Observation 5:  Prior to first LTM switch all the candidate nodes are prepared with Target security key configurations based on the current NCC value.

In this case, eventhough the cell switch command carry the NCC value it will be the same value as the one stored at UE. Hence if the UE experience RLF prior to cell switch in this case, it can make use of the current NCC value to generate target security configuration and recovery can be successful.

If RLF is triggered for this case the UE can make use of its current NCC value for generating keys for target cell even if Rel-19 ID is different from source cell.

Observation 6: In case of RLF prior to first LTM switch the fast recovery is possible at UE using its current NCC value.

Based on the above observations, we propose that the Inter-CU Fast recovery is allowed for RLF for the first LTM cell-switch after the preparation.

Proposal 4: Fast Recovery to Inter-CU candidates is supported for RLF prior to first LTM switch.

Issue 4: SK Counter list usage for Inter-MN LTM

RRC Running CR includes following EN.

Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

In RAN2-129bis, it is agreed that the SK-counter value configured within candidate configuration is used for S-KgNB generation during Inter-CU LTM switch where master key is regenerated. Also Inter-MN LTM and Inter-SN LTM can not be configured simultaneously in Rel-19. Hence we don’t see need for using SK-counter list for Inter-MN LTM. We propose to remove the FFS.

Proposal 5: SK-counter list configured for LTM candidates are only used for Inter-SN LTM. FFS in above EN is removed.

## 2.2	Remaining Issues for non-DC LTM

.2.2.2	Support for Non-Subsequent LTM

In RAN2#125bis the following were agreed with respect to subsequent LTM

Agreements on scenarios: - RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. - Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. - UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

The Inter-CU LTM involves security key change and also PDCP re-establishment as additional steps. Subsequent LTM is beneficial for fast switching without any higher layer impacts. Usage of subsequent LTM in Inter-CU is required for cases where a high number of ping-pongs is expected. For instance, consider generic movement, wherein the UE may move between different cells without any pattern. In this case, the user may or may not go back to the same cell. In such cases, preparing multiple candidates would likely lead to inefficient resource usage.

However, in some scenarios the NW may need to support one time mobility towards some candidates and subsequent mobility across other candidates. This case is similar to support of CPAC and subsequent CPAC together in single configuration. In such cases non-subsequent LTM towards selected candidate is needed to avoid maintaining the other configurations for which no further subsequent mobility is expected.  This support becomes more relevant for Inter-CU LTM where there could be some issues in candidate CU supporting reference configurations and exchange of information related to Early-sync procedures. It is better to configure non-subsequent mobility towards these candidate cells

Observation 7:  In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 6:  Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

### 2.2.3	Support of Configured Grant-based RACH-less LTM

In RAN2#126, the following agreement was made on the support of the first UL transmission for RACH-less inter-CU LTM

RAN2#126 Agreement: Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

Compared to dynamic grants (DGs), configured grants (CGs) enable faster access to the target cell, as the UE does not need to monitor PDCCH to receive the UL grant and can directly perform its first UL transmission on one of the valid occasions of the provided CG.

Nevertheless, a known issue of the use of CGs in LTM is the potential excessive resource reservation to support a single RACH-less LTM switch. For each UE prepared for RACH-less LTM and for each candidate cell, a CG must be included in the candidate’s RRC configuration provided to the UE during the preparation phase. Therefore, the PUSCH resources of these grants are reserved for a long period of time stretching from the preparation phase to the execution of one or multiple cell switches and they may never be used, if the UE does not switch to the corresponding cell. The issue may be even more pronounced in inter-CU LTM, as it cannot be solved or mitigated with intra-gNB resource scheduling optimizations.

Observation 8: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

. To optimise the PUSCH resource reservation issue,  the probabilistic nature of the use of the provided PUSCH resources can be exploited in a similar way as it is done for CFRA resources. PUSCH resources can be shared by multiple UEs, as it is unlikely that they switch to the same candidate cell at the same time. Still, it is possible that UEs sharing the same PUSCH resource will use the same CG occasion for their first UL transmission to the candidate. Therefore, coordination of the use of shared PUSCH resources is required. As cell switch execution is controlled by the source node, shared PUSCH resources can be provided by the candidate node to the source, which in turn can coordinate their use during cell switch.

Proposal 7: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

## 2.3	Remaining issues for DC LTM

### 2.3.1		Discussion on RRC running CR

In the latest version of RRC running CR, the following topics in our view are still fully not clear, and need further discussion:

- For NR-DC scenarios, when ltm-Config for Inter-SN SCG is included in ltm-ConfigNRDC, how the UE maintains this configuration needs further clarification.

- The handling of reference and candidate configurations for Inter-CU SCG LTM also requires further clarification.

Handling of ltm-ConfigNRDC at the UE

In Rel-18 LTM, for NR-DC, UE receives two independent ltm-Config, one associated with MCG and the other associated with SCG, and UE maintains two independent variables to store ltm-Config associated with MCG and SCG.

For Rel-19 inter-CU SCG LTM, one approach could be that the ltm-ConfigNRDC may be associated with MCG as it is signalled via RRCReconfiguration message received via SRB1, and that the inter-CU SCG LTM configuration must have both MCG and SCG parts. Which may be similar to Rel-18 S-CPAC. Alternatively, ltm-ConfigNRDC

can be maintained as SCG Configuration. In one more alternative, the inter-CU SCG LTM configuration received in ltm-ConfigNRDC can be stored in a new variable different from MCG and SCG. The pros and cons of each alternative must be discussed in RAN2 and agreed.

Proposal 8A: RAN2 to consider the TP in Annexure to discuss and clarify how the inter-CU SCG LTM configuration received in ltm-ConfigNRDC is maintained at the UE.

Reference configuration for inter-CU SCG LTM

RAN2 made the following agreements related to reference configuration and candidate configuration for Inter-CU SCG-LTM:

Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

In our view, the above agreement is not reflecting well in the field description for ltm-ReferenceConfiguration and ltm-CandidateConfig in the latest RRC running CR. Thus, we propose to clarify the associated field descriptions when they are included as part of ltm-ConfigNRDC.

Proposal 8B: RAN2 to consider the TP  in the Annexure to update and clarify the field descriptions for inter-CU SCG LTM in the RRC running CR.

### 2.3.2 	Inter-CU DC LTM with unchanged MCG/SCG scenarios

In the case of inter-CU SCG LTM, it is likely that the SCG configuration (related to bearers mapped to the SCG resources) prepared by all or some of the candidate SNs may be identical to the SCG configuration at the source; in such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN shall have identical MCG configuration. In such a case, an indication in ltm candidate configuration may enable the UE to avoid replacing the current MCG configuration during inter-CU SCG LTM execution.

Observation 9: In the case of inter-CU SCG LTM, it is likely that in many scenarios the SCG configuration prepared by all or some of the candidate SNs may have identical SCG configuration. In such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN remains unchanged.

Proposal 9:  RAN2 to discuss the inter-CU SCG LTM scenario with identical SCG configuration and agree to have a new indication in ltm candidate configuration to enable the UE to avoid replacing the current MCG configuration during inter-CU SCG LTM cell switch.

RAN2 made the following agreement in #129:

The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

As per the above agreement, even though SCG configuration is same across different candidate configuration of intra-CU MCG LTM, the SCG configuration is expected to be included in the MRDC-SecondaryCellGroupConfig IE, and the candidate configuration shall include MRDC-SecondaryCellGroupConfig ReleaseAndAdd; which results in SCG release followed by SCG addition even though SCG configuration remains same across candidate configurations. Which may be avoided by introducing a new indication in ltm candidate configuration, which may enable the UE to decide on whether to avoid a SCG release and add.

Observation 10: For the intra-CU MCG LTM with SCG unchanged, MRDC-SecondaryCellGroupConfig ReleaseAndAdd need not have to be performed.

Proposal 10: RAN2 to discuss the intra-CU MCG LTM scenario with SCG unchanged and agree on introducing a new indication in ltm candidate configuration to avoid SCG release and add procedure.

### 2.3.3	Inter-CU MCG LTM for candidate configurations with different SCG

RAN2 made the following agreement related to SCG Addition in the case of inter-CU MCG LTM.

[RAN2#128]: RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

When there are multiple candidate PSCells prepared associated with a candidate inter-MN target PCell, then an inter-CU MCG LTM execution which may result in SCG or PSCell addition (after an earlier SCG release) may result in a blind PSCell addition, which may potentially result in PSCell RL failure or sub-optimal DC throughput. In this case if the switching decision is only based L1 measurements of PCell the DU may need to choose between two configurations having same PCell but different PSCell blindly.

To enable the DU to choose the right candidate configuration, additional coordination related to PSCell L3 measurements will be required. The impacts of inter-CU MCG LTM with SCG addition were not yet analyzed completely in RAN2. We propose further discussion on this scenario and the required potential signaling changes.

Proposal 11: RAN2 to discuss signaling procedure to handle blind PSCell addition/change when two candidate LTM configurations have same MCG (PCell) but different SCG (PScell).

### 2.3.4	Co-existence Issue : SCG LTM And CHO

Current stage-2 specification for LTM indicate the support for UE for co-existence of any L3 handover when it has LTM configured already as given below.

TS38.300 : Section 9.2.3.5.1 While the UE has stored LTM candidate configurations the UE can also execute any L3 handover except for DAPS handover. In the RRC message which the UE applies for any L3 handover (except DAPS), LTM candidate configurations can be added/modified/released by the target cell.

As per the above it is also possible that UE can also support CHO for PCell change when it has LTM configured for SCG. In such a scenario, if the candidate configuration that includes SCG also includes LTM-configuration update there could be possible mismatch for the SCG-LTM switch command received at UE when it has CHO execution ongoing. The UE in this case does not know whether the candidate-ID referred in the SCG Cell-switch-command refers to the modified LTM configuration or earlier one.

One way to avoid this problem is to suspend SCG-LTM when CHO is configured for MCG. However, this may lead to SCG RLF as the switching is suspended for the duration of CHO evaluation.  In this case better option will be for the UE to drop the switch command received during execution until successful completion of pending CHO.

Proposal 12: RAN2 to discuss the UE behaviour for handling of SCG LTM cell switch command received during CHO execution for PCell.

# 3	Conclusion

In this paper, we analyzed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

Proposals related to Open Issues of Inter-CU LTM

Issue-1 :Fast Recovery candidate nodes

Observation 1: Update of security configuration at all candidate nodes can be easily integrated in current procedure and brings the benefit of recovery possibility at all candidate nodes for Inter-CU switch failure.

Observation 2: Update of security configuration at the time of switching can be second option which involves additional delay in switching.

Proposal 1: Remove the FFS and Confirm that Fast recovery to any candidate CU is supported after Inter-CU LTM switch failure.

Issue-2 :Revert back to Source configuration for Inter-CU Failure

Proposal 2: On Inter-CU LTM switch failure UE revert back to source cell configuration except the PDCP variables for SRB2. FFS whether additional Inter node signaling needed to handle the PDCP SN difference.

Issue 3: LTM Switch failure due to security issues

Observation 3: The LTM switch failure due to alteration of NCC (due to security attacks) on cell switch command is not known to the NW for Inter-CU cell-switch scenario.

Proposal 3: Inclusion of applied-NCC value in RRC message after LTM switch failure is supported.

Issue 4: Inter CU Fast recovery for RLF prior to first LTM switch

Observation 5:  Prior to first LTM switch all the candidate nodes are prepared with Target security key configurations based on the current NCC value.

Observation 6: In case of RLF prior to first LTM switch the fast recovery is possible at UE using its current NCC value.

Proposal 4: Fast Recovery to Inter-CU candidates is supported for RLF prior to first LTM switch.

Issue 5: Inter CU Fast recovery for RLF prior to first LTM switch

Proposal 5: SK-counter list configured for LTM candidates are only used for Inter-SN LTM. FFS in above EN is removed.

Remaining Issues of Inter-CU LTM

Observation 7: In some Inter-CU LTM scenarios subsequent mobility is not needed at some candidate cells. Release of configuration of other candidates is preferred in this case.

Proposal 6: Non-subsequent LTM towards selected candidates is supported for Inter-CU LTM.

Observation 8: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

Proposal 7: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

Remaining Issues of DC LTM

RRC Open Issues with Text Proposal

Proposal 8A: RAN2 to consider the TP in Annexure to discuss and clarify how the inter-CU SCG LTM configuration received in ltm-ConfigNRDC is maintained at the UE.

Proposal 8B: RAN2 to consider the TP  in the Annexure to update and clarify the field descriptions for inter-CU SCG LTM in the RRC running CR.

Other Remaining Issues

Observation 9: In the case of inter-CU SCG LTM, it is likely that in many scenarios the SCG configuration prepared by all or some of the candidate SNs may have identical SCG configuration. In such a case, the MCG part of the inter-CU SCG LTM candidate configuration prepared by the source MN remains unchanged.

Proposal 9:  RAN2 to discuss the inter-CU SCG LTM scenario with identical SCG configuration and agree to have a new indication in ltm candidate configuration to enable the UE to avoid replacing the current MCG configuration during inter-CU SCG LTM cell switch.

Observation 10: For the intra-CU MCG LTM with SCG unchanged, MRDC-SecondaryCellGroupConfig ReleaseAndAdd need not have to be performed.

Proposal 10: RAN2 to discuss the intra-CU MCG LTM scenario with SCG unchanged and agree on introducing a new indication in ltm candidate configuration to avoid SCG release and add procedure.

Proposal 11: RAN2 to discuss signaling procedure to handle blind PSCell addition/change when two candidate LTM configurations have same MCG (PCell) but different SCG (PScell).

Co-existence Issue : SCG LTM And CHO

Proposal 12: RAN2 to discuss the UE behaviour for handling of SCG LTM cell switch command received during CHO execution for PCell.

# 4	Annex

Text Proposal for 9A :

#### 5.3.5.18	LTM configuration and execution

##### 5.3.5.18.1	LTM configuration

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE.

In NR-DC, the UE may receive two independent ltm-Config:

-	an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and

-	an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, , embedded in an RRCReconfiguration message received via SRB1, or alternatively if an ltm-Config associated with SCG is included within an RRCReconfiguration message in ltm-ConfigNRDC via SRB1

Text Proposal for 9B

LTM-Candidate field descriptions

cltmExecutionConditions This field can only be included within an LTM-Config IE associated with the MCG.

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration. If this filed is included in ltm-Config within ltm-Config-NRDC, the RRC configuration will include configuration for both the cell groups.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

| Conditional Presence |  | Explanation |  |
| --- | --- | --- | --- |
| LTM-MCG |  | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |  |
| NR-DC |  | If ltm-ConfigNRDC is configured, this field is optionally present, Need M, within ltm-ConfigNRDC and absent within ltm-Config. If ltm-ConfigNRDC is not configured, this field is optionally present, Need M, within ltm-Config. Otherwise, the field is absent. If this field is included within ltm-ConfigNRDC, the RRC configuration includes configuration for MCG and SCG. |  |