3GPP TSG-RAN WG2 Meeting #130	R2-2504145

St. Julian's, Malta, 19 - 23 May 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss some remaining issues on inter-CU LTM.

# 2	Discussion

## 2.1	Inter-CU LTM in standalone

### 2.1.1	Fast recovery

In RAN2 #129bis meeting, RAN2 made the following agreements related to supporting fast recovery.

RAN2 #129bis - At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Reconfiguration failure (inter-CU LTM)

For reconfiguration failure (inter-CU LTM), the UE has obtained a new NCC before fast recovery. RAN2 agreement is: “If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.” According to the RAN3 key delivery design, for initial LTM cell switch, all candidate gNBs will receive security keys. For subsequent LTM cell switch, the new serving gNB can send updated keys to all candidate gNBs as well, by LTM configuration update procedure. The LTM configuration update procedure can be used for all candidate gNBs, instead of only to the target gNB.

RAN3 #127bis For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

In this case, each candidate gNB will receive updated keys (different key for different candidate cell). If the UE accesses one of the candidate gNBs, this gNB is able to use a correct key, and the UE can also derive the correct key, i.e., revert to the old key and then derive the key for recovery with the previously received NCC and the selected cell configuration (e.g., PCI, DL frequency information). Therefore, the UE can perform recovery to a candidate cell with a Rel-19 ID which is not the one of the target cell.

Proposal 1a: In inter-CU LTM cell switch failure, the UE can perform recovery to a candidate cell with at Rel-19 ID different from the target cell.

If the selected cell for recovery has the same Rel-19 ID like the source cell, in principle, no security key update is needed. If the Rel-18 LTM cell switch failure is used, the UE reverts back to the source configuration, except for the PDCP state variables for SRBs. For example, if the source cell is cell 1 (in gNB1) and PDCP COUNT is x, then inter-CU LTM cell switch to cell 2 (gNB2) is triggered, so the UE resets the PDCP COUNT to 0 for all RBs and increases the COUNT of SRB1 when sending the RRCReconfigurationComplete message. Later when this cell switch fails, the UE selects cell 3 (in gNB1) and the UE maintains PDCP COUNT value, which results in keystream re-use as the UE may already have used the COUNT value 1 for SRB1 with the source cell before.

TS 38.331 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6: 4> revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG;

Observation 1: In inter-CU LTM cell switch failure, if the selected cell has the same Rel-19 ID with the source cell, keystream re-use may happen if the UE follows Rel-18 procedure in which the UE reverts back to the source configuration, except for the PDCP state variables for SRBs.

One direct way to solve this problem is that the UE fully reverts to the source configuration, including PDCP state variables and buffers. However, in Rel-18 discussions, some companies thought that it would be difficult to revert the PDCP to the source after a PDCP re-establishment, and this is the reason why LTM fast recovery is not supported in Rel-18 when L3 HO fails or CHO fails. Considering this, we propose that the UE uses the received NCC to derive a new key when the selected cell for recovery has the same Rel-19 ID as the source cell, in order to avoid the keystream re-use problem.

Observation 2: In Rel-18 LTM, fast recovery is not supported when L3 HO fails or CHO fails, since it is difficult to revert to the source configuration for PDCP state variables and buffers after a PDCP re-establishment.

Proposal 1b: In inter-CU LTM cell switch failure, if the selected cell for recovery has the same Rel-19 ID as the source cell, the UE performs key update and PDCP re-establishment.

RLF

When RLF occurs, the UE has not obtained an updated NCC value. When source cell RLF is declared or an intra-CU LTM cell switch fails, if the UE selects a cell which has a different Rel-19 ID as the source cell, how can the UE access to this cell by recovery procedure? There are two simple solutions. Solution 1 is to introduce a new MAC CE to deliver an NCC value from the source cell to the UE, which can be sent immediately after successful path switch procedure, and the UE stores in order to allow recovery to any LTM candidate in case of RLF. This new NCC MAC CE does not mean to revert the previous RAN2 agreement that the NCC is included in the LTM cell switch command MAC CE, it is only to be used to recover from RLF while no LTM cell switch command MAC CE was received.

The procedure of solution 1 is shown in the figure below. In current RAN3 design, after each inter-gNB cell switch, the target gNB sends LTM configuration update message to other candidate gNBs, which could include the keys for so the candidate gNBs know which gNB is this UE’s previous serving gNB.

Figure 1: inter-CU LTM recovery from RLF, NCC provided by the source cell

Solution 2 is that the UE obtains a new NCC from the selected cell, e.g., in msg3 of CBRA, the UE requests the network to provide the NCC and in msg4, the network provides the NCC, then the UE can use this new NCC to derive a new key, apply the new configuration and transmit the RRCReconfigurationComplete message.

Figure 2: inter-CU LTM recovery from RLF, NCC provided by the target cell

Proposal 2: In source cell RLF case, support fast recovery if the selected cell for recovery has a different Rel-19 ID from the source cell. Discuss the following two solutions:

- Solution 1: introduce a new MAC CE for NCC delivery independently of LTM cell switch.

- Solution 2: the UE obtains a new NCC from the selected cell during the CBRA procedure.

### 2.1.2	Enhanced LTM cell switch command MAC CE

In the current MAC running CR, the NCC value is optional present in the newly introduced “Enhanced LTM Cell Switch Command MAC CE” with a complicated present/absent condition. When it is absent, the MAC CE content is the same as the Rel-18 LTM Cell Switch Command MAC CE. It is easier to make the NCC value a mandatory present field in this MAC CE. For the case when the key update is not needed, the network can send a Rel-18 LTM cell switch command MAC CE.

Proposal 3: In the Rel-19 Enhanced LTM Cell Switch Command MAC CE, the NCC value is mandatory present. For the case when the key update is not needed, the network sends a Rel-18 LTM cell switch command MAC CE.

## 2.2	Inter-CU LTM in DC

### 2.2.1	MCG LTM

RAN2 agreed that MCG LTM cell switch can setup an SCG or modify the SCG, including PSCell change, as long as the SN is not changed. In Rel-18, upon MCG LTM cell switch, the target DU can be notified, but there is no notification of the SN. In Rel-19, as the SCG configuration can be changed, the SN needs to know which configuration to apply. RAN2 agreed that SCG release and addition will always be used for SCG reconfiguration. One solution could be that the UE is allocated different dedicated preambles for each candidate LTM configuration, but the SN would need to reserve more preambles for each UE, which is not convenient from a network perspective. Therefore, the MN should notify the SN of the target configuration.

Proposal 4: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

In Rel-19 MCG LTM, if the MN key is changed, the SN key also needs to be changed, and it is derived by the target MN from the MN key. For MN key change, the UE derives the new MN key from the NH and from an NCC value, and derives the SN key from the MN key and the sk-counter, and the SN receives the SN key from the MN.

In Rel-15, the SN key is provided in the SgNB addition request, but for LTM, this message is likely to be used only at the preparation of an LTM candidate cell, not at LTM cell switch execution. As a notification from the MN to the SN is needed at LTM cell switch to indicate the candidate configuration ID, the same notification could include the SN key.

Depending on the exact scheme designed by SA3, it may be possible to provide the SN key earlier, so that operation on SN-terminated MCG bearers could start without waiting for the notification from the target MN, but this would require further specification work and the MN could send the notification early enough to the SN.

Proposal 5: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

### 2.2.2	Coexistence of intra-CU and inter-CU SCG LTM

In Rel-18 SCG (intra-CU) LTM, LTM is configured by the SN without MN involvement. In Rel-19 SCG LTM, LTM is configured by the MN and the candidate LTM configuration is an MN RRC message that includes the SCG configuration, possibly with other changes by the MN. Besides, in order to support the co-existence of intra-SN candidate cells and inter-SN candidate cells, RAN2 agreed that intra-SN LTM candidate configuration can be configured by the MN as well, unlike the Rel-18 SCG LTM configuration.

RAN2 #128 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

When there are intra-SN LTM candidate cells and inter-SN LTM candidate cells, to simplify the LTM preparation and configuration procedure, we can re-use a Rel-18 S-CPAC principle i.e., when there are inter-SN and intra-SN candidate cells, it is the MN that configures all the candidate configurations.

Proposal 6: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

### 2.2.3	SCG LTM UL grant handling

In inter-CU SCG LTM, when SCG LTM is triggered, the UE applies the LTM candidate configuration, performs RACH-less or RACH-based LTM cell switch to the target PSCell and sends an RRCReconfigurationComplete message to the MN. Then, the MN transfers the SN RRCReconfigurationComplete message to the SN.

In Rel-18, if SRB3 is configured, the UE will transmit the RRCReconfigurationComplete message to the SN directly. However, in Rel-19, even if SRB3 is configured, the UE still transmits the RRCReconfigurationComplete message via SRB1.

Observation 3: In Rel-18 intra-CU LTM, when SRB3 is configured, the UE sends the RRCReconfigurationComplete message to the SN directly. However, in Rel-19 SCG LTM, the UE sends the RRCReconfigurationComplete to the MN, regardless whether SRB3 is configured.

In Rel-19 SCG LTM, if the SCG LTM cell switch is RACH-less, the UE may not have uplink data or signalling to transmit to the SCG. If the network configures UL grant skipping, the SCG may not receive any UE uplink transmission and consider RACH-less cell switch to be failed. To avoid this problem, the network should not configure the UE to skip UL grants in RACH-less SCG LTM procedure, e.g., not configure enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured and skipUplinkTxDynamic.

Proposal 7: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless whether SRB3 is configured.

# 3	Conclusion

This contribution makes the following proposals:

Fast recovery

Proposal 1a: In inter-CU LTM cell switch failure, the UE can perform recovery to a candidate cell with at Rel-19 ID different from the target cell.

Proposal 1b: In inter-CU LTM cell switch failure, if the selected cell for recovery has the same Rel-19 ID as the source cell, the UE performs key update and PDCP re-establishment.

Proposal 2: In source cell RLF case, support fast recovery if the selected cell for recovery has a different Rel-19 ID from the source cell. Discuss the following two solutions:

- Solution 1: introduce a new MAC CE for NCC delivery independently of LTM cell switch.

- Solution 2: the UE obtains a new NCC from the selected cell during the CBRA procedure.

Enhanced LTM cell switch command MAC CE

Proposal 3: In the Rel-19 Enhanced LTM Cell Switch Command MAC CE, the NCC value is mandatory present. For the case when the key update is not needed, the network sends a Rel-18 LTM cell switch command MAC CE.

MCG LTM

Proposal 4: For Rel-19 MCG LTM, at LTM cell switch execution, the (target) MN indicates the target LTM candidate configurations ID to the SN.

Proposal 5: For Rel-19 MCG LTM, if there are SN-terminated bearers in the target LTM candidate configuration, the target MN provides the SN key to the SN at LTM cell switch execution.

Coexistence of intra-CU and inter-CU SCG LTM

Proposal 6: When there are inter-SN and intra-SN LTM candidate cells for SCG LTM, it is the MN that configures all the LTM candidates (with a MN RRC message).

SCG LTM UL grant handling

Proposal 7: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch, regardless whether SRB3 is configured.

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.