3GPP TSG-RAN WG2 Meeting #130	R2-2504146

St. Julian's, Malta, 19 - 23 May 2025

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# Introduction

At RAN #107, the WID for Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (C-LTM) should be specified for the intra-CU case in Rel-19, with limitations defined for the scenario where the UE is in a non-DC case.

RP-250339, Revised Work Item: NR mobility enhancements Phase 4 - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In RAN2 meetings, RAN2 has already made some agreements. In this contribution, we discuss the CG-based RACH-less procedures for intra-CU C-LTM and C-LTM TAT maintenance, re-acquisition, handling for the candidate cells and that for the target cell.

# 2	Discussion

## 2.1	CG-based RACH-less cell switch

### 2.1.1	Beam selection for L3-based CLTM

For beam selection in C-LTM triggered by an L1 based execution condition, RAN2 had the following agreements:

RAN2#129 meeting Agreements on C-LTM: 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

For L3-based C-LTM execution, RAN2 did not decide how to perform beam selection.. A L3-based condition does not applies on individual beams, and its parameters are selected to be applied on the cell quality consolidated from multiple beams, so it may not be suitable for beam selection.

In the Rel-18 RACH-based LTM cell switch procedure, the UE selects an SSB with an SS-RSRP above the rsrp-ThresholdSSB among the candidate SSBs, where the parameter rsrp-ThresholdSSB is configured during the LTM preparation procedure.

TS 38.321-i40 clause 5.1.2 1> else if contention-free Random Access Resources have not been explicitly provided by an LTM Cell Switch Command MAC CE, the Random Access procedure was not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3, contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available: 2> select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs; 2> set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB. …… 1> else (i.e. for the contention-based Random Access preamble selection): 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

A similar threshold could be used to determine whether a beam is suitable for RACH-less LTM cell switch. If the candidate cell satisfies the L3 execution condition, the UE should select a target beam that exceeds the threshold. If none of the beams above the threshold is associated with a CG occasion, the UE should perform RACH-based LTM cell switch.

Proposal 1: If the L3 execution condition is met, the UE can only select beams above a threshold (FFS whether to reuse an existing threshold or define a new one).

### 2.1.2	Beam - CG association

In Rel-18, the CG for RACH-less configuration can indicate a subset of SSBs.

CG-RRC-Configuration-r18 ::=   SEQUENCE {

cg-RRC-RetransmissionTimer-r18 INTEGER (1..288)                            OPTIONAL,   -- Need R

cg-RRC-RSRP-ThresholdSSB-r18   RSRP-Range                                  OPTIONAL,   -- Need R

rrc-SSB-Subset-r18             CHOICE {

shortBitmap-r18                BIT STRING (SIZE (4)),

mediumBitmap-r18               BIT STRING (SIZE (8)),

longBitmap-r18                 BIT STRING (SIZE (64))

}                                                                          OPTIONAL,   -- Need S

rrc-SSB-PerCG-PUSCH-r18        ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight,

sixteen}  OPTIONAL,   -- Need M

rrc-P0-PUSCH-r18               INTEGER (-16..15)                           OPTIONAL,   -- Need M

rrc-Alpha-r18                  ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08,

alpha09, alpha1} OPTIONAL, -- Need M

rrc-DMRS-Ports-r18             CHOICE {

dmrsType1-r18                  BIT STRING (SIZE (8)),

dmrsType2-r18                  BIT STRING (SIZE (12))

}                                                                           OPTIONAL,  -- Need M

rrc-NrofDMRS-Sequences-r18  INTEGER (1..2)                                  OPTIONAL,  -- Need M

...

}

sdt-SSB-Subset, rrc-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not included in the SSB subset for SSB to CG PUSCH mapping while value 1 indicates that the corresponding SS/PBCH block is included in SSB subset for SSB to CG PUSCH mapping. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs. In case of an RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

If rrc-SSB-Subset can be configured for C-LTM, some SSBs will not have any associated CG occasion, then if the UE would select those SSBs when the C-LTM execution condition is met, the UE would not be able to perform RACH-less LTM cell switch.

To avoid this problem, one solution could be that the network does not configure rrc-SSB-Subset for any LTM candidate configuration for which an execution condition is configured. However, the CG configuration for RACH-less is determined by target cell while the usage of conditional LTM, as opposed to LTM triggered by a MAC CE, is decided by the source cell, so such a restriction seems difficult to ensure.

Therefore, it should be the UE that takes into account that some SSBs of the candidate cell may have associated CG occasions for RACH-less cell switch, while other SSBs of the same candidate cell don't have any associated CG occasion for RACH-less cell witch.

Proposal 2a: If the C-LTM execution condition is met and the UE has a valid TA for the associated candidate cell, to select a beam, the UE prioritizes beams that have associated CG occasions.

Proposal 2b: If no beam with an associated CG occasion meets the L1-based C-LTM execution condition or is above the beam selection threshold (for L3-based C-LTM execution), the UE performs RACH-based LTM cell switch.

### 2.1.3	Separate TCI mode

If the target cell uses joint TCI mode, for a particular CG occasion, the target cell knows which UL beam to use to receive the UE UL message and also knows which DL beam to use to schedule the UE afterwards. If the target cell uses separate TCI mode, the network may not know the beam to use for DL transmissions since RAN2 agreed that the CG occasions are associated with UL TCI states when separate TCI mode is used. However, it would be possible for the UE to include in the first UL transmission an indication of a good DL beam, so that the network can further schedule the UE. There is a risk that this first UL transmission is not successfully decoded by the network and then the network cannot schedule a retransmission, but the network could configure automatic repetition in the CG so that the UE autonomously performs retransmissions if the network does not provide a further UL grant.

Proposal 3: For CG-based RACH-less cell switch, if separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS).

## 2.2	TA delivery and maintenance

### 2.2.1	LTM candidate with MIMO 2TA

RAN2#128 meeting Agreements on C-LTM: 10. Candidate cell TA is maintained by a new timer.

RAN2#129bis meeting Agreements on C-LTM: 4. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

For PDCCH-ordered early RACH, RAN2 has agreed that the candidate cell TA is maintained by a new timer. Additionally, in the last RAN meeting, it was also agreed that C-LTM can coexist with MIMO 2TA. Therefore, when the candidate cell is configured with MIMO 2TA, the UE must maintain two separate TA timers, each associated with the corresponding TA value for a single candidate cell, if the PDCCH-ordered early RACH has been been performed.

In the MIMO two TA scenario, two distinct TA timer lengths can be configured via RRC configuration. As a result, the UE can maintain separate TA values with individual TA timers if the network provides these two values.

Proposal 4: For LTM candidates with MIMO two TAs, the UE should maintain separate TA values with individual TA timers, provided that the network supplies these two values.

What’s more, RAN2 should also consider how to perform the C-LTM cell switch if the candidate cell is configured with MIMO two-TA scenario. In this context, the UE selects the target beam, and the candidate beam of the target cell may belong to any of the two TA values. If the UE selects a beam with a valid TA, a RACH-less LTM cell switch procedure can be executed. However, if the UE selects a beam that does not have a valid TA, the UE should initiate the RACH procedure in the MIMO two-TA scenario for the candidate cell.

Proposal 5: If the UE selects a beam without a valid TA, it should perform the RACH procedure for the MIMO two-TA scenario of the candidate cell.

### 2.2.2	C-LTM TA re-acquisition

If the validity timer for the C-LTM TA of an LTM candidate cell is about to expire while the UE still might perform a cell switch to the candidate cell, the source may want to send a PDCCH order to renew the C-LTM TA so that RACH-less LTM cell switch is still possible.

Proposal 6: The source DU should maintain the C-LTM TAT and be capable of recognizing when the TAT has expired.

### 2.2.3	C-LTM TAT handling for the candidate cells (except the target cell)

RAN2#129bis meeting Agreements on C-LTM 1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

According to the agreement made in the last meeting, the TA value should be maintained, and the corresponding C-LTM TA timer should continue running after the cell switch. Thus, the TA value and its associated C-LTM TA timer must be transmitted from the source cell to the target cell. This ensures that the target cell will not perform the PDCCH-ordered early RACH again if the TA value has been previously received and still remains valid. This action impacts RAN3. Therefore, it is necessary to inform RAN3 that the source cell should transmit both the TA values and their corresponding remaining time of the C-LTM TATs for the candidate cells to the target cell.

Proposal 7: The source cell should transmit both the TA values and the corresponding remaining time of the C-LTM TATs for the candidate cells to the target cell. This action impacts RAN3.

### 2.2.4	C-LTM TAT handling for the target cell

RAN2#129bis meeting Agreements on C-LTM 2. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

Furthermore, for the target cell, the C-LTM TAT should not be stopped after the LTM cell switch. Instead, the target cell should maintain both the C-LTM timer and the PTAG timer simultaneously. The remaining time from the C-LTM timer should be used in PTAG timer in the target cell. This implies that the source cell must also transmit the TA value along with its associated C-LTM TA timer to the target cell, which has an impact on RAN3.

Proposal 8: Inform RAN3 that the source cell needs to transmit the remaining time of the C-LTM TAT to the target cell.

# 3	Conclusion

This contribution makes the following proposals:

CG-based RACH-less cell switch

Proposal 1: If the L3 execution condition is met, the UE can only select beams above a threshold (FFS whether to reuse an existing threshold or define a new one).

Proposal 2a: If the C-LTM execution condition is met and the UE has a valid TA for the associated candidate cell, to select a beam, the UE prioritizes beams that have associated CG occasions.

Proposal 2b: If no beam with an associated CG occasion meets the L1-based C-LTM execution condition or is above the beam selection threshold (for L3-based C-LTM execution), the UE performs RACH-based LTM cell switch.

Proposal 3: For CG-based RACH-less cell switch, if separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS).

TA delivery and maintenance

Proposal 4: For LTM candidates with MIMO two TAs, the UE should maintain separate TA values with individual TA timers, provided that the network supplies these two values.

Proposal 5: If the UE selects a beam without a valid TA, it should perform the RACH procedure for the MIMO two-TA scenario of the candidate cell.

Proposal 6: The source DU should maintain the C-LTM TAT and be capable of recognizing when the TAT has expired.

Proposal 7: The source cell should transmit both the TA values and the corresponding remaining time of the C-LTM TATs for the candidate cells to the target cell. This action impacts RAN3.

Proposal 8: We should inform RAN3 that the source cell needs to transmit the remaining time of the C-LTM TAT to the target cell.

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4 Apple Inc, China Telecom.