3GPP TSG-RAN WG2 Meeting #130	R2-2504147

St Julian's, Malta, 19 - 23 May 2025

Title: 	Discussion on MBS broadcast over NTN

Source: 	Huawei, HiSilicon, Turkcell

Agenda Item:	8.8.4

Document for:	Discussion and decision

# Introduction

One of the objectives for Rel-19 NTN WI [1] is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #129bis meeting, the following agreements in [2] were made:

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency.

- FFS whether and how the association between intended service area and frequencies is acquired.

In this contribution, we provide our views on the remaining issues.

# Discussion

## 2.1	Frequency prioritization

One FFS from the last meeting is whether and how the UE acquires the association between the intended service areas (ISA) and frequencies. The discussion is mainly focused on two options:

Option1: Add intended service area IDs in SIB21

Option2: UE rely on USD info combined with FSAI/frequency info in SIB21 with no ISA introduced in SIB21

According to TS 24.237, the RAN is configured with FSA ID(s) and their mapping frequencies by OAM. Then the RAN receives the MBS FSA ID(s) and the Intended Service Area(s) for the MBS session from the AMF at MBS session start and MBS session update. The RAN nodes then use those MBS FSA ID(s) to determine cells/frequencies within the MBS service area to broadcast MBS session data and can look at the ISA definitions to determine the frequency/ies that provide the MBS broadcast service in each ISA. The RAN nodes can then indicate, for each frequency in SIB21, the list of ISAs covered in this frequency. According to TS 38.423, the Xn Setup and NG-RAN node Configuration update can indicate the list of FSAs for each served cell. If considered useful, by adding ISAs for each served cell, the neighbour nodes could also indicate ISAs according to coverage by neighbour nodes.

For option 2, the UE compares the MBS FSA IDs(s) in the service announcement with the MBS FSA ID(s) in SIBs for frequency selection. The, the UE needs to match the MBS FSA ID in the service announcement with the ISAs. The service announcement defined in TS 23.247 may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information.

For local MBS service, the Service Announcement may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information. <omitted> If the MBS Session is for broadcast, the Service Announcement may include the MBS FSA ID(s) and optional frequency information associated with the broadcast MBS session and may also include an indication that the MBS Session is intended for RedCap UEs, for non-RedCap UEs or both for RedCap UEs and non-RedCap UEs as defined in TS 26.502 [18].

However, according to TS 26.502, the MBS service area along with the FSA ID does not support the geographical area format, as indicated as below:

Table 4.5.9-1: Mapping of baseline parameters to MBS Session Context parameters MBS Session Context parameter; Source; Clause; Source parameter. State; MBS Distribution Session.; - 4.5.6; State.. Source-Specific Multicast (SSM)IP address; MBS Distribution Session.; - 4.5.6; MBS Session Identifier. TMGI. MBS Service Area; MBS Distribution Session.; - 4.5.6; Target service area (see NOTE 2). Area Session Identifier; Assigned by MB-SMF.; - 4.5.6; Location-dependent service flag. MBS Frequency Selection Area (FSA) ID (see NOTE 1); MBS Distribution Session.; - 4.5.6; MBS Frequency Selection Area. <omitted>. NOTE 1: Applicable to Broadcast MBS Session only. NOTE 2: Target service area is signalled to the MBSF at reference point Nmb10/Nmb5 in the form of a Tracking Area Identifier (TAI) list and/or Cell ID list. External-facing target service area identification at reference point N33 is translated by the NEF into the appropriate form(s) as required..

Therefore, option 2  requires that SA4 enhance the service announcement. Due to limited time, we suggest that RAN2 support to add ISAs to FSA ID/frequency in SIB21.

Proposal 1: Indicate ISAs to for each frequency in SIB21.

## 2.2	Skip MCCH acquisition

The last RAN2 meeting agreed that:

If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

The UE gets to know the ISAs applicable for an MBS session through the MCCH information (MBSBroadcastConfiguration message provided via MCCH), so this seems to imply that the UE needs to first acquire MCCH and information for all the MBS broadcast services of interest, and then, if the UE is not in any of the applicable ISA, the UE can skip monitoring MCCH.

However, an NTN cell can be large and it is possible that all the ISAs of all the MBS broadcast service only cover a subset of the cell. In this case, if the UE reads SIBx and sees that it is not in any of the ISAs, it does not need to acquire MCCH. However, if the cell provides an MBS broadcast service that is relevant for the whole cell coverage, the UE should still acquire MCCH to determine whether ISAs are applicable to the service (and which ones). This means that there should be an indication in SIBx whether UEs that do not know the applicable ISA(s) for their MBS service(s) of interest can skip MCCH acquisition is they are not in any ISA in SIBx.

Proposal 2a: SIBx indicates (e.g., 1-bit) whether the cell may broadcast MBS broadcast services relevant in the whole cell coverage.

Proposal 2b: If SIBx does not indicate that the cell may transmit MBS broadcast services relevant in the whole cell coverage, a UE interested in receiving one or more MBS broadcast service but not in any ISA of the cell may skip reading MCCH.

If the UE has read MCCH and acquired the list of ISAs applicable to the MBS broadcast services of interest, and the UE is not located in any of the ISAs, the UE may skip reading MCCH.

The MCCH change notification message can be used to indicate to the UE to perform MCCH information re-acquisition i.e., acquire the updated MBSBroadcastConfiguration message. One possible case is that the ISAs applicable to an ongoing MBS broadcast service have changed, so the UE that was not in any ISA of an MBS service of interest might now be in such an ISA. Such a modification may require updating SIBx but it is also possible that no such update occurs, only MCCH information is updated. So it is necessary that the UE monitors MCCH change notification and re-acquire MCCH if needed.

Proposal 3a: The UE that has acquired MCCH and is out of the ISAs of the MBS service of interest shall monitor MCCH change notification and re-acquire MCCH again.

The MCCH change notification can indicate whether a new MBS service has started or whether there is a modification of MCCH information other than the change caused by start of new MBS service(s). If the bit indicating "modification of MCCH information other than the change caused by start of new MBS service(s)" is used when the ISAs of a service are modified, for any modification of MCCH information, the UEs currently out of any ISA of the MBS services of interest will re-acquire MCCH at any modification of MCCH information other than the change caused by start of new MBS service(s), which such modifications are normally only useful for UEs currently receiving MBS services. Such modifications may be rather frequent, so that the UE power consumption may be unnecessarily increased.

In order to mitigate such issue, we can consider alternatives such as:

-	upon MCCH change notification, the UE only re-acquires MCCH when the bit indicating "service start" is set ;

-	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

-	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx;

Proposal 3b: Discuss how to reduce unnecessary MCCH re-acquisition for the UE that has acquired MCCH and is out of the ISAs of the MBS services of interest, such as:

alt. 1)		upon MCCH change notification, the UE only re-acquire MCCH when the bit indicating "service start" is set ;

alt. 2)	upon MCCH change notification, the UE only re-acquires MCCH when a new bit indicating "change of ISAs of ongoing MBS service" is set;

alt. 3)	the UE only monitors MCCH change notification every x MCCH modification periods, where x is provided to the UE e.g., in SIBx.

The previous RAN2 meeting agreed that the MCCH information includes a list of service area IDs for each MBS session. The last RAN2 meeting confirmed that the area ID corresponding to the ISA of the current serving cell in the new SIB. In previous releases, the MCCH information, i.e., MBSBroadcastConfiguration, provides the support status of MBS sessions not only for the serving cell, but also for neighbour cells. That means, except for the MBS session ID list supported by the serving cell, it also indicates the MBS session ID list provided by the neighbour cells. If MBSBroadcastConfiguration also provides the intended service area IDs for neighbour cells, upon mobility to a neighbour cell, the UE that has previously obtained the ISA Ids for the service sees the definitions of ISAs from the new SIB and immediately knows whether it is currently in an ISA of the service, and if not, it may not read MCCH.

Observation 1: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid monitoring MCCH when moving to a neighbour cell but out of the ISAs of the service.

Besides, based on neighbour cell information provided in the MBSBroadcast configuration, the UE can request unicast reception of the service of interest before moving to a cell not providing the MBS session. If the UE is in RRC_IDLE, this involves establishing an RRC connection. If the UE is in RRC_CONNECTED, the UE may request to switch to unicast mode for MBS session data reception as it comes closer to a target cell.

However, when moving to the new cell, if the UE is not in any of the intended service area, there is no use for the UE to start performing unicast reception of the service. If the MCCH also provides ISA IDs for neighbour cells and the ISA IDs are aligned between the serving and the neighbour cell, the UE can avoid switching to unicast reception unnecessarily.

Observation 2: If the MCCH also provides ISA IDs for neighbour cells, the UE can avoid unnecessarily switching to unicast reception before moving to a neighbour cell but out of the ISAs of the service.

Proposal 4: The MCCH also indicates neighbour cells MBS session specific intended service area ID(s).

Proposal 5: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

# Conclusion

In this document, we discussed the support of broadcast service via NR NTN, and proposed the following:

Proposal 1: Indicate ISAs for each frequency in SIB21.

Proposal 2a: SIBx indicates (e.g., 1-bit) whether the cell may broadcast MBS broadcast services relevant in the whole cell coverage.

Proposal 2b: If SIBx does not indicate that the cell may transmit MBS broadcast services relevant in the whole cell coverage, a UE interested in receiving one or more MBS broadcast service but not in any ISA of the cell may skip reading MCCH.

Proposal 3a: The UE that has acquired MCCH and is out of the ISAs of the MBS service of interest shall monitor MCCH change notification and re-acquire MCCH again.

Proposal 4: The MCCH also indicates neighbour cells MBS session specific intended service area ID(s).

Proposal 5: To determine the ISA of an ISA ID indicated in MCCH of the serving cell for an MBS service in a neighbour cell, the UE can use the new SIB of the serving or of the neighbour cell.

# References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session chair (ZTE)