3GPP TSG-RAN-WG2 Meeting #130	                                   R2- 2504156

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item: 8.1.2.2

Source: Sharp

Title: On Explicit Reporting of Functionality Inapplicability

Document for: Discussion and Decision

# Introduction

In RAN2#129, following agreements were made [1]:

- Agreements applicability reporting and management

Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

The provided periodic CSI configuration should be consistent with reported UE capabilities

FFS option B

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

In RAN2#129bis, following agreements were made [2]:

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

This tdoc examines the FFS topics outlined in the above RAN2 agreements, with a focus on the reporting of functionality inapplicability and the associated cause of inapplicability. The focus is on enabling the UE to indicate both inapplicability of a functionality and the reason for its inapplicability via for e.g., a simple, standardized cause value.

We further discuss that simple cause value for functionality inapplicability could be reported by the UE, however, it should not be restricted solely to model (un)availability (e.g., the need to download an ML model prior to activation). Instead, a standardized set of cause values could be defined to reflect a broader range of reasons for inapplicability that may include for e.g., UE-side constraints such as insufficient resources (e.g., memory, power), unsupported configurations, or operational state limitations. We discuss how such cause values can be defined and signalled, and to support their inclusion in relevant signalling procedures.

# On Explicit Reporting of functionality (in) applicability with feedback or cause

In this section, the reporting of the changes of the applicability of a functionality is discussed based on the following agreements in RAN2 #129bis [2].

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

In RAN2 #129 [1], it was discussed and agreed to support the explicit reporting of applicability or inapplicability in the initial report while any subsequent reports will be sent only when there is a change in applicability (e.g., via UAI). However, whether to report the explicit cause for inapplicability was still under discussion.

In RAN 2 #129bis [2], UE reporting of AI/ML applicability cause was discussed and, several viewpoints have been expressed regarding the appropriate level of detail in signalling. One widely supported approach favours a simple binary indication, where the UE reports only whether the model is currently available or not, without specifying reasons. This is seen as sufficient for basic coordination while minimizing signalling overhead. Another viewpoint from some companies propose to include a simple, standardized cause for inapplicability (e.g., "temporarily unavailable"), though there is ongoing debate about the necessary scope and complexity of such causes. However, this introduces a new challenge, i.e., the distinction between temporary and permanent inapplicability remains unclear. In principle, all functionalities are inapplicable until the necessary conditions are met for functionalities to become applicable. As such, further discussion is needed to clearly define what constitutes temporary versus permanent inapplicability, including criteria, duration, and the conditions that must change for applicability to be reassessed. Without such clarification, the categorization risks being vague and incomplete, leading to inconsistent and potentially misleading signalling and UE behaviour.

In our view, indicating that a functionality is "temporarily unavailable" may not be sufficient to fulfil the intended purpose of informing the network about the cause of inapplicability. We believe that, while a simple cause value could be reported by the UE, it should not be restricted solely to model availability. Instead, a standardized set of cause values could be defined to reflect a broader range of reasons for inapplicability. The reasons may include for example:

- Outdated or invalid AI/ML model

- Unavailable AI/ML model

- Insufficient UE processing capability/UE side issues

- Resource conflict with other concurrent UE operations

- Unavailability of required CSI-RS resources

- Network side conditions

- UE side conditions or environment

Providing such granular cause values would allow the network to better understand the underlying limitations affecting applicability and potentially adapt its behaviours accordingly. This approach supports enhanced transparency and more informed decision-making in the network, while still maintaining manageable signalling complexity.

Observation 1: Indicating a functionality to be temporarily unavailable may be too broad and not serve the purpose of inapplicability cause indication.

A contrasting opinion also expressed by some companies is that no cause is needed, as the network does not require insight into internal UE reasoning to proceed with its operations. A further perspective suggests that a cause should only be included when it has a potential impact on network or UE behaviour. Overall, the discussion centres on balancing signalling efficiency with the potential value of added context in the applicability reporting procedure.

However, we believe, that the cause indication for inapplicability reporting is needed. In the absence of such cause reporting, if one or more configurations provided by the gNB are inapplicable due to UE-side conditions—such as hardware/battery or compute resource limitations, environmental factors, capability restrictions, or model unavailability/obsolescence—and the cause of inapplicability is not reported by the UE, it leads to several challenges. Without visibility into the specific reason for inapplicability, the gNB may attempt to configure the UE with a new configuration while waiting for the status update on the reported inapplicable configuration.

The network may also repeatedly attempt to (re)configure the UE, assuming the inapplicability is temporary or resolvable (e.g., by providing an updated or a new configuration), which results in multiple RRC Reconfiguration messages. This not only creates unnecessary signaling overhead but also causes inefficient use of radio resources. Additionally, repeated reconfiguration attempts can delay valid reconfiguration procedures, increasing overall latency. The frequent attempts also put a strain on UE power consumption, affecting UE battery life. Thus, knowing the cause for functionality inapplicability will help the network to make an informed decision on whether further reconfiguration attempts are necessary.

Moreover, the gNB might misallocate resources, assuming that the UE can eventually support the requested configuration or that one of the configurations provided to the UE will be applicable in the near future. This could lead to suboptimal resource distribution and network inefficiencies. Persistent reconfiguration attempts may also cause an unstable RRC state at the UE, complicating network management procedures.

This issue stems from the lack of appropriate feedback regarding the cause of inapplicability, which prevents the gNB from understanding whether the problem is temporary or permanent, or whether it results from UE-side conditions or other factors. As a result, the gNB may misinterpret the inapplicability as a transient issue and continue to send reconfiguration requests, further increasing signalling overhead and exacerbating network inefficiencies.

Observation 2: Lack of feedback regarding the cause of inapplicability from the UE can lead to significant inefficiencies in signalling, resource usage and allocation, increased latency, and higher power consumption at the UE-side.

Observation 3: Knowing the cause of a functionality's inapplicability offers advantages to the network by enabling it to:

- Decide whether the current inference configuration needs to be changed or if any further reconfigurations are needed.

- Initiate a new model transfer or update, potentially including dataset and model parameters.

Proposal 1: RAN2 to agree that cause for inapplicability can be reported by the UE.

Proposal 2: A set of cause values needs to be defined to reflect a broader range for inapplicability causes for e.g., Outdated or invalid AI/ML model, Unavailable AI/ML model, Insufficient UE processing capability/UE side issues.

Furthermore, as per the studies in 3GPP, in case of two-sided model transfer, the network may send datasets or model parameters to the UE or a UE-side OTT server. UE reporting of model applicability, including the inapplicability cause, enables the network to trigger appropriate model transfer procedures and pause inference configurations until the UE confirms the new model's availability, optimizing signalling and resources.

When the UE receives a request to activate a functionality or receives a CSI-ReportConfig. (Option a or option b) that depends on a locally stored AI/ML model not currently available on the UE, the UE may initiate a model download procedure prior to activation of the requested feature.

If the inapplicability is due to model unavailability or required model update, the UE shall indicate to the network that the applicability of the requested functionality is subject to model download and associated delay together with the cause value. This indication may include an estimated time for functionality to become applicable or estimated delay for downloading/updating the model.

Proposal 3: The UE shall indicate to the network that the applicability of the requested functionality is subject to model download (e.g. via cause value) and associated estimated delay.

To support alignment with the network, the UE may indicate that a model update is in progress. Upon completion of the model download and/or successful activation of the functionality, the UE may send a Model/parameter download progress report (complete/in-progress/failed/not-started etc.) notification together with functionality applicability status in UAI or RRC message to the network.

Proposal 4: UE may indicate model/parameter download progress to the network.

The cause for inapplicability may not need to be reported every time; therefore, the network can either request the inapplicability cause when needed or configure the UE to report it only when necessary.

Proposal 5: The network may request the cause for inapplicability from the UE when needed.

Upon resolution of the cause for inapplicability, and once the functionality or CSI-ReportConfig. becomes applicable, the network may activate it via for e.g., MAC CE. The UE shall not autonomously initiate periodic CSI reporting based solely on UAI transmission (as UAI is for primarily for indicating assistance information to the network) but shall await explicit activation or deactivation via network signalling, such as a MAC CE.

Proposal 6: If an Option A configuration (e.g., Full CSI-ReportConfig.) provided in Step 3 that is temporarily inapplicable and later becomes applicable after the cause for inapplicability is resolved:

- UE must report its renewed applicability via UAI

- UE shall not autonomously activate the periodic CSI-ReportConfig.

Proposal 7: The UE shall wait for the network to activate the periodic CSI-ReportConfig. for example, via MAC CE.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Observation 1: Indicating a functionality to be temporarily unavailable may be too broad and not serve the purpose of inapplicability cause indication.

Observation 2: Lack of feedback regarding the cause of inapplicability from the UE can lead to significant inefficiencies in signalling, resource usage and allocation, increased latency, and higher power consumption at the UE-side.

Observation 3: Knowing the cause of a functionality's inapplicability offers advantages to the network by enabling it to:

- Decide whether the current inference configuration needs to be changed or if any further reconfigurations are needed.

- Initiate a new model transfer or update, potentially including dataset and model parameters.

Proposal 1: RAN2 to agree that cause for inapplicability can be reported by the UE.

Proposal 2: A set of cause values needs to be defined to reflect a broader range for inapplicability causes for e.g., Outdated or invalid AI/ML model, Unavailable AI/ML model, Insufficient UE processing capability/UE side issues.

Proposal 3: The UE shall indicate to the network that the applicability of the requested functionality is subject to model download (e.g. via cause value) and associated estimated delay.

Proposal 4: UE may indicate model/parameter download progress to the network.

Proposal 5: The network may request the cause for inapplicability from the UE when needed.

Proposal 6: If an Option A configuration (e.g., Full CSI-ReportConfig.) provided in Step 3 that is temporarily inapplicable and later becomes applicable after the cause for inapplicability is resolved:

- UE must report its renewed applicability via UAI

- UE shall not autonomously activate the periodic CSI-ReportConfig.

Proposal 7: The UE shall wait for the network to activate the periodic CSI-ReportConfig. for example, via MAC CE.

# References

[1] Chair notes, RAN2 129 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, 2025

[2] Chair notes, RAN2 129bis 3GPP TSG-RAN WG2 Meeting #129bis, Wuhan, China, 2025