3GPP TSG-RAN-WG2 Meeting #130	                                   R2-2504157

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item: 8.1.2.2

Source: Sharp

Title: Considerations for Functionality Activation and Reporting

Document for: Discussion and Decision

# Introduction

In RAN2#129, following agreements were made [1]:

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signalling details for option B (e.g. whether it is signalling in CSI-Report Config or otherconfig)

In RAN2#129bis, following agreements were made [2]:

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

In this tdoc, we discuss FFS topics based on RAN1 agreements [3] regarding whether additional information, alongside functionality applicability, needs to be reported to the network. Specifically, we address scenarios/issues where the UE may indicate its preferred configuration(s) to the network, while the network may also provide either preferred or default configurations to the UE, such as a preferred CSI-ReportConfig, within the functionality or inference configuration. Such information may enable the network in selecting and activating a stable configuration at the UE

Additionally, in this tdoc we discuss, and address challenges related to functionality (de)activation, particularly the impact of frequent changes in functionality applicability. These changes can lead to increased signalling overhead and negatively affect system performance, as well as the efficient management of AI/ML functionalities.

# Discussion on Functionality Activation and Reporting of Preferred or Default Configuration

In this section, the activation/deactivation of a functionality or configuration is discussed based on the following agreements in RAN2 #129 [1].

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

The provided periodic CSI configuration should be consistent with reported UE capabilities

FFS option B

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI

Fig. 1 depicts the signalling framework used as baseline in RAN2 discussion for beam management use case for UE-sided model:

Figure 1: Signalling flow for applicable functionality reporting

The final applicability of a functionality may be determined either by the UE or the network, depending on whether the UE has sufficient information to evaluate the functionality's applicability. If the UE has adequate information, such as full CSI-ReportConfig. (inference configurations), associated ID(s) or additional conditions, it may autonomously determine the applicability.

The potential for multiple functionality/inference configurations (e.g., CSI-ReportConfig.) provided to the UE, as well as the possibility of simultaneously activating multiple functionalities, are critical factors that must be incorporated to enhance flexibility and ensure optimal performance. In a beam management use case, the network can leverage AI/ML inference configurations to optimize beam prediction, selection and signal quality. These inference configurations or parameters can be communicated to the UE through AI/ML CSI-ReportConfigs.

As per the RAN 1 agreements [3], for beam management, multiple CSI-ReportConfigs. for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. This implies that each UE may be assigned multiple CSI-ReportConfig. instances, tailored for different reporting types, such as periodic or aperiodic or semi-persistent reporting.

In Step 4 (Fig. 1), the UE reports the applicability and inapplicability of one or more CSI-ReportConfigs. and/or sets of inference-related parameters. It is still to be determined whether any additional information, apart from the applicability, needs to be included in the applicability report. One or more of CSI-ReportConfig. for inference configuration (wherein the associated ID may be configured in CSI-ReportConfig. as a working assumption) is configured in Step 3, the applicable aperiodic and semi-persistent CSI reports can be activated or triggered by the network after receiving the applicability report from the UE using MAC CE or DCI indication [1]. For applicable periodic CSI reports, they are activated by the UE only if the applicability of the corresponding CSI-ReportConfig. is reported in the RRCReconfigurationComplete message to the network (as per RAN2 # 129 agreement [1]). In RAN2 129bis [2], following was agreed: Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly

Observation 1: If the UE reports a periodic CSI report configuration (i.e., a full inference configuration provided in CSI-ReportConfig.) as non-applicable, the UE is still required to retain all received full inference configurations—regardless of their applicability status—until they are explicitly released by the network via RRC signalling.

However, it is not clear what should be the UE behaviour when an applicable periodic CSI-ReportConfig. becomes inapplicable. In this case, it should be discussed if the UE can autonomously deactivate or de-configure the functionality or wait for the network to de-activate it.

Proposal 1: When an applicable periodic configuration/functionality (CSI-ReportConfig.) becomes inapplicable in the UE may autonomously de-activate it and report it to the network.

In Step 5, the network can optionally configure the CSI-ReportConfig. for inference configuration through the RRCReconfiguration message. The Step 5 may be optional if the UE has already been configured with the CSI-ReportConfig. in Step 3 and may only be applicable to semi-persistent and aperiodic CSI reporting case.

With multiple configurations present, both the network and UE may need to decide on a primary report or configuration to use at any given time.

To facilitate this selection process, the network can designate a default or preferred CSI-ReportConfig. during configuration (e.g., in step 3). This default setting helps ensure that the UE has a clear starting point for reporting in scenarios where multiple options exist, simplifying the decision-making process.

Proposal 2: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig. either in step 3 or 5 respectively.

As per RAN 1 agreements [3], In step 4, the UE reports applicability for all the above option A) one or more CSI-ReportConfig. and/or option B) set(s) of inference related parameters. It is FFS on whether/what other information along with the applicability is needed. Thus, in step 4 the UE may also indicate to the network its preferred configuration or parameter set. The network may consider UE’s preference when deciding which configurations/functionality to activate or deactivate.

A similar agreement was made for data collection aspects, where the UE may select a preferred configuration from a list of candidate configurations provided by the network.

Following similar approach, the UE can indicate its preferred configuration(s) to the network based on its applicability. This may help network in selecting/activating a stable configuration at the UE.

Proposal 3: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig.) in step 4.

# Frequent Activation-deactivation of functionality or configurations     when they become applicable or inapplicable

Another issue is how to manage and avoid frequent activation-deactivation of functionality or configurations when they become applicable or inapplicable respectively. When a functionality becomes inapplicable after activation, the UE is required to report it to the network. Frequent changes in applicability/inapplicability may lead to increased signaling from the UE to the network. If due to fluctuating conditions, the applicability of a functionality changes frequently, it may lead to the "ping-pong" phenomenon, where the functionality repeatedly activate and deactivate. This situation not only creates confusion and inefficiencies within UE but also negatively impacts the overall user experience. Furthermore, it can result in unnecessary signalling (e.g., frequent UAI updates from the UE) between the UE and the network, increasing the signalling overhead.

The frequent changes between different functionality statuses can significantly impact functionality LCM. These frequent transitions may require more resources for performance monitoring and may lead to repeated activation and deactivation, which results in higher signalling and resource overhead.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

From the network's perspective, it is preferable to choose functionalities for inference that are likely to remain stable and applicable for an extended period and are not prone to frequent status changes—particularly those affected by dynamic UE-side conditions. To assist the network in selecting the appropriate configurations, it would be beneficial for the UE to provide more than just the basic applicability information.

Specifically, the UE could send an indication called "applicable functionality stability index" or StabilityIndex. The StabilityIndex may help the UE and the network to evaluate and determine the stability of an applicable functionality for example, the probability of how long a functionality may remain applicable under given conditions. If the UE anticipates that a certain inference operation might for example, exceed its processing capacity or that a model is only applicable under specific conditions (such as the current UE speed), this could be reflected in StabilityIndex and indicated to the network.

The UE may compute a metric for e.g., applicable functionality index using for example, UE side internal conditions, environmental factors and other related factors. With the help of StabilityIndex, the UE determines probability (estimation) of how long a functionality maybe ‘applicable’ (ready to be activated) if not activated and how long it can stay activated once it is activated.

Using an StabilityIndex, the UE may evaluate and report to the network an estimate of the expected duration or probability with which a given functionality is anticipated to remain applicable, based on current UE conditions and operational context.

The StabilityIndex, along with parameters such as Estimated Applicability Duration and Applicability Probability, may be included in the UAI message or embedded within RRC message.

With this information, the network can make a more informed decision, balancing signalling/resource overhead and system performance guarantees. If the functionality’s applicability is expected to remain stable for a relatively long period, fewer resources would be needed for monitoring. On the other hand, if the applicability is uncertain or expected to change quickly, the network may decide not to activate the functionality or may choose to activate it with additional resources dedicated to performance monitoring.

By incorporating the StabilityIndex, the network can optimize functionality management, reduce unnecessary signalling, and improve the overall performance of the system.

Observation 3: Frequent functionality applicability changes or updates could negatively affect signalling overhead, system performance, and the network’s ability to manage AIML functionality effectively.

Proposal 4: RAN2 should explore methods to minimize the necessity for frequent applicability status updates facilitating the stability of inference operation once activated.

Proposal 5: The UE could provide a “StabilityIndex” metric or indication for each applicable functionality, for e.g., in UAI or RRC message, to enhance operational consistency and reduce signalling overhead.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Observation 1: If the UE reports a periodic CSI report configuration (i.e., a full inference configuration provided in CSI-ReportConfig.) as non-applicable, the UE is still required to retain all received full inference configurations—regardless of their applicability status—until they are explicitly released by the network via RRC signalling.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Observation 3: Frequent functionality applicability changes or updates could negatively affect signalling overhead, system performance, and the network’s ability to manage AIML functionality effectively.

Proposal 1: When an applicable periodic configuration/functionality (CSI-ReportConfig.) becomes inapplicable in the UE may autonomously de-activate it and report it to the network.

Proposal 2: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig. either in step 3 or 5 respectively.

Proposal 3: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig.) in step 4.

Proposal 4: RAN2 should explore methods to minimize the necessity for frequent applicability status updates facilitating the stability of inference operation once activated.

Proposal 5: The UE could provide a “StabilityIndex” metric or indication for each applicable functionality, for e.g., in UAI or RRC message, to enhance operational consistency and reduce signalling overhead.

# References

[1] Chair notes, RAN2 129 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, 2025

[2] Chair notes, RAN2 129bis 3GPP TSG-RAN WG2 Meeting #129bis, Wuhan, China, 2025

[3] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model