3GPP TSG-RAN2 Meeting #130	R2-2504222

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.2.4

Source:	Qualcomm Incorporated

Title:	Data Transmission and Other General Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made the following agreements regarding the A-IoT data transmission and general aspects in terms of AS ID, segmentation, MAC PDU format and etc.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

RAN2 #129bis made the following agreements regarding the A-IoT data transmission.

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

In this paper, we will discuss the data communication and other aspects of Ambient IoT (A-IoT).

# Background

In Rel-19, the traffic types of DO-DTT and DT with the focus on indoor inventory and command are considered. The service requirements on inventory procedure and command procedure should be supported from RAN perspective.

In inventory procedure, the A-IoT application in core network would like to discover the A-IoT device(s) which are nearby the Reader(s) and sends the inventory request to the Reader(s). The inventory request message may include filter criteria, and other upper layer information. The Reader that receives the inventory request performs the inventory procedure. The Reader forwards the inventory request to the A-IoT devices to be inventoried. The inventory request from Reader could be the group signaling which are sent to all A-IoT devices who have harvested enough energy and are able to receive the request.

In RAN2#125bis meeting, it has been agreed that the A-IoT access procedure is triggered by the Reader and Reader may provide the information that the A-IoT devices need to respond for the A-IoT access trigger. The A-IoT devices who are triggered by the inventory request will perform the A-IoT access towards the Reader in response to the received inventory request.

Figure 1 Ambient IoT inventory

In command procedure, the A-IoT application issues a Command Request including command container, (a list of) A-IoT device identifier(s) that the command is destined or filter criteria that identify the A-IoT devices that are supposed to act upon the command. Therefore, the trigger message sent from Reader may include the device information or filter criteria for triggering A-IoT devices to perform A-IoT access. The trigger message may also include the command container if A-IoT application provides commands to Reader in the command procedure.

# Discussion

## Segmentation

With the segment indication, Reader schedules additional resources for the subsequent segments’ transmission. Reader is responsible for recording the received segments and to estimate the remaining data size of the target A-IoT device. If an estimated D2R message size is provided by CN or AIOTF, Reader considers that as the assistance information to determine the TB size for subsequent segments.

When A-IoT device segments the A-IoT upper layer data, the size of segment can be determined by the TB size from the Reader. One possible way is that the size of segment is equal to the TB size. If the rest of the upper layer data is smaller than the scheduled TB size, padding bit(s) may be needed. Thus, the A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader.

A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader.

RAN2 #129bis has agreed that Reader sends the R2D message by including the upper layer command again for the retransmission of the first segment/unsegmented D2R message but FFS on the successful received data size field. The successful Received Data Size field is always zero in that case.

RAN2 #129bis also agreed that for the retransmission of the D2R segments FFS on whether the reader should always include the upper layer command. Because the A-IoT device should have context information after the first segment transmission and based on the indication of the successful received data size field.

So, some fields in R2D data message may be optional in different cases. Considering a simple A-IoT message format design and reducing too many optional fields with additional indication in the message format, we prefer the upper layer command and received data size field are always present in the R2D data message.

The upper layer command (Data SDU) and Received Data Size field are always present in R2D data message (i.e. R2D Upper Layer Data Transfer message in MAC running CR).

## R2D and D2R Data Message

MAC padding was agreed to be supported at least for D2R message from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. Since the size of D2R data or D2R segment could be variable, it is simple to have a length field to directly indicate the length of D2R data MAC SDU. The size of length field can be FFS.

A length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is FFS.

For the D2R and R2D data message, they target to schedule single A-IoT device or respond to one Reader. The MAC PDU payloads carrying D2R and R2D data message include the upper layer data and device ID/AS ID as well. Compared to the multiple MAC subPDU solution, one single MAC header for the MAC PDU can reduce the overhead caused by multiple MAC subheader. The length field and some optional assistance information can also be carried in the MAC header.

For R2D and D2R data message, a single MAC header is associated with multiple MAC subPDUs in an A-IoT MAC PDU.

## Logical and transport channels

In the A-IoT SI phase, it was agreed that legacy NR SDAP/PDCP/RLC protocol layers are not supported for A-IoT. In the WI scope [1], only MAC layer is included. For A-IoT MAC, not all legacy NR MAC functionalities are needed.

The NR logical channels are used to carry the control signaling and UP data. A-IoT data can be carried in A-IoT MAC PDU. In A-IoT MAC, upper layer signaling and A-IoT upper layer data can be mapped into one single logical channel and all MAC PDUs belonging to this one logical channel are concatenated in the single transport block to be delivered to the physical layer for transmission.

The concept of logical channels (similar to NR MAC) is also supported for A-IoT MAC.

Since RAN1 has agreed to support two physical layer channels, i.e., PRDCH and PDRCH, at least two transport channels mapping to physical channels should be supported in A-IoT MAC. Similar to NR DL-SCH and UL-SCH, A-IoT should support transport channels RD-SCH and DR-SCH.

At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

## A-IoT Device Registration, Subscription and CN Awareness

Due to very low complexity, cost as well as power consumption, and given that potentially a very large number of A-IoT devices may be present in a given vicinity but perform only bursty data transmissions as triggered by the Reader, one of the fundamental architectural differences between A-IoT and existing 3GPP technologies may be how the devices are exposed to the Core Network functions.

SA2 conclusions in TR [2] includes the following (highlighting added):

### 8.2.2 Subscription Management The AIoT device may or may not have operator’s subscription data in the network. If the AIoT device doesn’t have the operator’s subscription data in the network, the corresponding related data (e.g. Device ID or credentials) is stored in the AAA server external to the network. Subscription data for an Ambient IoT Device is required in case the Ambient IoT Device is managed by the network: - It is used by the AIOTF to check whether the device is subscribed. NOTE 1: Checking whether the device is subscribed needs coordination with SA WG3 study outcomes on validation of the ambient IoT Device identifier. - It is dedicated subscription data for Ambient IoT Devices, i.e. different with UE subscription data. - It contains at least a permanent Ambient IoT Device ID, etc. NOTE 2: SA WG2 will align security related materials in subscription data with SA WG3 during the normative phase. <<skip>>

From RAN2 point of view, we think requiring A-IoT devices to support legacy UE-registration-like procedures would be very ambitious not only because of the complexity of the procedure and protocol, but also due to the number of communication steps required. The A-IoT devices will not have enough energy source or energy storage to perform multiple steps just for the purpose of registration. Additionally, SA2 conclusions already indicate that the A-IoT ‘subscription’ is not similar to legacy UE subscriptions since per A-IoT device subscription information may be a high burden for the core network to manage.

The A-IoT device may harvest energy, accumulate it up to the required minimum amount of energy for communication, perform the communication steps and may fully deplete its energy. If the A-IoT device runs out of energy after some communication steps, it may not be able to store any configurations, i.e. it will likely ‘forget’ everything. In other words, it is plausible that every communication session starts as if the device has just been powered up from the power off state. Then, it would not make sense to have the A-IoT device perform the legacy registration procedures every time it needs to communicate.

From RAN2 perspective, legacy NR UE registration-like procedure is not supported by A-IoT device.

## NAS functions

NAS control protocol in NR performs the functions listed in TS 23.501 such as registration, authentication, mobility management, security control etc. Supporting all these legacy NAS functionalities by the A-IoT devices is not feasible especially given their complexity, cost, processing power and energy/power constraints.

In our view, the functionalities like security and authentication for A-IoT communication should be provided by A-IoT function (AIOTF) using A-IoT NAS protocol. We understand details are being discussed in SA2, SA3 and CT1.

From RAN2 perspective, legacy NR NAS functions are not supported by A-IoT device.

# Protocol stack

During the SI phase, it was agreed that based on the targeted lower complexity and cost of A-IoT devices, traditional Control Plane functions like RRC connection management, RRM L3 measurement reporting, ASN.1 encoding/decoding, legacy periodical SIB/MIB, legacy paging message, legacy RRC layer, legacy DRX, tracking area update from the device etc. will not be supported by A-IoT. Subsequently, RAN2#128 agreed:

=> RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only).   There is no CP/UP protocol stack differentiation on A-IoT interfaces.

Subsequently, the current draft stage-2 running CR [3] captures the following figure which is applicable to both A-IoT CP and UP (i.e. no differentiation between CP and UP protocol stacks).

Figure 2 A-IoT protocol stack (in current stage-2 running CR [3])

As some companies commented during the CR review process, there has been no explicit RAN2 agreement about the protocol layer above A-IoT MAC. We tend to agree with other companies which suggested that a more comprehensive figure should be captured in the stage-2 specifications. So, we propose the below figure replace the current figure in the stage-2 CR.

Figure 3 A-IoT protocol stack (to update the Figure 16.x.3-1 in the stage-2 running CR [3])

Update Figure 16.x.3-1 in the current stage-2 running CR to include the AOTF as well as A-IoT NAS layer (take Figure 3 from R2-2504222 as baseline).

# Conclusion

Based on the discussion above, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1:	A-IoT device segments and transmits the A-IoT upper layer data based on the scheduled TB size from the Reader.

Proposal 2:	The upper layer command (Data SDU) and Received Data Size field are always present in R2D data message (i.e. R2D Upper Layer Data Transfer message in MAC running CR).

Proposal 3:	A length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is FFS.

Proposal 4:	For R2D and D2R data message, a single MAC header is associated with multiple MAC subPDUs in an A-IoT MAC PDU.

Proposal 5:	The concept of logical channels (similar to NR MAC) is also supported for A-IoT MAC.

Proposal 6:	At least two transport channels, i.e., RD-SCH and DR-SCH are supported for A-IoT.

Proposal 7:	From RAN2 perspective, legacy NR UE registration-like procedure is not supported by A-IoT device.

Proposal 8:	From RAN2 perspective, legacy NR NAS functions are not supported by A-IoT device.

Proposal 9:	Update Figure 16.x.3-1 in the current MAC running CR to include the AOTF as well as A-IoT NAS layer (take Figure 3 from R2-2504222 as baseline).

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, 3GPP

- R2-2504398, Stage-2 38.300 Running CR for Ambient IoT, CMCC