3GPP TSG-RAN WG2 Meeting #130   								 R2-2504240

St Julian's, Malta, 19 - 23 May, 2025

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on LPP open issues for Case 1

Document for:	Discussion and decision

Introduction

After RAN2#129bis, the email discussion [POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm) was to collect companies' views on open issues.

In [1], some open issues have been identified and they may need more RAN2 discussions. In this paper, we provide our views for these issues.

Discussion

List of open issues

[Open Issues for further discussion (e.g., based company contributions and further RAN1 input)]

Open issue LPP-7: Applicability of NR-SelectedDL-PRS-IndexList

Open issue LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

Open issue LPP-13: Location server error causes

Open issue LPP-14: Target device error causes

[New Issues]

Open issue LPP-15: Applicability of Positioning Integrity to AI/ML positioning

Open issue LPP-16: Signalling of Monitoring Outcome

Open issue LPP-17: Signalling of "ground-truth label" information

Discussion on Open issue LPP-7

Background

The email discussion is copied as below:

Open issue LPP-7: Applicability of NR-SelectedDL-PRS-IndexList

Issue description:

In the case of assistance data for multiple NR positioning methods are provided, the IE NR-DL-PRS-AssistanceData shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData.

If the IE NR-DL-PRS-AssistanceData is applicable to NR AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideAssistanceData.

Status in running CR:

Captured as "Editor's Note" in IEs NR-SelectedDL-PRS-IndexList, NR-DL-TDOA-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData.

Relevant Agreements:

From RAN1#119 (see Annex in LPP running CR [1] for the full list of RAN1 and RAN2 agreements):

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: […]

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the NR-DL-PRS-AssistanceData and associated signalling is also applicable to NR AI/ML positioning Case 1. Since the NR-DL-PRS-AssistanceData can then be included in the assistance data for each NR positioning method (incl. NR AI/ML positioning), they need to be provided only once in the case of multiple NR positioning methods are requested and an index indicates the applicable configuration(s) for each method. There seems no reason why NR AI/ML positioning Case 1 should not follow the same logic.

Proposal 7:	The IE NR-SelectedDL-PRS-IndexList is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3, 6.5.10.1, and 6.5.11.1 can be removed.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Maybe | Given that we need to wait for RAN1 to understand the applicability of DL-PRS, PRS processing capabilities, on-demand PRS and other PRS related IEs etc, it is easier to keep the FFS also on this NR-SelectedDL-PRS-IndexList and resolve it all together as the PRS assistance details becomes clear from RAN1 features list. |
| Xiaomi | Yes |  |
| LG | No | The applicability of NR-SelectedDL-PRS-IndexList depends on whether a shared NR-DL-PRS-AssistanceData is used across multiple positioning methods. While this IE allows each method, including AI/ML positioning Case 1, to refer to a subset of the common PRS resources via index, it can only be used when the assistance data itself is shared. If AI/ML requires a distinct PRS assistance configuration, a separate instance may be needed and the index-based referencing may not apply. |

Rapporteur's Summary:

4 out of 6 responses generally agree that the IE NR-SelectedDL-PRS-IndexList is also applicable to NR AI/ML positioning Case 1. However, some concerns were raised:

- DL-PRS assistance data in general may depend on RAN1, and therefore, it may be better to keep the FFS/Editor's Note for now.

- If NR-DL-PRS-AssistanceData cannot be shared between the methods, the indexing scheme may not be applicable.

No Proposal is suggested for this issue and we keep the Editor's Note for now.

Analysis and proposal

For the IE NR-DL-PRS-AssistanceData, whether the UE needs to indicate the applicability or not depends on RAN1 progress.

Observation 1: Open issue LPP-7 is up to RAN1.

Discussion on Open issue LPP-10a

Background

The email discussion is copied as below:

Companies are requested to provide their view on whether batch reporting (i.e., up to 32 instances (location results) can be reported at once) should also be applicable to NR AI/ML positioning Case 1.

| Company | Applicable/Not Applicable | Comment/Reason |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | This may need new UE capability indication, but we could discuss it later. |
| ZTE | No | This batch reporting is a R17 enhancement rather than a fundamental function of a positioning method. So this should be explicited confirmed before adding to the running CR. Also this is not an essential function to be supported in AI/ML positioning. |
| Nokia | See comment | We think it is up to RAN1 to decide if there are any benefits in using batch reporting for AI/ML positioning also. |
| Xiaomi | See comment | Agree with Nokia that this is up to RAN1 whether batch reporting needs to be supported or not. |
| LG | Yes, but | We support applying batch reporting to NR AI/ML positioning Case 1, for consistency with other NR positioning methods defined in Rel-17. Batch reporting may also be beneficial in scenarios where the AI/ML model performs inference across multiple time instances. However, it may require a corresponding UE capability indication, which can be further discussed in RAN1 or RAN2. |

Rapporteur's Summary:

2 out of 6 responses agree that "batch reporting" should also be applicable to AI/ML positioning.

On the capability aspect mentioned in the comments, note that any request/provide field typically has a corresponding LPP capability. For batch reporting, this is the existing multiLocationEstimateInSameMeasReport (for UE-based) and multiMeasInSameMeasReport (for UE-assisted).

1 out of 6 responses do not agree that "batch reporting" should be applicable to AI/ML positioning, since considered as an enhancement.

2 out of 6 responses think this should be up to RAN1 to decide.

I suggest making this is separate open issue, which can then further be discussed.

Open issue LPP-10a: Applicability of "batch reporting" for AI/ML positioning.

Analysis and proposal

In Rel-17, "batch reporting" was introduced for DL-TDOA and DL-AoD positioning methods, which allows the target device to support multiple location estimate instances in a single measurement report.

In our opinion, the AI/ML based positioning may be able to provide location estimates in different time instances, and then "batch reporting" may be beneficial for network side. However, we also understand that some companies are concerned about the workload, and then "batch reporting" is not a high-priority feature for AI/ML based positioning. After checking the stage-2 and stage-3 running CRs for positioning, we also think that the workload of main functionalities is already high, so we think this "batch reporting" can be discussed once the main functionalities are completed.

Proposal 1: For Open issue LPP-10a, "batch reporting" for AI/ML positioning can be discussed once the main functionalities are completed.

Discussion on Open issue LPP-13

Background

The email discussion is copied as below:

Open issue LPP-13: Location server error causes

Issue description:

The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively.

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.

Status in running CR:

The IE NR-DL-AI-ML-LocationServerErrorCauses currently includes the error causes from DL-TDOA.

Captured as "Editor's Note" in IE NR-DL-AI-ML-LocationServerErrorCauses.

Proposed resolution:

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Analysis and proposal

In current TS 37.355, the error causes for DL-TDoA have been defined as below. We think they can be used as the baseline for AI/ML based positioning. New causes can be discussed case by case.

–	NR-DL-TDOA-LocationServerErrorCauses

The IE NR-DL-TDOA-LocationServerErrorCauses is used by the location server to provide NR DL-TDOA error reasons to the target device.

-- ASN1START

NR-DL-TDOA-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

...,

on-demand-dl-prs-NotSupportedByServer-v1700,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700

},

...

}

-- ASN1STOP

Observation 2: For Open issue LPP-13, the existing IE NR-DL-TDOA-LocationServerErrorCauses-r16 can be used as the baseline.

Discussion on Open issue LPP-14

Background

The email discussion is copied as below:

Open issue LPP-14: Target device error causes

Issue description:

The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively.

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.

Status in running CR:

The IE NR-DL-AI-ML-TargetDeviceErrorCauses currently includes the error causes from DL-TDOA.

Captured as "Editor's Note" in IE NR-DL-AI-ML-TargetDeviceErrorCauses.

Proposed resolution:

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Analysis and proposal

In current TS 37.355, the error causes for DL-TDoA have been defined as below. We think they can be used as the baseline for AI/ML based positioning. New causes can be discussed case by case.

–	NR-DL-TDOA-TargetDeviceErrorCauses

The IE NR-DL-TDOA-TargetDeviceErrorCauses is used by the target device to provide NR DL-TDOA error reasons to the location server.

-- ASN1START

NR-DL-TDOA-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA,

locationCalculationAssistanceDataMissing,

...

},

...,

[[

remoteUE-Indication-r18			ENUMERATED {true}		OPTIONAL		-- Cond NR

]]

}

-- ASN1STOP

Observation 3: For Open issue LPP-14, the existing IE NR-DL-TDOA-TargetDeviceErrorCauses-r16 can be used as the baseline.

Discussion on Open issue LPP-15

Background

The email discussion is copied as below:

Open issue LPP-15: Applicability of Positioning Integrity to AI/ML positioning

Issue description:

Per RAN1 agreement: "For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE."

The RAN1 agreement already includes info #15 for positioning integrity:

Data facilitating the integrity results determination of the calculated location

However, companies have different views on whether this implies support for integrity.

Analysis and proposal

For positioning integrity functionality, it was introduced in Rel-17, and  the target device can assess the correctness of information and assess the positioning estimation accuracy. The positioning integrity functionality is defined per positioning method.

Similar to open issue LPP-10a, the positioning integrity may be seen as an enhancement, so it can be discussed once the main functionalities are completed.

Proposal 2: For Open issue LPP-15, the positioning integrity for AI/ML positioning can be discussed once the main functionalities are completed.

Discussion on Open issue LPP-16

Background

The email discussion is copied as below:

Open issue LPP-16: Signalling of Monitoring Outcome

Issue description:

In terms of monitoring for AI/ML positioning, RAN1 agrees to support:

- Option A: The target UE side performs monitoring metric calculation. The target UE may signal the monitoring outcome to the LMF.

- the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

This includes all the sub-options A-1, A-2 and A-3. The details are upon RAN1 still, but there are expected RAN2 impact.

Analysis and proposal

For monitoring outcome, RAN1 should be still discussing the details, so it may be better to check them after May RAN1 meeting.

Observation 4: Open issue LPP-16 is up to RAN1.

Discussion on Open issue LPP-17

Background

The email discussion is copied as below:

Open issue LPP-17: Signalling of "ground-truth label" information

Issue description:

Following up on sub-option A-1 (see #LPP-16), RAN1 agreed that:

Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

The details on whether a new message or existing message is used to provide the ground truth label to the target UE is also expected to have RAN2 impact.

Analysis and proposal

In the current TS 37.355, the IE LocationCoordinateTypes has been defined, which is aligned with RAN1 view.

–	LocationCoordinateTypes

The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15].

In the running TS 37.355 CR, a new IE NR-AI-ML-PositioningProvideAssistanceData is introduced in the ProvideAssistanceData message. And we think the new IE can be used to carry ground truth label, and the IE LocationCoordinateTypes can be used as a starting point.

Proposal 3: For Open issue LPP-17, the existing IE LocationCoordinateTypes can be used as a starting point for indicating ground truth label. The new IE NR-AI-ML-PositioningProvideAssistanceData can be used to carry new IEs.

Conclusions

In this paper, we are addressing the remaining open issues for LPP. Here are our observations and proposals:

Observation 1: Open issue LPP-7 is up to RAN1.

Observation 2: For Open issue LPP-13, the existing IE NR-DL-TDOA-LocationServerErrorCauses-r16 can be used as the baseline.

Observation 3: For Open issue LPP-14, the existing IE NR-DL-TDOA-TargetDeviceErrorCauses-r16 can be used as the baseline.

Observation 4: Open issue LPP-16 is up to RAN1.

Proposal 1: For Open issue LPP-10a, "batch reporting" for AI/ML positioning can be discussed once the main functionalities are completed.

Proposal 2: For Open issue LPP-15, the positioning integrity for AI/ML positioning can be discussed once the main functionalities are completed.

Proposal 3: For Open issue LPP-17, the existing IE LocationCoordinateTypes can be used as a starting point for indicating ground truth label. The new IE NR-AI-ML-PositioningProvideAssistanceData can be used to carry new IEs.

References

[1] R2-250xxxx_([POST129bis][015][AI PHY] LPP Open issues Discussion_v08_Rap.docx