3GPP TSG-RAN WG2 #130	R2-2504255

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.11.2

Source:	Ericsson

Title:	SBFD RA remaining aspects

Document for:	Discussion

# 1	Introduction

In the WID [1], the following objective regarding random access operation has been defined for RAN2

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

We discuss the RAN2 aspects for the above objective in this paper. In the below, we use the term SBFD RA operation to stand for random access operation in SBFD symbols unless otherwise stated. We use the term legacy RA operation to stand for random access operation during non SBFD symbols.

In this paper, a SBFD aware UE means a UE which is capable of operating in a cell configured with SBFD feature, i.e., the cell/the gNB transmits DL and receive UL simultaneously in SBFD slots and symbols within a carrier. The UE can be aware of SBFD configurations so that the UE knows which slots/symbols are SBFD capable, which are also referred to as SBFD slots/symbols. This doesn’t mean that the UE needs to support full duplex operation.

In this document, we discuss some remaining RA aspects:

-	SBFD awareness early indication

-	LTM

-	SBFD MAC modell

-	RA-RNTI collision

# 2	Discussion

## 2.1	SBFD awareness early indication

Regarding SBFD awareness early indication, RAN2 has made the below agreement:

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The remaining issue is whether a SBFD aware UE can indicate SBFD awareness to the gNB via Msg3 or Msg5. We think this is not needed either due to below reasons:

- This would require an indicator to be introduced in either MAC layer or RRC signaling, which would increase unnecessary design efforts for RAN2.

- With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction would be marginal given that the UE will anyway provide its capability report to the gNB after the RACH procedure.

- The UE doesn’t choose SBFD RO for Msg1 transmission, meaning that the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension). In this sense, it is unnecessary for the gNB to schedule SBFD resources to the UE before the gNB receives the UE  capability from the UE or CN.

Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Therefore, in our view, it is unnecessary to introduce early indication for SBFD awareness. Anyway, an SBFD aware UE can be identified by the gNB at an early stage when the UE uses SBFD RO, which is sufficient.

SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

## 2.2	LTM

RAN2 has made the below agreement at RAN2#129bis

- Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

Regarding whether SBFD operation is supported for LTM, we have the below arguments for not supporting SBFD operation for LTM

- one additional indicator on RO type is not enough, more ASN.1 changes are needed.

- SBFD RACH configuration/resource needs to be configured in EarlyUL-SyncConfig if the indicator indicates SBFD RO

- the UE also needs to know SBFD T/F configuration of neighbour/target cell (which is signalled in SIB1 of the target cell. So the serving gNB/cell needs to signal SBFD T/F configuration of neighbour/target cel to the UE. Accordingly, the UE performs measurements towards neighbour cells according to the SBFD T/F configuration of neighbour cells. This results into cross-work group impacts, i.e., affect both RAN1, RAN3 and RAN4.

Due to limited TU left, we suggest to not support SBFD operation for LTM in this release.

Random access procedure in SBFD symbols is not supported for LTM.

## 2.3	SBFD MAC modelling

RAN2 has made the below working assumption at RAN2#129bis

- Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

RAN2 can just confirm the above working assumption to facilitate MAC spec drafting.

RAN2 to confirm that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

## 2.4	RA-RNTI collision

At RAN2#127bis, some companies have proposed to study the issue on RA-RNTI collision. They declare that:

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

In our view, we think the issue is invalid due to the below reasons

- According to RAN1 agreements,

a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE:

- The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH.

- The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH.

- UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols

- There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH.

- Other assumptions are not precluded.

NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

For a SBFD RO across SBFD symbols and non-SBFD symbols, if the SBFD RO is overlapping with the legacy RO on a 2nd symbol or beyond, there is no issue, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Regarding whether a SBFD RO across SBFD symbols and non-SBFD symbols, may be overlapping with a legacy RO in the first symbol, we think it can be either avoided by NW configuration, or RA-RNTI collision can addressed by allocating different preamble list to the legacy RACH configuration and additional RACH configuration. o

- In addition, a configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

RAN2 has discussed this issue, however no consensus has been reached. We have further queried our RAN1 colleagues and learned additional information the following:

RACH configuration option 2 is typically to be configured with different PRACH preamble format (e.g., longer PRACH preamble format) compared to the legacy PRACH configuration. Given this, the network can easily to detect PRACH preamble transmissions from UEs on the overlapping ROs, wherein the SBFD aware UE applies a longer PRACH preamble format while the legacy UE applies a shorter PRACH preamble format.

All in all, we see no issue for RA-RNTI confusion. Suggest RAN2 to conclude no further discuss is needed for this issue.

Update the RA-RNTI formula for SBFD RA configuration is not pursued.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	Support SBFD awareness early indication via Msg3 or Msg5 would require RAN2 to introduce an indicator in either MAC layer or RRC signalling, which would increase unnecessary design efforts for RAN2.

Observation 2	With early indication via Msg3 or Msg5, the gNB may be only feasible to schedule SBFD resources to subsequent signalling transmissions. The achieved latency reduction or other performance enhancements would be marginal given that the gNB will anyway receive the UE capability from UE or CN after the RACH procedure.

Observation 3	If the UE sees no need to apply SBFD RA (to exploit latency reduction or coverage extension), the UE would not choose SBFD RO for Msg1 transmission.

Observation 4	For a RO across SBFD symbols and non-SBFD symbols, the RO may be overlapping with a legacy RO on 2nd symbol or beyond, in such case, at least the s_id would be different, which would be sufficient to distinguish RA-RNTIs between the SBFD RO and the legacy RO which are overlapping.

Observation 5	A configured RO starting from non SBFD symbols and ending in SBFD symbols, is invalid.

Based on the discussion in the previous sections we propose the following:

Proposal 1	SBFD awareness early indication via Msg3 or Msg5 is not pursued for SBFD aware UE.

Proposal 2	Random access procedure in SBFD symbols is not supported for LTM.

Proposal 3	RAN2 to confirm that for SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 4	Update the RA-RNTI formula for SBFD RA configuration is not pursued.

# 4	References

- [1] 	RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei,

- Shanghai, China, June 17-20, 2024, RAN#104.