3GPP TSG-RAN WG2 #130					      		 R2-2504259

St. Julians, Malta – May 19 - 23, 2025

Agenda item:	8.13.2

Source: 	Kyocera

Title: 	Relay (re)selection under multihop relay

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key FFS issues to address for relay discovery and (re)selection under U2N multihop relay.

# Discussion

During RAN2#129bis, the following agreements were reached on discovery and relay (re)selection.  In this contribution, we address the FFSs that are highlighted below.

Agreements:

The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop.

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected).

When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE).  FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment).  This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message.

RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item.  This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI.

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

## 2.1	Relay (re)selection

2.1.1	RRC IDLE/INACTIVE scenarios

During the discussion in RAN2#129bis, it is already clear that under certain scenario(s) the parent U2N relay UE (e.g., intermediate relay UE) should be allowed to perform relay (re)selection.  However, it is not yet clear if and when an indication (aka notification) should be sent to the child UE (i.e., intermediate relay UE or remote UE), and if so whether the indication is different from that received from the parent U2N relay UE, for addressing the following FFSs.

When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE.

FFS detailed information in the indication and child UE handling.

FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

Since the child UE is also assumed to be in IDLE/INACTIVE, the reception of an indication from the parent U2N relay UE should be beneficial for the child UE’s relay reselection which may be followed by RRC Setup Request or RRC Resume Request if a suitable path can be found. As part of the relay reselection procedure, it is also an option for the child UE to release the PC5 connection to its parent U2N relay UE.

- Observation 1	For child UEs in IDLE/INACTIVE, the only benefit for the child UE to receive a notification/indication from its PC5 connected parent U2N relay UE, is for its use in performing relay reselection, followed by RRC Setup Request or RRC Resume Request if a suitable path can be found.

Then regarding the FFS below:

FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count.

The main benefit for suppressing the indication to the child UE is to reduce the burden for the child UE to perform relay reselection in case the parent U2N relay UE was able to successfully reselect a path to the gNB. Some companies were concerned that not sending the indication to the child UE immediately would delay the remote UE’s own relay reselection which is a valid point. However, one should also consider that the parent U2N relay UE is assumed to be closer to the gNB and should, in general, have a better chance of finding a suitable path to the gNB than the child UE.

- Observation 2	Since the parent U2N relay UE is assumed to be closer to the gNB and should, in general, have a better chance of finding a suitable path to the gNB than the child UE.

Assuming the parent U2N relay UE will anyway need to perform relay reselection when it detects PC5 RLF in the upstream (e.g., during Model A discovery procedure), it is questionable whether it is necessary for the parent U2N relay UE to send indication to the child relay UE immediately before the parent U2N relay UE has a chance to find a suitable path to the gNB.  If we mandate the parent U2N relay UE to always send indication to the child UE before it has a chance to perform relay reselection, and such an indication is propagated to multiple hops downstream, we would end up with many simultaneous relay reselection procedures which may not be desirable. Such an issue is not present in Rel-17, since only one relay UE is supported; therefore, adopting the legacy behavior isn’t always beneficial and may even increase the latency for the child UE to find a suitable path.

- Observation 3	Making it mandatory for the parent U2N relay UE to send indication immediately to its PC5 connected child UE for all downstream UEs will result in many simultaneous relay reselection procedures.

Assuming the parent U2N relay UE does not send indication to the child UE immediately, it should also be considered under what condition(s) the indication should be sent to the child UE. If the parent U2N relay UE’s relay reselection is completed successfully (e.g., it is PC5 connected to another last relay UE), the parent U2N relay UE shouldn’t need to notify the remote UE, at least for the case the number of hops to the gNB is not increased. And if the parent U2N relay UE has already received SRB0 message (RRC Setup Request) from the child UE during its relay reselection, the SRB0 message can be stored in the intermediate relay UE and forwarded to the last relay UE after relay reselection.

- Proposal 1	After successful relay reselection, the IDLE/INACTIVE parent U2N relay UE should not need to notify its PC5 connected child UE if the number of hops to the gNB has not increased.

In case the parent relay UE’s relay reselection is unsuccessful, it should be necessary for the parent relay UE to notify its PC5 connected child UE of the relay reselection failure which would trigger the remote UE to perform its own relay reselection. However, we should also consider that the parent U2N relay UE would continuously perform relay reselection until a path is found, even if no suitable path is readily available.  Therefore, unless we define a rule (e.g., a timer) for when relay reselection fails, the parent U2N relay UE may not consider relay reselection as failed and the indication would not be sent to the child UE. Considering the difficulty in defining a rule for when relay reselection fails, the parent U2N relay UE should be allowed the flexibility to decide when to send the indication to the child UE. For example, if the parent U2N relay UE is able to quickly find a suitable path to the gNB with the same number of hops (or fewer), there is no need to send an indication to the child UE.

- Proposal 2	The parent U2N relay UE that experiences PC5-RLF in the upstream should have the option to decide if and when to send an indication to its PC5 connected child UE.

It should be further clarified that it is always an option for the child UE to release the PC5 connection with the parent U2N relay UE as in legacy, so if for some reason the parent U2N relay UE doesn’t send an indication to the child UE for an extended period of time e.g., T300 expired associated with its SRB0 request, it can release the PC5 connection and perform relay reselection.

- Observation 4	Leaving the indication to the parent U2N relay UE implementation does not preclude the child relay UE from using PC5-S signalling to release its PC5 connection to the parent U2N relay UE.

As mentioned above, the following FFSs related to relay reselection also needs further discussion:

- FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment)

- “FFS detailed information in the indication and child UE handling”.

Currently, it is already agreed that the child UE (remote UE) in IDLE/INACTIVE triggers relay reselection under the following conditions:

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network, release of RRC connection between an intermediate relay UE and the network, PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

Two additional indications that may be worth considering for assisting the child UE are as follows:

- The number of hops from the parent U2N relay UE to the gNB.

- In the case of RLF, the link that failed (e.g., the L2ID of the relay UE that has caused the PC5/Uu RLF).

With 1), the number of hops to the gNB may be helpful for the case the parent U2N relay UE successfully reselects a new path, but with a larger number of hops than the original path, since the number of hops in the new path may exceed the maximum number of hops supported by the child UE. Currently, for Model A discovery [1], it is optional for the U2N relay UE to include the number of hops in the discovery message, so it is also another possibility to rely on upper layer to provide this information.

- Proposal 3	RAN2 should consider whether the parent relay UE should indicate to the child UE when the number of hops increased upon successful relay reselection.

With 2), the indication may include the L2ID of the relay UE that experienced either the PC5 RLF or Uu RLF.  In Rel-17 only the Uu RLF can be included in indication since there is only one relay UE.  The new indication in 2) can be provided to the child UE’s upper layer for deciding whether to release or keep the PC5-S connection with its parent U2N relay UE. Additionally, if the child UE initiates relay reselection, the information from 2) may assist the child UE with the selection of a suitable target parent U2N relay UE (e.g., a parent U2N relay UE that isn’t indicated in 2)).

- Proposal 4	RAN2 should consider whether the parent relay UE should indicate to the child UE the L2ID of the relay UE that experienced either the PC5 RLF or Uu RLF.

Then the third FFS “FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery)” considers whether indication is needed for the case when the parent U2N relay UE performs relay reselection.  Considering RAN2 has already agreed on the existing indications that can be used to inform the child UE, there is little incentive to rely on upper layer signalling, esp. since the likely link problem is associated with PC5 RLF in one of the links, RAN2 should continue to adopt the use of indications using the existing PC5-RRC signalling (i.e., NotificationMessageSidelink) to support all indications.

- Proposal 5	All indications to the child UE should use legacy PC5-RRC signalling.

2.1.1	RRC connected scenarios

For the RRC Connected scenario, the relay reselection initiation for the parent U2N relay UE is not much different from the case for the IDLE/INACTIVE scenario. For the RRC Connected child UE (i.e., also a remote UE) the main issue is whether it is always necessary for the child UE to send RRC Re-establishment message to the gNB regardless of which entity, the parent U2N relay UE or the child UE selected the new path. If the parent U2N relay initiates relay reselection towards the same cell with the same number of hops (or fewer), there is no need for the remote UE to send the RRC re-establishment request.  The parent U2N relay UE may need to inform the serving cell that it is still PC5 connected to a remote UE, but if the remote UE’s RRC re-establishment is not needed towards the same cell, it won’t be necessary for the intermediate relay UE to send notification/indication to the remote UE, since the existing SL-RLC configuration and SRAP configuration (i.e., Local ID) can be re-used for the remote UE and PDCP may be continued without re-establishment. It should be further considered if this is applicable not just for the same cell case, but also if the target cell belongs to the same gNB.  In Figure 1, the signalling flowchart illustrates the condition for the intermediate relay UE’s RRC re-establishment procedure, whereby it is assumed the notification to the remote UE can be skipped.

- Proposal 6	RAN2 should consider whether the remote should perform RRC re-establishment procedure if the intermediate relay UE completes the RRC re-establishment procedure towards the same cell with the same number of hops (or fewer), and whether the intermediate relay UE needs to notify the remote UE.

It may also be further discussed, if it’s necessary for the intermediate relay UE to inform the remote UE to suspend UL data transmission until its connection re-establishment is completed.

- Proposal 7	It may be further discussed whether it is necessary for the intermediate relay UE to notify the remote UE for the purpose of suspending UL data transmission prior to the intermediate relay UE’s completion of RRC re-establishment.

# Conclusion

In this contribution, we addressed the issues that may arise with the intermediate relay UE’s relay (re)selection procedure.  RAN2 is kindly asked to take into account the proposals below:

- Observation 1	For child UEs in IDLE/INACTIVE, the only benefit for the child UE to receive a notification/indication from its PC5 connected parent U2N relay UE, is to trigger it to perform relay reselection, followed by RRC Setup Request or RRC Resume Request if a suitable path can be found.

- Observation 2	Since the parent U2N relay UE is assumed to be closer to the gNB and should, in general, have a better chance of finding a suitable path to the gNB than the child UE.

- Observation 3	Making it mandatory for the parent U2N relay UE to send indication immediately to its PC5 connected child UE for all downstream UEs will result in many simultaneous relay reselection procedures.

- Proposal 1	After successful relay reselection, the IDLE/INACTIVE parent U2N relay UE should not need to notify its PC5 connected child UE if the number of hops to the gNB has not increased.

- Proposal 2	The parent U2N relay UE that experiences PC5-RLF in the upstream should have the option to decide if and when to send an indication to its PC5 connected child UE.

- Observation 4	Leaving the indication to the parent U2N relay UE implementation does not preclude the child relay UE from using PC5-S signalling to release its PC5 connection to the parent U2N relay UE.

- Proposal 3	RAN2 should consider whether the parent relay UE should indicate to the child UE when the number of hops increased upon successful relay reselection.

- Proposal 4	RAN2 should consider whether the parent relay UE should indicate to the child UE the L2ID of the relay UE that experienced either the PC5 RLF or Uu RLF.

- Proposal 5	All indications to the child UE should use legacy PC5-RRC signalling.

- Proposal 6	RAN2 should consider whether the remote should perform RRC re-establishment procedure if the intermediate relay UE completes the RRC re-establishment procedure towards the same cell with the same number of hops (or fewer), and whether the intermediate relay UE needs to notify the remote UE.

- Proposal 7	It may be further discussed whether it is necessary for the intermediate relay UE to notify the remote UE for the purpose of suspending UL data transmission prior to the intermediate relay UE’s completion of RRC re-establishment.

# References

- TS 23.304, “Proximity based Services (ProSe) in the 5G System (5GS)”.