# 3GPP TSG-RAN2 Meeting #130	R2-2504261

# Malta, May 19-23, 2025

Agenda Item:	8.2.2

Work Item:	Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Remaining aspects of Ambient IoT Paging

Document for:	Discussion/Decision

# Introduction

RAN#106 approved WI for Ambient IoT in [1]. One of the objectives for RAN2 is listed as follows:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

This document provides details about paging for Ambient IoT.

# Paging Message Content

## Msg1 resource indication and subsequent R2D/Paging message

RAN2 previously agreed in RAN2#126:

2. AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

RAN2#129bis further agreed to introduce a new R2D trigger message.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

It is understood from the prior agreements that the initial paging message can indicate multiple sets of Msg1 resource opportunities, and the start of the subsequent set(s) is indicated by the new R2D trigger message. Therefore, whether there is one or multiple set(s) of Msg1 resource opportunities should be indicated by the A-IoT paging message.

Whether there is one or multiple set(s) of Msg1 resource(s) is indicated by the A-IoT paging message.

Consequently, if the A-IoT paging message indicates only one set of Msg1 resources in the A-IoT paging round, there would be no need of any R2D trigger messages, as explained in our companion paper [4]. That is because the R2D trigger messages is used to indicate the start of the subsequent set(s) of A-IoT Msg1 resource(s), but not the first set.

If the A-IoT paging message indicates a single set of Msg1 resource(s), there is no need of R2D trigger message for that paging round.

In addition, there was some discussion about the naming of the messages during the email discussion. Particularly for the new message referenced above, rapporteur suggested to use ‘Access Occasion Trigger message’ while some other companies suggested to use ‘R2D trigger message’. We prefer to call it ‘R2D trigger message’ since that is easier to understand and has been used in RAN2 agreements before.

For the new R2D message indicating the start of a set of MSG1 resources that were configured in the paging message, keep the name ‘R2D trigger message’ (i.e. no need to change to ‘Access Occasion Trigger message’).

If the A-IoT paging message indicates a single set of Msg1 resource(s), R2D trigger message is not used for that paging round.

Similarly, as also described in detail in our companion paper [4], we think it is better to use more generic and commonly-understood MSG1 and MSG2 when describing the RA procedure, instead of the more restrictive names like Random ID message and Random ID Response message.

It is fine to call the MAC CE that carries Random ID as ‘Random ID MAC CE’. Then MSG1 contains Random ID MAC CE in addition to any other L1 items based on RAN1 agreements.

Use A-IoT Msg1 and A-IoT Msg2 as message names in the A-IoT CBRA procedure.

Finally, about the assumption in the previous agreement, the proponents argued that having some information about slot number/count down number in the R2D trigger message can help the device for power saving.

We agree power saving for device is important. However, we do not see how this can conclusively help on device power saving. To enable the device to go to ‘sleep’ and only wake up later to check the R2D trigger message and find out the slot number/counter, there would need to be prior knowledge about the periodicity of the R2D triggers. RAN2 hasn’t agreed on such mechanism. Even when the periodicity is known, there is uncertainty about the clock accuracy. Consequently, the device would have to determine when to wake up to check. If the device has ability to make this determination even without the knowledge of periodicity, then it could use the proprietary power saving strategies without such counter. If the device doesn’t have such ability, and there is still a chance to miss the R2D trigger message, then having the counter doesn’t help much.

Therefore, we have the following proposal:

RAN2 confirms that R2D trigger message does not include slot number/count down number.

## Indication about subsequent R2D message

The subsequent R2D message after D2R transmission may be a failure indication for Msg3 transmission or could be the acknowledgement like the Msg4 of NR random access. RAN2#128 agreed the following:

Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

While some scenarios would need the subsequent R2D message (e.g. for acknowledging the reception of the A-IoT device ID and/or A-IoT upper layer data sent in the previous D2R message or for triggering the next round of A-IoT re-access or A-IoT data communication), in other scenarios there would be no such subsequent R2D message (e.g. Reader requesting information from temperature sensors and no intention to ACK/NACK the reception).

It is beneficial for A-IoT devices to have correct expectation on potential future steps after transmitting a D2R communication in response to the A-IoT paging. To facilitate the A-IoT device actions, the A-IoT paging message can indicate to the A-IoT device whether a subsequent R2D message would be present, e.g. for a follow-up command.

The A-IoT paging message indicates the presence and information carried in the subsequent R2D message after D2R transmission (e.g. ACK/NACK feedback indication, follow-up command).

  - Indication of Device ID in D2R

Suppose the AIOTF has some idea about the A-IoT devices in a given area (e.g. based on deployment information within a warehouse) but does not have the most up to date inventory information. In such case, the AIOTF may want to initiate an inventory procedure. There is benefit of allowing to send a single Paging message in the R2D link to indicate to the sensors (satisfying the criteria) to reply with their temperature values in the D2R. In such scenario, the device ID may be absent in the D2R message. Then the device ID in D2R response should be optional and when to include it should be indicated by Reader in the Paging message.

In some scenarios, the device ID may be absent in the D2R message in response to the A-IoT paging. When to include it should be indicated by Reader in the A-IoT paging message.

The A-IoT Paging Message indicates whether A-IoT Device ID needs to be included in D2R message.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	If the A-IoT paging message indicates a single set of Msg1 resource(s), there is no need of R2D trigger message for that paging round.

Observation 2.	In some scenarios, the device ID may be absent in the D2R message in response to the A-IoT paging. When to include it should be indicated by Reader in the A-IoT paging message.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Whether there is one or multiple set(s) of Msg1 resource(s) is indicated by the A-IoT paging message.

Proposal 2:	For the new R2D message indicating the start of a set of MSG1 resources that were configured in the paging message, keep the name ‘R2D trigger message’ (i.e. no need to change to ‘Access Occasion Trigger message’).

Proposal 3:	If the A-IoT paging message indicates a single set of Msg1 resource(s), R2D trigger message is not used for that paging round.

Proposal 4:	Use A-IoT Msg1 and A-IoT Msg2 as message names in the A-IoT CBRA procedure.

Proposal 5:	RAN2 confirms that R2D trigger message does not include slot number/count down number.

Proposal 6:	The A-IoT paging message indicates the presence and information carried in the subsequent R2D message after D2R transmission (e.g. ACK/NACK feedback indication, follow-up command).

Proposal 7:	The A-IoT Paging Message indicates whether A-IoT Device ID needs to be included in D2R message.

# References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, Dec 2024

[2] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[4] R2-2504221, Views on Random Access Aspects of Ambient IoT, Qualcomm Incorporated, RAN2#130, May 2025