3GPP RAN WG2 Meeting #130	R2-2504262

St.Julians, Malta May 19th – 23rd, 2025

Agenda Item:	8.6.2

Source:	Ofinno

Title:	Discussion on open issues of inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss open issues on inter-CU LTM.

# Identified open issues:

## SN-key generation in MCG LTM with SCG (RRC-7)

A UE can be configured with candidate cells for MCG LTM. Some of these candidate cells may belong to the same CU as serving cell while others may belong to a different CU. Further, some LTM candidate configurations of the candidate cells may consist of new SCG configuration while others may simply keep or release the existing SCG configuration (i.e., SCG configuration on the UE at the time of an LTM cell switch to the candidate cell).

When UE executes MCG LTM cell switch to a candidate cell, the UE derives new security keys for the MCG (i.e., performs security key update in MCG) if the Rel-19 ID is different between the source cell and the candidate cell i.e., the target cell. RAN2 agreed that network configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

UE may execute a first MCG LTM with SCG to a first candidate cell resulting in UE performing security key update in MCG and generating SN key, for adding SCG, using sk-Counter in the LTM candidate configuration of the first candidate cell. However, LTM allows for subsequent cell switches without receiving an RRC reconfiguration message between the cell switches. This can lead to a case where UE executes a second MCG LTM with SCG to the first candidate cell resulting in UE performing security key update in MCG and generating SN key, for adding SCG, using the same sk-Counter that was used during the first MCG LTM with SCG i.e., sk-Counter in the LTM candidate configuration of the first candidate cell.

Observation 1: (RRC-7) Configuring only one sk-Counter in the LTM candidate configurations for MCG LTM with SCG leads to reuse of sk-Counter during subsequent LTMs.

Reusing the same sk-Counter for the same SN is not used for other cases e.g., SCPAC, SCG LTM etc. We think in MCG LTM with SCG, the UE should maintain the same principle and avoid reusing the same sk-Counter for multiple LTM cell switches. We think a list of sk-Counters can ensure that the sk-Counter is not reused and an unused value of sk-Counter is used for generating SN-key for each instance where the SCG is added as a results of MCG LTM with SCG. There are multiple ways in which this can be achieved. In a first option, the network configures a list of sk-Counter in all LTM candidate configurations, and UE uses the first entry in the list of sk-Counter for generating the SN key when security key update is performed in MCG. UE removes the entry from the list of sk-Counter once it is used to generate the SN key. In this option, a separate list of sk-Counter is signalled for each candidate cell.

In a second option, the network configures a list of sk-Counter corresponding to each R19 ID of candidate cells for MCG LTM, and UE uses the first entry in the list of sk-Counter corresponding to the R19 ID of the MCG candidate cell, for generating the SN key when security key update is performed in MCG. UE removes the entry from the list of sk-Counter once it is used to generate the SN key. In this option, a separate list of sk-Counter is required only for each CU on which candidate cells configured to the UE belong to. Thus, the number of sk-Counter list signalled is much lesser than that of first option.

If UE is configured with LTM on SCG, then the UE already maintains a separate list of sk-Counter corresponding to the R19 ID of an SCG candidate cell for SCG LTM. This is in addition to the sk-Counter lists, that are explained above, for MCG LTM. In a third option, the signalling for sk-Counter list can further be reduced as compared to that of the second option. According to the third option, the network can reuse the sk-counter list provided for SCG LTM. The network configures the R19 ID of SCG candidate cell for SCG LTM. The UE uses the list of sk-Counter corresponding to the R19 ID of the SCG candidate cell, to generate the SN key when security key update is performed in MCG.

Proposal 1: (RRC-7) For MCG LTM with SCG, network configures a list of sk-Counter based on of the of the following options:

Option 1: network configures a list of sk-Counter in all LTM candidate configurations, and UE uses the first entry in the list of sk-Counter for generating the SN key when security key update is performed in MCG.

Option 2: network configures a list of sk-Counter corresponding to each R19 ID of candidate cells for MCG LTM, and UE uses the first entry in the list of sk-Counter corresponding to the R19 ID of the MCG candidate cell, for generating the SN key when security key update is performed in MCG

Option 3: network configures a R19 ID of an SCG candidate cell in all LTM candidate configurations for MCG LTM. The UE uses the first entry in the list of sk-counter corresponding to the R19 ID of the SCG candidate cell, for generating the SN key when security key update is performed in MCG.

Proposal 2: (RRC-7) UE removes the entry from the list of sk-Counter once it is used to generate the SN key.

## Fast failure recovery (RRC-8)

RAN2 meeting #129bis agreed that UE performs fast failure recovery if the selected candidate cell has the same Rel-19 ID as the source cell. RAN2 also agreed that the UE performs fast failure recovery after a reconfiguration with sync failure (i.e., T304 expiry) to the target cell if the selected candidate cell has the same Rel-19 ID as the target cell.

If the selected candidate cell has a different Rel-19 ID as the source cell, following an RLF on the source cell, the UE does not have NCC to perform MCG security key update for the selected candidate cell.

Proposal 3: (RRC-8): Fast failure recovery to a candidate cell with different Rel-19 ID (from the serving/ target cell) is not supported.

# Other open issues:

## Eary TA acquisition of SCG within MCG LTM

For a candidate PSCell of SCG LTM, an early TA acquisition is supported. A UE may skip a random access procedure toward the candidate PSCell using a TA acquired by the early TA acquisition procedure. It reduces a switching time of a SCG LTM cell switch procedure for the candidate PSCell. However, when a UE applied a SCG configuration within MCG LTM configuration. the UE need to perform a random access procedure toward a candidate PSCell of SCG within MCG LTM.

Observation 2: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for a candidate PSCell of SCG within MCG LTM.

For example, the candidate PSCell of SCG within MCG LTM may be of SCG LTM. The UE may acquire a TA value of the candidate PSCell based on UE based TA measurements. The UE may use the TA value of the candidate PSCell to access to the candidate PSCell. It is beneficial to use the TA of the candidate PSCell for access to the candidate PSCell.

Proposal 4: RAN2 discuss whether to support early TA acquisition for a candidate PSCell of SCG within MCG LTM.

## CG resource for LTM

A network may postpone allocating a RACH resource for LTM until transmitting an LTM cell switch command indicating the RACH resource. However, the network may allocate a configured grant (CG) resource when transmitting to a UE an LTM configuration. The network may need to keep allocating the CG resource of a candidate cell until the UE execute an LTM cell switch procedure for the candidate cell. The UE may not execute the LTM cell switch procedure for the candidate cell. The CG resource is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Observation 3: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

The network may need to release or update the CG resource (e.g., to avoid the waste of the CG resource). For releasing or updating the CG resource, the network needs to transmit an RRC reconfiguration message comprising an LTM configuration. It causes signal overheads and is not desired for subsequent LTM without the RRC reconfiguration.

## Proposal 5: RAN2 discuss dynamic management of CG resource of candidate cell for LTM.

## Other issues on fast failure recovery

According to the section 5.3.7.3 of TS 38.331, for LTM fast recovery, a UE is allowed to use a configuration for MCG LTM with SCG. RAN2 needs to discuss whether a configuration for MCG LTM with SCG is considered for an LTM fast recovery. For CHO, the candidate cells for which condExecutionCondPSCell is configured is not considered for CHO based failure recovery, as indicated in the blue highlight below. We think a similar procedure needs to be adopted for LTM as well.

#### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: … 1> if attemptCondReconfig is configured; and 1> if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and 1> if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig: 2> if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell; 2> apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3; 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> if the selected cell does not have the field ltm-NoSecurityChangeID configured and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 2> if the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 2> if the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the seleted cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected: 3> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

Proposal 6: LTM candidate configuration for MCG LTM with SCG is not considered for LTM fast recovery.

# Conclusions

The contribution discussed aspects related to remaining open issue for inter-CU LTM. The following observations and proposals are made:

For identified open issues:

Observation 1: (RRC-7) Configuring only one sk-Counter in the LTM candidate configurations for MCG LTM with SCG leads to reuse of sk-Counter during subsequent LTMs.

Proposal 1: (RRC-7) For MCG LTM with SCG, network configures a list of sk-Counter based on of the of the following options:

Option 1: network configures a list of sk-Counter in all LTM candidate configurations, and UE uses the first entry in the list of sk-Counter for generating the SN key when security key update is performed in MCG.

Option 2: network configures a list of sk-Counter corresponding to each R19 ID of candidate cells for MCG LTM, and UE uses the first entry in the list of sk-Counter corresponding to the R19 ID of the MCG candidate cell, for generating the SN key when security key update is performed in MCG

Option 3: network configures a R19 ID of an SCG candidate cell in all LTM candidate configurations for MCG LTM. The UE uses the first entry in the list of sk-counter corresponding to the R19 ID of the SCG candidate cell, for generating the SN key when security key update is performed in MCG.

Proposal 2: (RRC-7) UE removes the entry from the list of sk-Counter once it is used to generate the SN key.

Proposal 3: (RRC-8): Fast failure recovery to a candidate cell with different Rel-19 ID (from the serving/ target cell) is not supported.

For other open issues:

Observation 2: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for a candidate PSCell of SCG within MCG LTM.

Proposal 4: RAN2 discuss whether to support early TA acquisition for a candidate PSCell of SCG within MCG LTM.

Observation 3: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

## Proposal 5: RAN2 discuss dynamic management of CG resource of candidate cell for LTM.

Proposal 6: LTM candidate configuration for MCG LTM with SCG is not considered for LTM fast recovery.