3GPP RAN WG2 Meeting #130	R2-2504270

St.Julians, Malta May 19th – 23rd, 2025             	revision of R2-2502604

Agenda Item:	8.10.2

Source:	Ofinno

Title:	Discussion on MRO for LTM

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss open issues on MRO for LTM.

# Open issues:

## Measurement information in MRO

An LTM may be performed based receiving a cell switch command MAC CE from network, in response to UE transmitting a measurement report. The measurement report may either be L3 measurement report or L1 measurement report. Current specifications, TS 38.331, supports logging (i.e., storing) of RLF information/ SHR information on detecting RLF/ SHR. This logged information contains the measurement results of the serving cell based on L3 measurements. RAN2 already agreed that for LTM, UE logs available L1 measurement results for the serving cell. However, when logging the measurement results, there is no indication whether the logged measurement is based on L1 measurement result or based on L3 measurement result. Without this information, the network cannot properly adjust its measurement configuration/ cell switch configuration for LTM. To properly adjust this, it is beneficial for the network to know the kind of measurements performed (i.e., L3 measurement/ L1 measurement) for target cell and neighbor cells.

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

## RLF Report

In release 18, an LTM is supported between intra-CU cells. The LTM can support mobility with reduced interruption time. Early UL synchronization is a big component of the LTM that helps in reducing the interruption time during the LTM cell switch. For early UL synchronization, a UE receives TA value of a candidate cell via a cell switch command MAC CE. On receiving TA and considering that CG is configured for the candidate cell, the UE performs an RACH less LTM cell switch procedure to access to the candidate cell. The LTM cell switch procedure may still fail resulting in a handover failure and the UE may generate RLF report. However, sending the RLF report indicating failure of the LTM cell switch procedure does not provide the complete useful information to the network. The network may not have information whether UE received TA for the candidate cell to which the LTM cell switch procedure failed. The failure may be due to wrong TA, weak candidate cell etc. To adjust the LTM related parameters correctly, the knowledge of whether the UE detecting the LTM failure received TA or not is helpful.

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

In the current specification, TS 38.331, on detecting an RLF, a UE stores the RLF information in an RLF report. However, the UE does not store the RLF information for all the RLFs that are detected by the UE. The UE stores them only if the UE has the capability to store the RLF information for the RLF report. At the time of the RLF, if a measurement result for a candidate cell for CHO is available, then the UE stores CHO related information in the RLF report only if the UE supports the RLF report for CHO. Similarly, the UE stores DAPS related information in the RLF report only if the UE supports the RLF Report for DAPS. An LTM is independent to any other mobility mechanisms already supported in the specification. In order to store any LTM information in the RLF report, a new capability to indicate a UE supporting storing of LTM information in the RLF report needs to be introduced.

Proposal 4: Introduce a UE capability for indicating whether a UE supports an RLF report for LTM.

## SHR Report

RAN2 already agreed to include an explicit indicator in a SHR report whether the successful LTM execution was RACH-less or RACH-based. This is introduced with the intention to enable the network to properly adjust the configuration associated with access the target cell. This information, although useful, seems limited and in incomplete. For example, if a UE performs a RACH-less LTM cell switch procedure to access to the LTM target cell but the time elapsed for T304 is greater than the configured threshold (i.e., configured percentage of T304), the UE triggers the SHR report indicating RACH-less access. Limited to this information, what the network can adjust in its configuration, is very limited. For example, even if the network has provided the UE with a configured grant, it does not know if the UE used it. If the UE transmitted via a dynamic grant, the network does not know that the CG is not valid. The network does not know which SSB associated with the CG is validated by the UE. We think more information for RACH-less access has to be logged by the wireless device in the SHR report.

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

- Explicit indication whether RACH-less access is based on CG or DG

- Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

RAN2 already agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in a SHR report to indicate whether a neighbour cell is an LTM candidate cell or not. We think there are multiple ways to implement this. For example, for each neighbor cell for which measurement result is available and that is an LTM candidate cell, introduce an explicit indication indicating the neighbor cell is an LTM candidate cell. In another example, L1 measurement results can only logged for LTM candidate cells. Based on the logged L1 measurement results (e.g., based on proposal 2) in the SHR report, network can identify if the neighbor cell is an LTM candidate cell or not.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not.

A TA acquisition based on early UL synchronization and LTM cell switch are 2 independent procedures. A UE may perform early UL synchronization with a candidate cell but can receive a cell switch command MAC CE sooner or later. Based on how long it takes for the cell switch command MAC CE to be received after performing early UL synchronization to the candidate cell (i.e. time elapsed from performing early UL synchronization to receiving cell switch command MAC CE, the TA value of the candidate cell may be valid or invalid (i.e., the UE may or may not receive the TA value). This may also have impact on the measurement configuration associated with the measurement reporting of candidate cell.

Proposal 8: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.

# Conclusions

The contribution discussed aspects related to open issues for MRO for LTM. The following observations and proposals are made:

For measurement information in MRO

Proposal 1: Indicate in RLF report/ SHR report whether the LTM to the target cell is based on L3 measurement or L1 measurement.

Proposal 2: Indicate in RLF report/ SHR report whether the neighbor cell (i.e., LTM candidate cells) measurements is based on L3 measurement or L1 measurement.

For RLF report:

Proposal 3: Indicate in RLF-report whether the UE received TA from cell switch command for the candidate cell.

Proposal 4: Introduce a UE capability for indicating whether a UE supports an RLF report for LTM.

For SHR report:

Proposal 5: Indicate whether RACH-less access is based on CG or DG based on one of:

- Explicit indication whether RACH-less access is based on CG or DG

- Explicit indication whether CG for RAHC-less access is valid or invalid

Proposal 6: Indicate index of SSB where the CG is determined to be valid/ invalid.

Proposal 7: Indicate whether a neighbour cell is an LTM candidate cell or not.

Proposal 8: A UE logs the time elapsed between receiving PDCCH order for triggering RACH (for early UL synchronization) on a candidate cell, and receiving LTM cell switch command MAC CE for cell switch to the same candidate cell.