3GPP TSG-RAN WG2 Meeting #130	R2-2504279
Malta, May 19 – 23, 2025

Agenda Item:	8.5.4

Source:	InterDigital

Title:	Time domain adaptation of common signalling and channels

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on adaptation of common signalling and channels:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In RAN2#129, the following agreements were made for RACH domain adaptation

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

This contribution discusses time domain adaptation mechanisms for SSBs, PRACH, and paging in the time-domain, with focus on higher layer impacts and assumptions.

# Discussion

## Adaptation of paging

RAN2 agreed to extend the values of N to have increased interval between PFs (e.g. N = T/32 and FFS on T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. In order to limit RRC overhead, it was agreed to limit the value of newly introduced NS values (e.g., Ns=8 and FFS for other larger values)

One consideration to take into account for this agreed option for paging adaptation is the ability for the network to provision additional PRACH resources for the additional PMOs, to accommodate the additional compressed paging load within the PF. Another R19 NES objective however provides means for additional PRACH resources, as explain in 2.2.

Proposal 1:	RAN2 to consider provisioning additional PRACH resources, e.g. provided in R19 NES PRACH adaptation, to accommodate the increased number of PMOs/paging load within the PF.

## Adaptation of PRACH in time domain

The adaptation of both the RO periodicity and the number of ROs can be beneficial for energy savings since they both allow conserving energy associated with reception at NW, e.g. during low load scenarios, and quick provisioning of the resources when needed.  The network can save energy in the uplink direction by reducing the amount of power needed to blind decode on PRACH resources.

In this regard, the NES gains can be enhanced by configuring an energy efficient periodicity for the RACH occasions, which can then be complemented by varying the RACH resources periodicity (e.g. PRACH occasions). The network can adapt based on additional PRACH resources for NES-capable UEs in addition to existing PRACH resources for legacy UEs. The network can provide additional PRACH resources for NES-capable UEs in addition to existing PRACH resources for legacy UEs, where the configuration contains an applicable periodicity or pattern for the applicable PRACH occasions.

RAN1 has already agreed to support adaptation of PRACH in the time domain. For NES-capable UEs requiring additional PRACH resources, the configuration is provided by semi-static signaling, where the discussion on addressing the overlap with legacy PRACH resources is ongoing in RAN1. NES-capable UEs are allowed to use both additional and legacy PRACH resources. Additionally, support for SSB-RO mapping for the additional PRACH resources should be separate from that of the legacy PRACH resources, with the legacy SSB-RO mapping rule assumed to be applied.

Observation 1:	RAN1 agreed to support adaptation of PRACH in time-domain based on configuring additional PRACH resources for NES-capable UEs in addition to existing PRACH resources for legacy UEs. Those resources are activated by DCI addressed to the UE’s P-RNTI.

PRACH adaptation is applicable to all RRC states, including RRC Idle, Inactive, and connected states. If limited to connected state, the NES gain is eliminated anyway as the network would have to transmit those SSBs for Idle mode UEs. RAN1 agreed to support PRACH time domain adaptation in Idle, Inactive, and connected modes.

Observation 2: 	Time-domain PRACH adaptation is applicable in RRC Idle, Inactive, and connected states.

In the NR RACH configuration framework, cell-common CBRA resources configured in BWP-UplinkCommon in SIB1 can include common 4-step RA resources in rach-configCommon and additional resources for early indication for feature-combination indicatin (additionalRACH-ConfigList-r17).

In RAN2#129, it was agreed that from R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. It was also agreed that the UE will follow legacy mechanism regarding how to select RACH resource.

This can be realized by enabled the additional PRACH resources if activated by lower layers, which can be captured in MAC in terms of PRACH resource availability. Annex A provides a text proposal for capturing PRACH domain adaptation in MAC.

Proposal 2:	From a MAC perspective, additional PRACH resources are considered available if a PDCCH activation indication is received by lower layers; otherwise not considered available.

# Conclusion

This contribution addresses time domain adaptation mechanisms for SSBs, PRACH, and paging in the time-domain, with focus on higher layer impacts and assumptions. The following proposals are provided:

Proposal 1:	RAN2 to consider provisioning additional PRACH resources, e.g. provided in R19 NES PRACH adaptation, to accommodate the increased number of PMOs/paging load within the PF.

Observation 1:	RAN1 agreed to support adaptation of PRACH in time-domain based on configuring additional PRACH resources for NES-capable UEs in addition to existing PRACH resources for legacy UEs. Those resources are activated by DCI addressed to the UE’s P-RNTI.

Observation 2: 	Time-domain PRACH adaptation is applicable in RRC Idle, Inactive, and connected states.

Proposal 2:	From a MAC perspective, additional PRACH resources are considered available if a PDCCH activation indication is received by lower layers; otherwise not considered available.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

# Appendix A: Availability of additional PRACH resources in MAC

### 5.1.1c	Availability of the set of Random Access resources

The MAC entity shall for each set of configured Random Access resources:

1>	if eRedCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which eRedCap is not applicable.

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT by MO-SDT as specified in TS 38.331 [5].

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if msg1-Repetitions is set to true for a set of Random Access resources:

2>	if Msg1 repetition is not applicable to the current Random Access procedure; or

2>	if the set of Random Access resources is not associated with any of the Msg1 repetition number that is applicable to the current Random Access procedure:

3>	consider the set of Random Access resources as not available for the Random Access procedure.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

1>	if a set of Random Access resources is configured with [RRC name TBD: additional PRACH resources]:

2>	if PDCCH addressed for the P-RNTI is received from lower layers indicating the availability of the Random Access resources; or

2>	if a PDCCH order is received from lower layers with the [TBD DCI field name] indicating the availability of the Random Access resources:

3>	consider the set of Random Access resources as available.