3GPP TSG-RAN2 Meeting #130	R2-2504291

St Julian’s, Malta, 19 – 23 May 2025

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson, Huawei, HiSilicon, NEC, LGE, Apple, InterDigital, OPPO, CATT

Title:  	LP-WUS and secondary DRX

Document for:	Discussion and Decision

# Introduction

A way forward for LP-WUS and secondary DRX is discussed in this contribution.

# Background

RAN1

RAN plenary confirmed that FR2 is within the scope of the LP-WUS WID [1,2]. And RAN1 reached agreements specifically for LP-WUS in FR2, e.g. support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

RAN1 discussed possible ways to support LP-WUS with secondary DRX (option 1-1) but did not reach consensus, see appendix for further details.

RAN2

RAN2 also discussed how and whether to support LP-WUS with secondary DRX. But RAN2 did not reach agreement and will discuss the topic again:

FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

# Discussion

## A quick reminder on secondary DRX

When Carrier Aggregation (CA) is configured the Cell Group can be divided into two DRX groups: a default DRX group (e.g. FR1 cells) and a secondary DRX group (e.g. FR2 cells). The secondary DRX group can be configured with a separate (and shorter) on-duration timer and a separate (and shorter) drx-inactivity timer compared to the default group. Such a configuration enables the FR2 cells, which are more power consuming compared to the FR1 cells, to go to sleep more quickly compared to the FR1 cells:

The DRX operation is independent between the default and secondary DRX group, but the onduration starts at the same time in both DRX groups, i.e. at the beginning of the DRX cycle.

## Proposed way forward

When LP-WUS is configured with secondary DRX then the following configuration options can be identified:

- LP-WUS configured on PCell in FR1 only

- LP-WUS configured on an SCell in FR2 only

- LP-WUS configured on PCell in FR1 and configured on an SCell in FR2

For option 1, 2 and 3 the following observation can be made when LP-WUS is detected:

- With option 1 and 2 there are two possible behaviours:

  - LP-WUS wakes-up all the cells of the DRX group where it is configured, or

  - LP-WUS wakes-up all the cells of both DRX groups

- With option 3 the LP-WUS wakes up all the cells of the DRX group where it is configured.

When the UE is monitoring LP-WUS outside Active Time and there is a new data burst, then it is beneficial to wake-up the all the cells of both DRX group, i.e. to obtain maximum throughput.

Companies have expressed concerns with option 3, i.e. to support one LP-WUS on FR1 and another LP-WUS on FR2. There is additional complexity to support option 3 while option 3 provides little benefit compared to option 1 and 2 when the LP-WUS wakes up all the cells of both DRX groups.

Furthermore companies expressed concerns to support LP-WUS on a secondary cell in FR2, i.e. this may require further discussions in RAN1.

Therefore it is proposed:

LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

# Summary

RAN2 is kindly asked to discuss LP-WUS with secondary DRX:

Proposal 1	LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

# References

- RP-242361, Way forward on LPWUS for FR2, vivo, DISC, RAN#105

- RP-241824, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN#105

# Appendix: RAN1 discussion about CA with dual DRX

R1-2503055	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Conclusion

For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19.

- Alt1

  - Basic capability

    - LP-WUS can be configured only on PCell

    - LP-WUS indication is applicable to all serving cells of all DRX groups.

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - Advanced capability

    - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group

    - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

      - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

    - This capability has prerequisite of the above basic capability

- Alt2

  - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group

  - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored

    - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation

  - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}