3GPP TSG-RAN WG2 #130	R2-2504307

# Malta, MT, 19th – 23rd May 2025

Agenda Item:	 8.1.2.3

Source:	Ericsson

Title:	Remaining issue for LCM For Positioning

Document for:	Discussion

# 1	Introduction

In this paper, we discuss the remaining aspects for LCM procedure for AI/ML positioning case 1.

# 2 Training Data Collection for Case 1

### 2.1 Using Legacy Procedure for AD request for training

As per RAN1 agreements (Only keeping option for Case 1):

Agreement For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The training data collection for Case 1, which only requires PRS configuration, is similar to existing procedure where UE requests assistance data of PRS configuration for legacy positioning measurements. Thus legacy procedure/ways for UE to request PRS configuration including UE-initiated assistance data request and UE-initiated on-demand PRS procedure can be reused. The training data collection for Case 1 is part of procedure for AI/ML positioning, and the PRS configuration request for this should not be either prioritised or deprioritized compared with legacy positioning, thus UE does not include the purpose of request when sending request for PRS configuration.

For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Legacy procedure is reused for data collection for Case 1 without any enhancements.

### 2.2 Generating Label for data collection and associated parameters

RAN1 Agreements Working Assumption For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of the label and its related data is out of RAN1 scope.

AI/ML is used in area where there can be NLOS. Further, the PRS symbols may not be transmitted with perfect sync and there may be Real Time Difference. The LMF should be able to guide what is the expected positioning error during training phase to the UE based upon related Integrity parameters (e.g., standard deviation of RTD) so that the UE can understand if the obtained positioning can be used as a label (ground truth) or should be discarded. Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for AI/ML training may provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# Conclusion

In the previous sections we made the following observations:

Observation 1	For data collection for AI/ML models on the UE side, the UE may request assistance data to LMF using UE-initiated assistance data request procedure as specified in clause 8.12.3.1.2.2 of TS 38.305 or UE-initiated on-demand PRS procedure as specified in clause 7.6.2 of TS 38.305 without the need to include the purpose of request.

Observation 2	Similar to legacy expectedRSTD where LMF provides the expected RSTD value so that the UE can filter any outliers, the LMF for the case of AI/ML training can provide maximum uncertainty/error so that the UE can judge if the obtained positioning can be considered as ground truth or should be discarded.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Legacy procedure is reused for data collection for Case 1 without any enhancements.

Proposal 2	LMF provides the guidance for trustworthy label based upon Positioning Integrity (maximum allowed Positioning error or uncertainty for a trustworthy label)

# References

TS 37.355, LPP