3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504320

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.17

Source:	Qualcomm Incorporated

Title:	Discussion on new NB-IoT NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#130 meeting, following agreements were made.

Agreements:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

In this document, we provide details on RAN2 specific aspects that need to be addressed.

- Discussion

In the NB-IoT NTN TDD mode, a new DL frame/UL frame periodicity of 9 SFN is used (i.e., current H-SFN duration of 1024 SFN is not multiple 9 SFN). RAN1 has agreed that the downlink subframes within D=8 are fixed to: subframes [3 4 5 6 7 8 9 0] (across two consecutive radio frames), i.e., 82ms out of 90ms is not used for downlink.

RAN2 assume the cell with new TDD frame structure is called NB-IoT NTN TDD cell.

With the new NB-IoT TDD cell, the length of frame will be 90ms, i.e., 9 SFN. The frame structure will result in extremely low duty cycle operation, i.e., there is 8 valid subframes out 9 SFNs would be only valid DL SFN or valid UL SFN. Because of such constraints, we think RAN2 should further discuss following aspects.

- SI scheduling and acquisition

- Increased delay in Paging

- Measurements

SI scheduling and acquisition

Even though the new NB-IoT TDD cell is not accessible to legacy UEs, it is still possible that the legacy UEs may find the PSS/SSS and read MIB. In this case, the legacy UEs will end up wasting power for searching and reading SIB1 just to figure out the cell is not supported as there will be just one opportunity to read SIB1 every 180ms. It may be worth to discuss if there is any possibility for legacy UEs not to require reading SIB1 from the new TDD mode cell. For example, new SIB1 scheduling information for NB-IoT cell in TDD mode. The legacy UEs may bar the cell based on the event that SIB1 is not being able to acquire from the NB-IoT TBB mode cell.

Discuss whether to introduce new SIB1 scheduling information in MIB-NB in the new TDD mode cell.

In RAN1#120, following conclusion was made.

- It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

For NB-IoT, the start of SI window is determined as follows:

- the SI-window starts at the subframe #0 in the radio frame for which (H-SFN * 1024 + SFN) mod T = FLOOR(x/10) + Offset,

  - where T is the si-Periodicity of the concerned SI message and, Offset is the offset of the start of the SI-Window (si-RadioFrameOffset);

  - Where x = (n – 1)*w, where w is the si-WindowLength and the number n corresponds to the order of entry in the concatenated list of SI messages configured by schedulingInfoList, schedulingInfoListExt (if present) in SystemInformationBlockType1-NB in NB-IoT.

- The minimum value of si-WindowLength for NB-IoT is 160ms.

- The minimum si-Periodicity is 640ms, i.e., 64 SFNs.

Due to low duty cycle operation, the scheduled SI window may fall on the invalid DL SFNs and repetitions for the SI message should be postponed to the next valid DL subframes. However, it is possible that the postponed repetitions may cross over to another SI window or SI periodicity boundary or SI modification boundary.

Figure 1 An example of SI message scheduling in new TDD mode cell

As shown in figure 1, if the SI window length is not long enough, it may not include enough number of valid DL SFNs to schedule all the repetitions of a SI message within the SI window, for example at SFN 72. If the SI window length is configured long, it will impact the length of SI periodicity and SI modification boundary

We think the SI window length should be configured such that it includes a required number of DL subframes or SFNs for the repetitions. There is no need to change the scheduling parameters to align with 90ms periodicity, for example, the SI window in multiple of 90ms. In case, a SI window is not sufficiently long, we may need solutions such as the UE can be allowed to extend the SI window to include the sufficient number of valid DL subframes. The extended part of SI window crossing over the SI periodicity or SI modification boundary should be discarded.

The SI message repetitions falling on the invalid DL subframes are postponed to the next valid DL subframes within a SI window.

The SI scheduling information is not changed and discuss how to make sure the SI window length includes enough valid DL subframes required for the SI message repetitions.

Paging

Following is agreed in RAN2#129bis meeting.

- Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

Similarly, paging repetitions may completely or partially overlap with invalid DL subframes. The paging repetitions falling on invalid DL subframes can be postponed to the next valid DL subframes. However, this could lead to overlapping of the postponed repetitions with the beginning of the paging message with the next Paging Occasion (PO) and this is not desirable.

Therefore, the postponed paging repetitions falling in the next paging occasion should be dropped. It means in the beginning of a PO, a new repetition should start.

Expectation is that the network will configure value of NB (gap between two POs) such that the repetitions will not cross over the next PO. For example, if NB = T/64, then there are 64 subframes gap between two POs. From the starting subframe of a PO, if X = (number of postponed subframes + non-DL NB-IoT subframes + npdcch-NumRepetitionPaging-r13 – 64), then the X subframes will overlap with the next PO. The network should configure the paging parameters such that X <= 0. If the X > 0, then the X number of last repetitions should be ignored/dropped.

The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

Network configures the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13.

Repetitions may be larger than 8 DL subframes. In this case, multiple valid DL SFNs would be needed for a given UE. Due to SI message scheduling or resource unavailability in such as low duty cycle scenario, network may postpone the paging message delivery to the next PO, which is at least 1.28s later in NB-IoT as minimum paging DRX cycle is 1.28s. We think this can be discuss if the paging is postponed to the next valid DL subframe and that valid DL subframe has no PDSCH resource available (e.g., SI message may also be postponed to the same next valid DL subframe), and how to minimize the paging delivery delay. In this frame structure, the next valid DL subframe would be subframe 3. We think the network should prioritize paging message over other unicast messages.

When the start of the paging is postponed to the next valid DL SFN, clarify which subframe the UE should monitor the paging.

Measurements

During the invalid DL period (i.e., last 82ms out of 90ms), it may need to be clarified that UE is not required to perform any IDLE mode tasks such as, Paging monitoring, Serving cell measurement, Logged measurements, Neighbor cell measurements. UE’s regular serving cell measurements may need to be postponed to the next valid DL subframes. We may need to check with RAN4 for the potential serving cell or neighbor cell measurement relaxation.

First, for neighbor cell measurements, UE may need to know neighbor’s timing information such that it knows when the neighbor’s invalid DL SFN duration is. A NB-IoT NTN cell operating in TDD mode may need to be deprioritized as there may be NB-IoT cells operating in normal FDD mode. Therefore, we think first a new list of NB-IoT neighbor cells operating in TDD mode should be introduced. Then we should consider additional assistance information such as neighbor’s SFN offset for the neighbor cell measurements or cell reselection.

Introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

Discuss how to provide the UEs with timing information of neighbor cell that is operating in NB-IoT TDD mode for measurements.

- Conclusion

Following proposals are made:

Proposal 1	RAN2 assume the cell with new TDD frame structure is called NB-IoT NTN TDD cell.

Proposal 2	Discuss whether to introduce new SIB1 scheduling information in MIB-NB in the new TDD mode cell.

Proposal 3	The SI message repetitions falling on the invalid DL subframes are postponed to the next valid DL subframes within a SI window.

Proposal 4	The SI scheduling information is not changed and discuss how to make sure the SI window length includes enough valid DL subframes required for the SI message repetitions.

Proposal 5	The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

Proposal 6	Network configures the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13.

Proposal 7	When the start of the paging is postponed to the next valid DL SFN, clarify which subframe the UE should monitor the paging.

Proposal 8	Introduce a new list of neighbor cells operating in TDD mode for measurements and cell reselection.

Proposal 9	Discuss how to provide the UEs with timing information of neighbor cell that is operating in NB-IoT TDD mode for measurements.