3GPP TSG-RAN WG2 Meeting #130	R2-2504327

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.11.2

Source:	InterDigital, Inc.

Title:	Discussion on RACH aspect in SBFD

Document for:	Discussion

# 1. Introduction

In RAN2#127bis meeting, following agreement was made [1]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In RAN2#129bis meeting, following agreement was made [2]:

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

In RAN1#119 meeting, following agreement was made [3]:

Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

In this contribution, we discuss switch from SBFD-ROs to legacy-ROs considering the transmission power of PRACH, and also, how to indicate whether it’s an SBFD-aware UE during random access procedure. More details on related discussions are found in our companion contribution [5].

# 2. Discussion

Switching to legacy-ROs with transmission power

In RAN2#127bis meeting [1], RAN2 agreed that a SBFD-aware UE is allowed to switch from SBFD-ROs (e.g., additional-ROs) to legacy-ROs after certain number of re-attempts (i.e., if the UE firstly selected SBFD-ROs). Also, RAN2 agreed [2] that the SBFD-aware UE to switch form legacy-ROs to SBFD-ROs after the number of re-attempts.

According to the legacy RACH procedure, upon detection of initial transmission failure (e.g., no reception of RA response (RAR)), a SBFD-aware UE can set increased (ramped-up) transmission power with ramping step then will perform re-attempt with the increased power. According to the power ramping step with re-attempts, the SBFD-aware UE may reach the maximum transmission power (Pcmax) of PRACH while not yet reaching the configured max number of re-attempts. Then, there is a remaining issue on whether the UE has to keep re-attempting using SBFD-ROs or do early fallback/switching to legacy-ROs (due to consistent failures even with the capped max Tx power Pcmax).

In our view, it is not desiable to keep performing re-attempts once reached the maximum power of PRACH using SBFD-ROs. The reason of continuous re-attempts being occurred may be because of PRACH reception failures at the network due to a congestion of using SBFD-ROs and higher cross-link-interference (CLI) level in symbols of SBFD-ROs, and/or RAR reception failures at the UE if received on SBFD symbols affected by UE-to-UE CLI, and so on. Also, a network cannot estimate when the maxmimum power of PRACH would be reached after number of re-attepts because each of the SBFD-aware UE may set different transmission power based on the measured path-loss value between UE and network.

To mitigate the excessive CLI level of the cell and achieve successful RACH procedures for UEs in the cell, once the power of PRACH transmission has been reached to the maximum transmission power, the SBFD-aware UE should immediately switch from SBFD-ROs to legacy-ROs while in the middle of performing re-attempts (i.e., even not yet expiry of the parameter of max number of re-attempts). In other words, reaching of max power of PRACH can indicate that the SBFD-aware UE already had attempted with the best possible ways utilizing SBFD-ROs, so that there is no reason to make the UE try more on SBFD-ROs, which rather increases undesired interference and overall congestion rates impacting to other UEs in the cell, without any benefits.

Proposal 1. Support to switch from SBFD-ROs to legacy-ROs upon reaching maximum power of PRACH transmission during performing re-attempts with power ramping, regardless of the remaining possible counts of the configured max number of re-attempts.

Separate maximum power parameter of PRACH for SBFD-ROs

In RAN1#119 meeting [3], RAN1 agreed to support separate power control parameter, i.e., the parameter of preamble received target power for SBFD-ROs, separated from that for legacy-ROs. In our view, it would be better and straightforward to support separate maximum Tx power parameter of PRACH for SBFD-ROs (e.g., Pcmax,SBFD), separated from that for legacy-ROs (Pcmax), to properly control CLI level in consideration of other potential victim UEs that need to receive downlink channels on the same SBFD symbols. Based on the separated configuration of maximum power of PRACH transmission, the newtwork has more flexibility to control proper transmission power level of PRACH transmission using SBFD-ROs with SBFD symbols based on various conditions (e.g., CLI level or congestion level).

Proposal 2. Support to separately configure a parameter of the maximum transmission power (Pcmax) of PRACH for SBFD-ROs using SBFD symbols, from that for legacy-ROs.

PRACH with SBFD-ROs for LTM

In RAN2#129bis meeting [2], RAN2 agreed to support RACH procedure in SBFD symbols, but FFS for LTM case. To support RACH procedure using SBFD symbols, we need to include RO type indication (e.g., SBFD-ROs) associated configuration within the legacy LTM candiate cell configuration (e.g., earlyUL-syncconfig). Moreover, RAN2 needs to specify LTM procedure using SBFD symbols and non-SBFD symbols, respectively. In our view, this is an enhacement of LTM based mobility using SBFD symbols which is out of scope Rel-19 SBFD WI.

Proposal 3. Do not support PRACH transmission using SBFD symbols with LTM procedure in Rel-19.

Early indication via MSG3

In RAN1#116 meeting, the following agreement was made [4], and it was left for FFS on how gNB identifies whether a UE is SBFD aware UE or non-SBFD aware UE:

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

This agreement can be beneficial for SBFD-aware UE in RRC-IDLE/INACTIVE states, where the UE can decide on whether to connect to the selected cell based on SBFD operation or connect to the selected cell based on non-SBFD operation. For example, in case the measured CLI (based on received SSB) is low, the UE could connect to the cell based on SBFD operation. Otherwise, if the measured CLI (based on received SSB) is high, the UE could connect to the cell based on non-SBFD operation.

In another example, the UE can transmit random access preamble in ROs that coincide with SBFD symbols (e.g., via additional-ROs). As such, upon reception of the preamble, the gNB will recognize that the UE is SBFD-aware, and that the UE is operating based on SBFD operation. So, gNB can further transmit RAR in SBFD symbols.

The SBFD-aware UE that has decided to perform RA based on legacy-ROs based on conditions of ROs selection, could indicate its support for SBFD operation later and as part of Msg3.

Proposal 4. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via MSG3.

# 3. Conclusion

Proposal 1. Support to switch from SBFD-ROs to legacy-ROs upon reaching maximum power of PRACH transmission during performing re-attempts with power ramping, regardless of the remaining possible counts of the configured max number of re-attempts.

Proposal 2. Support to separately configure a parameter of the maximum transmission power (Pcmax) of PRACH for SBFD-ROs using SBFD symbols, from that for legacy-ROs.

Proposal 3. Do not support RA procedure using SBFD symbols with LTM procedure in Rel-19.

Proposal 4. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via MSG3.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #127bis.

[2] Chairman’s notes, 3GPP TSG RAN WG2 #129bis.

[3] Chairman’s notes, 3GPP TSG RAN WG1 #119.

[4] Chairman’s notes, 3GPP TSG RAN WG1 #116.

[5] R1-2503624, Discussion on SBFD random access operation, InterDigital, Inc.