3GPP TSG-RAN WG2 Meeting #R2-130		        R2-2504331     St Julians, Malta, May 19-23, 2025

Agenda Item:	8.6.3

Source: 	Rakuten

Title:  	Remaining issues of L1 event triggered measurement reporting

Document for:	Discussion, Decision

# Introduction

In the WID Error! Reference source not found., the following objective is defined for inter-CU LTM:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID objective includes Event based L1 measurements for LTM.

In this contribution, we discuss the feasibility of configuring Event based L1 measurements and using them for LTM candidate cell addition/release. Further, we discuss redundancy of L3 and L1 measurements for LTM.

# Discussion

## Open issue from RAN2#126bis

## Scheduling of L1 measurement report MAC CE

The below agreement was made in RAN2 #126bis.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

While it was agreed that legacy SR procedure for resource allocation is the baseline, requesting resources using BSR to send event-triggered L1 measurements MAC CE was not agreed. We believe there is a need to include some information of MR MAC CE size, otherwise if MR MAC CE size is variable and large, there is no way for the network to assign a grant to guarantee max size.

Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

## Open issue from RAN2#129bis

## Measurement of activated semi-persistent resources

Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options. • Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch • Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch • Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch Will be revisited next meeting.

It was agreed to revisit the issue of how to handle the already activated semi-persistent resources after LTM cell switch. We think that option 1 is the simplest and easiest for implementation.

UE stops to measure the activated semi-persistent CSI-RS resources after LTM cell switch.

## Measurement of activated semi-persistent resources

Proposal 6a: Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells. Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation. Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

We support configuring both mTRP and event triggered measurement reporting simultaneously. And when they are configured simultaneously, the network shall indicate which TRP is to be considered for the LTM event-triggered measurements evaluation. This shall be indicated to the UE beforehand and should be possible to be changed dynamically.

When LTM event-triggered reporting and mTRP operation coexist in the serving cell, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation. This shall be able to be modified subsequently.

## Support for MIMO 2 TA

Following agreements were made in R2#126 regarding Measurement enhancements for LTM.

RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# Conclusion

We have the following observations:

Error! Reference source not found.	Error! Reference source not found.

Error! Reference source not found.	Error! Reference source not found.

We have the following proposals:

Proposal 1	Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

Proposal 2	UE stops to measure the activated semi-persistent CSI-RS resources after LTM cell switch.

Proposal 3	When LTM event-triggered reporting and mTRP operation coexist in the serving cell, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation. This shall be able to be modified subsequently.

Proposal 4	RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

Proposal 5	RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes