3GPP TSG-RAN2 Meeting #130	R2-2504335

# St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		4.1

Source:	Nordic Semiconductor ASA

Title:					Clarification to NTN SI message accumulation over SI-windows

Document for:		Discussion and Decision

# Introduction

In R17, upon introduction of SIB31/32 and in R18 SIB33, it was agreed and clarified that normal change of system information procedure does not apply to SIB31(-NB) and SIB33(-NB). Quoting TS36.331:

5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by

<skip>

Furthermore, it is clarified that:

5.2.3a	Acquisition of an SI message by BL UE or UE in CE or a NB-IoT UE

When acquiring an SI message, the BL UE or UE in CE or NB-IoT UE shall:

<skip>

1>	if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB and SystemInformationBlockType1-NB. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

1>	else:

2>	receive and accumulate SI message transmissions on DL-SCH on narrowband provided by si Narrowband, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength-BR, only in radio frames as provided in si-RepetitionPattern and subframes as provided in fdd-DownlinkOrTddSubframeBitmapBR in bandwidthReducedAccessRelatedInfo, or until successful decoding of the accumulated SI message transmissions;

- if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message;

There are known practical challenges with the SI accumulation across SI-windows e.g., in NGSO systems (especially LEO), where the transmitted SIB31(-NB) message potentially must change from one SI-window to the next due to rapidly changing serving satellite ephemeris information. For example, assume a typical SI-window si-Periodicity-r13 of rf512 (i.e. 5120ms) and a LEO satellite orbit at ~400km altitude, the satellite would travel ~7700m/s and therefore ~40km from one SI-window to another. At the same time the modification period for SIs can be over 20s (for example, assuming defaultModificationPeriodCoeff-r13 of 16 and defaultPagingCycle-r13 of rf128). While signaled Kepler/SV can ensure precise prediction for 30s to the future, different UEs fetch SIB31 at different times. And there could be situations where UE acquired 30s old Kepler parameters and would use it for another 30 seconds. Therefore, for the accuracy reasons (at least the low orbit) the satellites should signal a new location every SI window instead of letting the UE to predict the satellite location and speed far in future.

From our point of view the current specification text should clarify that an NTN capable UE cannot assume SI accumulation would work across SI-windows for SI messages containing SIB31(-NB) or SIB33(-NB). Specifically, due to fact that the UE has no indication from the network side of a potential change in the SI message content for SIB31(-NB) and SIB33(-NB).

# SI accumulation

If a NB-IOT UE is not able to decode a SI message transmission by the end of the SI-window, the UE may continue reception and accumulation of SI messages transmission on DL-SCH in the next SI-window occasion for the concerned SI message.

Also, we do understand that it is feasible from eNB point of view to transmit SIB31 message to be the same across one or more H-SFNs, however, there are known practical challenges with this feature e.g., in NGSO systems (especially LEO), where the transmitted SIB31(-NB) message can potentially be different in each SI-window due to rapidly changing serving satellite ephemeris information. Furthermore, SIB31(-NB) does not follow the normal SI Change Notification procedures, that could be understood as the eNB does not need to follow the normal modification period to change the SI content either. However, this is not stated in the current specifications. As an outcome, the UE cannot rely on SI being unmodified except across repetitions within one SI-window a. Therefore, the UE cannot reliably accumulate from a current SI-window to a next SI-window without potentially worsening the SI acquisition performance.

Although, the case of SIB33(-NB) is not elaborated in this paper, the observations for the need of SI accumulation clarifications also apply to SIB33(-NB).

Observation 3: The SI accumulation feature should not be used when acquiring SIB31(-NB) and SIB33(-NB).

Proposal 2: Clarify in the specifications that the SI accumulation shall not be used when acquiring SIB31(-NB) and SIB33(-NB).

We propose a clarification into TS36.331 concerning the SI accumulation across SI-windows. The proposal is detailed in Section 4 of this paper.

# Conclusions

Related to SIB31(-NB) and SIB33(-NB) acquisition we also made the following observation and the proposal:

Observation 3: The SI accumulation feature should not be used when acquiring SIB31(-NB) and SIB33(-NB).

Proposal 2: Clarify in the specifications that the SI accumulation shall not be used when acquiring SIB31(-NB) and SIB33(-NB).

# Text proposal

3GPP TS36.331 clarification proposal:

#### 5.2.1.2a	Scheduling for NB-IoT

<skip>

Within the SI-window, the corresponding SI message can be transmitted a number of times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS. The UE acquires the detailed time/frequency domain scheduling information and other information, e.g. used transport format for the SI messages from schedulingInfoList field in SystemInformationBlockType1-NB. The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition. For NTN SI messages for which modification period is not applicable the UE is not expected to accumulate SI messages across multiple SI-windows.

SystemInformationBlockType1-NB configures the SI-window length and the transmission periodicity for all SI messages.