3GPP TSG RAN WG2 #130	R2-2504348

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	Fraunhofer HHI, Fraunhofer IIS

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

In the last RAN#106 plenary, a new work item on solutions for Ambient-IoT (Internet of Things) in NR was approved [1]. Ambient-IoT is a new 3GPP IoT technology that relies on ultra-low complexity devices with ultra-low power consumption for very-low-end IoT applications. In the last RAN2#129bis [2] the following agreements were made in regards the A-IoT paging agenda-item:

RAN2#129bis

Agreements for paging: - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to tak this into account for their design.

In alignment with the agreements made in the last RAN2 meeting, our contribution further explores these functionalities and discuss potential areas for improvement.

# Discussion on multi-reader scenario

A key challenge in A-IoT lies in managing situations where a single device receives multiple paging messages from different readers for the same service request. As devices may be in a low-power state, there is an inherent risk of ambiguous behavior when multiple readers simultaneously initiate a procedure for the same service request. Two main behavioral options are currently under consideration:

- Option A: The device ignores all new requests and R2D messages not associated with the ongoing procedure.

- Option B: The device terminates the ongoing procedure and responds to the latest request.

## Views on option A: ignore new requests

Option A minimizes state transitions in devices by maintaining session continuity. Once a device receives and acknowledges a paging message, it commits to the ongoing session and ignores further messages from other readers that are not associated with that session. This approach offers simplicity and reduces the frequency of procedure resets.

On the other hand, this behavior may lead to deadlock scenarios. For instance, if the session with the initiating reader fails due the reader being turned off, the device may remain in a waiting state and unable to respond to new valid requests. Without an effective session termination mechanism, the device could become unresponsive, waiting for a timeout or requiring external intervention. Furthermore, Option A needs the use of transaction identifiers (IDs) to associate incoming messages with the current session. This imposes memory and computational overhead, even though the latter one is not exclusive to Option A.

Observation 1: Session persistence under Option A may result in deadlocks if the procedure fails silently and there is no proactive session termination mechanism.

Proposal 1: Introduce a session termination mechanism, such as a maximum inactivity threshold based on time or counter.

## Views on option B: respond to the latest

Option B enables a more adaptive device behavior by allowing it to respond to the most recent valid request, even if it interrupts an ongoing session. This approach enhances resilience in dynamic environments, as devices are not bound to potentially non-responsive readers. Besides, it supports a quick recovery by re-aligning the device with an active reader.

Despite its flexibility, Option B presents also challenges. Suddenly leaving an active session can result in lost data, and inefficient use of radio resources. Furthermore, readers that initiated the session may not be informed about such transition, leading to inconsistent behavior across the network.

Proposal 2: Devices implementing Option B should notify the previous reader of the session termination using a lightweight signal (e.g., a short NACK or release message).

Additionally, an initial timer can be introduced to temporarily delay the start of the response to a paging message. During this waiting interval, the device may collect multiple incoming requests from different readers. This buffering period enables the device to evaluate and compare multiple candidates before selecting the most appropriate one to respond to. For instance, the selection criteria may include signal quality (e.g., RSSI, SNR), or reader priority.

Proposal 3: An initial timer can be introduced to temporarily delay the response to new paging messages.

## End of procedure

Determining the end of a session is critical for enabling devices to quickly return to a low-power state, and avoiding unnecessary latency during an ongoing procedure. In this context, the absence of a standardized or clearly defined end-of-procedure indication may lead to inefficiencies. Devices might wait unnecessarily for further messages or acknowledgments, while readers may keep resources allocated longer than needed.

To address such issue, in our perspective, an explicit end-of-procedure indication should be standardized. This indication can serve as a key mechanism to minimize ambiguity, and prevent unnecessary delays. One proposed implementation involves including an additional flag that explicitly denotes session completion within an ACK message. This allows reuse of established signaling formats with minimal overhead. Alternatively, a new lightweight control signal can be defined.

Proposal 4: Introduce end-of-procedure signal that marks the termination of a session.

In addition to ending the main interaction, the end-of-procedure signal can also serve a dual purpose in signaling slot termination. This can be particularly relevant when contention-based access is used, where the reader monitors each slot for potential transmissions. If no device is transmitting, the same end-of-procedure indicator can be broadcast to announce slot closure.

Proposal 5: The end-of procedure signal can also be used for slot termination if no device is detected.

# Conclusion

In this contribution we discussed the A-IoT paging mechanism. Based on the discussion in the previous sections we made the following observations and proposals:

Observation 1: Session persistence under Option A may result in deadlocks if the procedure fails silently and there is no proactive session termination mechanism.

-----

Proposal 1: Introduce a lightweight session termination, such as a maximum inactivity threshold based on time or counter.

Proposal 2: Devices implementing Option B should notify the previous reader of the session termination using a lightweight signal (e.g., a short NACK or release message).

Proposal 3: An initial timer can be introduced to temporarily delay the response to new paging messages.

Proposal 4: Introduce end-of-procedure signal that marks the termination of a session.

Proposal 5: The end-of procedure can also be used for slot termination if no device is detected.

# References

- 3GPP RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG-RAN #106, Madrid, Spain, Dec 9th-12th, 2024.

- 3GPP “Draft Report of 3GPP TSG RAN WG2 meeting #129bis”, 3GPP TSG RAN WG2 #129bis, Wuhan, China, April 7th-11th, 2025