3GPP TSG-RAN WG2 Meeting #130	R2-2504352

Malta, Malta, 19th – 23rd May, 2025

Agenda item:	8.8.3

Source:	Samsung

Title:	On RAN2 aspects of uplink OCC

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#106 the latest NR-part of the WID NR NTN Phase in Rel-19 was agreed [1] where the relevant parts to this contribution are:

2. Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide a discussion on RAN2 aspects of uplink capacity/cell throughout enhancement for FR1-NTN.

# Discussion

## Configuring uplink OCC

From the perspective of RAN2, in order for the UE to perform an uplink transmission with OCC, the UE needs to be configured with an OCC length, which gives how many UEs that can be multiplexed on a resource, as well as the specific sequence within the OCC. The OCC length either needs to be RRC-configured or pre-configured. The OCC sequence could be indicated dynamically or also be RRC-configured:

Agreement

OCC length and OCC sequence for OCC with CG PUSCH Type 1 are configured by RRC higher-layers.

- Up to RAN2 whether to signal this with one or two RRC parameters.

- Note: OCC lengths and sequences to be provided in the table of RRC parameters to be prepared by the WI rapporteur.

Agreement

For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options:

- Option 1: Configured by RRC higher-layer parameter.

  - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration

- Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number.

- Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2.

  - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration

- Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values).

- Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2.

Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2.

Agreement

For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI

- FFS: with a new field or reusing an existing field

As can be seen above, there are a number of agreements that RAN2 may need to take into account when implementing the RRC parameters, however we believe it would be a better use of time to discuss this when the RRC parameter list is received from RAN1.

Proposal 1: RAN2 can discuss the RRC parameters based on the received parameter list from RAN1.

## RAN2-related impacts

As is made clear from the WID and the agreements, the enhancement targets PUSCH. One aspect where whether OCC is applicable that has yet to be discussed is whether OCC can apply to PUSCH transmissions during RACH-less handovers. We think that allowing for OCC to be applied during PUSCH transmission for RACH-less can be beneficial, especially in a non-terrestrial network where system-wide there is expected to be a lot of handovers being performed, thus it is important to ensure that the handovers are performed efficiently. In fact this in our view is one of the key argument for introducing RACH-less handovers for NTN Release 18.

Observation 1: Applying OCC for RACH-less handovers can make the handovers more efficient.

Proposal 2: RAN2 to confirm OCC is applicable for RACH-less handovers.

# Conclusion

In this contribution we discussed issues related to uplink capacity/cell throughput enhancements.

Proposal 1: RAN2 can discuss the RRC parameters based on the received parameter list from RAN1.

Proposal 2: RAN2 confirm OCC is applicable for RACH-less handovers.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#106, Madrid, Spain, December 2024.