3GPP TSG-RAN WG2 Meeting #130		     R2-2504353

Malta, MT: 19th May – 23rd May 2025

Source:	Qualcomm Incorporated

Title:	Open Issues on LCM for UE-sided Models for Beam Management and CSI Prediction

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, RAN #102, a new work item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses open issues on “LCM of UE-sided models for beam management use case,” considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Details on the Training Data Collection Configuration

In the RAN2#129 meeting [4], RAN2 agreed to extend the agreements on beam management as a general framework for data collection configuration

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In RAN2#129bis meeting [5], RAN2 agreed that the UE can indicate preferred configurations for data collection from the list of candidate configurations provided by the network.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In RAN1#121bis meeting [6], RAN1 agreed that for UE-side models, the configuration of resources for data collection purposes is provided using CSI-ReportConfig.

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

The following are further highlighted by the rapporteur as open issues on training data collection for UE side model training:

- How to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW?

- Where/what the NW provides as candidate configurations?

- What should be the content of otherConfig for enabling UE data collection requests in UAI (e.g., just a flag, the list of candidate UE data collection configurations, etc.)?

This section discusses signaling for UE-side data collection configuration (see Fig. 2.1.1-1) to address open issues on the training data collection for UE-side training.

Fig. 2.1-1: Signaling for UE side data collection configuration.

Proposal 1: RAN2 is requested to consider Fig. 2.1-1 as signaling for UE side data collection configuration.

Proposal 2: In STEP 2, the UE provides the UE capability information containing its capabilities for UE-side data collection.

Proposal 3: In STEP 3, the otherConfig (in the RRC Reconfiguration) may contain the following

- A Flag indicating whether UE is allowed to send UAI for requesting data collection configuration, and/or

- A List of candidate configurations for data collection for UE-side training (e.g., one or more CSI-ReportConfigs for UE-side data collection).

Proposal 4: In STEP 4, the UE Assistance information contains

- An indication that UE is interested in training data collection, if otherConfig in STEP 3 does not contain the list of candidate configurations for UE-side data collection, or

- Preferred data collection configuration (e.g., identifiers of data collection configuration signaled in STEP 3).

Proposal 5: [OPTIONAL] In STEP 5, the otherConfig (in the RRC Reconfiguration) contains

- A List of candidate configurations for data collection for UE-side training (e.g., one or more CSI-ReportConfigs for UE-side data collection), if not provided in STEP 3.

Proposal 6: [OPTIONAL] In STEP 6, the UE indicates one or more selected configurations for UE-side data collection (e.g., CSI-ReportConfig IDs).

Proposal 7: After the data collection configuration for UE-side model training, the activation/deactivation of the data collection can be performed by

- The network based on network decisions (network wants to STOP transmitting RS, or if the network determines that configured resources/resource sets are not suitable), or

- UE request (if the UE wants to STOP collecting the training data or if the UE determines that configured resources/resource sets are not suitable).

## 2.2 Applicability Reporting for Option B

In the RAN2#129bis meeting [5], the following agreements were made for option B

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference-related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - To take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Based on the agreements in RAN2#129bis meeting [5], RAN2 should wait for RAN1 LS on RRC parameter list for option B to resolve other open issues.

Proposal 8: RAN2 should wait RRC parameter list from RAN1 before further discussing signaling or procedure for option B.

## 2.3 Indicating the cause of inapplicability

In RAN2#129bis meeting [5], RAN2 agreed on the following

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Based on the discussion in RAN2#129bis meeting [5], the UE may not have a model available for inference configuration provided to the UE due to

- The trained model (for the inference configuration) is not available at the UE, i.e., the UE-side has trained a model for the inference configuration, but it is not available at the UE/ device, or

- UE does not have a trained model (for the inference configuration), i.e., UE-side has not trained a model for the inference configuration.

Observation 1: The UE may not have a model available for inference configuration provided to the UE due to

- The trained model (for the inference configuration) is not available at the UE, i.e., the UE-side has trained a model for the inference configuration, but it is not available at the UE/ device, or

- UE does not have a trained model (for the inference configuration), i.e., UE-side has not trained a model for the inference configuration.

Proposal 9: The UE indicates to the network the cause of inapplicability as either:

- Trained model not available: For the case when the UE side has a trained model for the inference configuration, but the model is not available at the UE/device.

- No trained model: For the case when the UE side does not have a trained model for the inference configuration.

## 2.4 Handling of non-applicable periodic CSI-ReportConfig containing inference configuration

- In RAN2#129 meeting [4], RAN2 agreed on following for periodic CSI-ReportConfigs

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- In RAN2#129bis meeting [5], RAN2 further agreed on the following

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- As the full inference configuration is kept at the UE, the periodic CSI-ReportConfigs may become applicable (if the UE downloads the model for periodic CSI report configuration after earlier signaling as non-applicable, or other UE-side internal reasons). In such a scenario, the UE behavior for non-applicable periodic CSI-ReportConfig is unclear, i.e., what is the UE expected to do if periodic CSI-ReportConfig is not applicable when sending RRCReconfigurationComplete / UAI? Due to limited memory at the UE, downloading the required model may be required in most cases. In such cases, if UE is not allowed to report the applicability of period CSI-ReportConfig later using UE Assistance Information, then configuring UE with CSI-ReportConfig may not be useful. Therefore, UE should be allowed to activate periodic CSI-ReportConfig and send its applicability in the UE Assistance Information, when a periodic CSI-ReportConfig becomes applicable after sending RRCReconfigurationComplete containing applicability information.

- Observation 2:  UE behavior for non-applicable periodic CSI-ReportConfig is not clear, i.e., what is the UE's expected behavior if periodic CSI-ReportConfig becomes applicable after earlier signaled as inapplicable in RRCReconfigurationComplete/UAI?

Proposal 10: The UE autonomously activates applicable periodic CSI-ReportConfig (full inference configuration) upon sending the UE Assistance Information (when a periodic CSI-ReportConfig for inference becomes applicable after earlier signaled as inapplicable in RRCReconfigurationComplete / UAI).

## 2.5 Clarification on applicability report

- In the current implementation of the applicability reporting [7]

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

- Observation 3: In the current implementation of the applicability reporting [7]

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

While full applicability/inapplicability information can be provided for the inference configuration provided in the latest RRCReconfiguration, there is no need to provide the full applicability/inapplicability information provided by previous RRCReconfiguration(s).

Proposal 11: The following applicability/inapplicability information is provided in the RRCReconfigurationComplete message

- Full applicability/inapplicability information for the inference configuration provided by the latest RRCReconfiguration message, and

- Change in applicability/inapplicability information for the inference configuration provided by the previous RRCReconfiguration message.

## 2.6 Content of otherConfig for enabling reports in UAI

The legacy CSI-ReportConfig can be differentiated from the CSI-ReportConfig for AI/ML-based beam management based on Information Elements (IEs) in the CSI-ReportConfig [7]. Therefore, UAI configuration setting up UE to report UAI for change in applicability/inapplicability information should be sufficient.

Observation 4: The legacy CSI-ReportConfig can be differentiated from the CSI-ReportConfig for AI/ML-based beam management based on Information Elements (IEs) in the CSI-ReportConfig.

Proposal 12: OtherConfig for enabling report in the UAI is a setup message (i.e., a FLAG), configuring the UE to report changes in applicability/inapplicability information.

## 2.7 On the inclusion of serving cell information in the applicability report

During the normative work, specification work is not limited to PCell, but rather CA and NR-DC scenarios are also in the scope. Therefore, both CA and NR-DC should be considered for applicability reporting.

Proposal 13: CA and NR-DC are considered for the specification work.

Note that the CSI-ReportConfig includes the serving cell index. The gNB-CU assigns a unique CSI-ReportConfigID to each cell or beam it wants to monitor for CSI reports. Therefore, as the UE reports the CSI-ReportConfigID in the applicability report, additionally reporting the serving cell index is not required.

Observation 5: CSI-ReportConfig includes the serving cell index. The gNB-CU assigns a unique CSI-ReportConfigID to each cell or beam it wants to monitor for CSI reports.

Proposal 14: As the UE reports the CSI-ReportConfigID in the applicability report, additional reporting of the serving cell index is not required.

For the NR-DC, if the applicability report is sent over SRB1, the UE will send the applicability report for the SCG using UL Information Transfer MRDC (similar to the legacy measurement reporting for SCG). Thereafter, the SN can determine the cells based on the reported CSI-ReportConfigID in the applicability report for SCG.

Proposal 15: For the NR-DC, if the applicability report is sent over SRB1, the UE will send the applicability report for the SCG using UL Information Transfer MRDC (similar to the legacy measurement reporting for SCG).

Proposal 16: The SN can determine the cells based on the reported CSI-ReportConfigID in the applicability report for SCG.

# Discussion on LCM of UE-sided Models for CSI Prediction

## 3.1	Details on the Training Data Collection Configuration

In the RAN2#129 meeting [4], RAN2 agreed to extend the agreements on beam management as a general framework for data collection configuration. In RAN2#129bis meeting [5], RAN2 agreed that the UE can indicate preferred configurations for data collection from the list of candidate configurations provided by the network. However, it was agreed in the agenda item for LCM for the UE side model for beam management. Based on the agreement in RAN2#129 [4], the agreements in RAN2#129bis meeting should be extended to CSI prediction.

Proposal 17: The UE can request measurement configuration for data collection of AI/ML based CSI Prediction (similar to the beam management use case). The request can contain one or more of the following:

- An indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Proposal 18: The UAI message is used by the UE to request data collection measurement configuration for CSI prediction (similar to the beam management use case). It is up to UE implementation when to send the request.

## 3.2	RRC-based Monitoring Report for CSI Prediction

In RAN1#121 [8], RAN1 agreed on the on the following alternatives for performance monitoring

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer FFS on how to quantize and report mechanism

The objective of performance monitoring is to identify data distribution drift. Even for a single data distribution, since there are good samples and bad samples, the prediction accuracy could have a wide variation. Given that fact, it will be hard to make any monitoring decision unless prediction accuracy is observed over enough number of samples to get a reliable statistic.

In light of above, Alt1 of SGCS reporting requires multiple monitoring report instances to facilitate NW to make the final decision based on SGCS statistics. Alt3 is a more reasonable option where the SGCS statistics or average of multiple monitoring occasions is calculated at UE side and report the metric in one monitoring report. The monitoring window can be defined by number of inference report in the past. Alt2 is much more complicated than the other two without clear benefit.

Observation 6: Objective of performance monitoring is to identify data drift, it will be hard to make any monitoring decision unless prediction accuracy is observed over enough number of samples to get a reliable statistic. Monitoring latency could be tens or even hundreds of milliseconds.

As discussed in our RAN1 paper [9], UE assisted monitoring requires the UE to buffer the predicted CSI which increases the memory cost dramatically. This increases the storage requirement at the UE significantly to compute SGCS statistics or average of multiple monitoring occasions at the physical layer.

Figure 3.2-1 Buffering of inference results for UE assisted monitoring.

To address the buffering concern, one potential solution is to consider RRC based mechanism for UE assisted monitoring. The reason is that L3-signalling does not have stringent timeline requirement so that UE would be able to offload the inference result to a lower cost memory and perform monitoring using low-end implementations.

Observation 7: The buffering burden can be relaxed if L3 signalling or mechanism is used for performance monitoring.

Proposal 19: Support RRC based mechanism for UE assisted monitoring report for CSI Prediction.

# Conclusion

LCM for Beam Management

Proposal 1: RAN2 is requested to consider Fig. 2.1-1 as signaling for UE side data collection configuration.

Proposal 2: In STEP 2, the UE provides the UE capability information containing its capabilities for UE-side data collection.

Proposal 3: In STEP 3, the otherConfig (in the RRC Reconfiguration) may contain the following

- A Flag indicating whether UE is allowed to send UAI for requesting data collection configuration, and/or

- A List of candidate configurations for data collection for UE-side training (e.g., one or more CSI-ReportConfigs for UE-side data collection).

Proposal 4: In STEP 4, the UE Assistance information may contain

- An indication that UE is interested in training data collection, if otherConfig in STEP 3 does not contain the list of candidate configurations for UE-side data collection, or

- Preferred data collection configuration (e.g., identifiers of data collection configuration signaled in STEP 3).

Proposal 5: [OPTIONAL] In STEP 5, the otherConfig (in the RRC Reconfiguration) may contain

- A List of candidate configurations for data collection for UE-side training (e.g., one or more CSI-ReportConfigs for UE-side data collection), if not provided in STEP 3.

Proposal 6: [OPTIONAL] In STEP 6, the UE indicates one or more selected configurations for UE-side data collection (e.g., CSI-ReportConfig IDs).

Proposal 7: After the data collection configuration for UE-side model training, the activation/deactivation of the data collection can be performed by

- The network based on network decisions (network wants to STOP transmitting RS, or if the network determines that configured resources/resource sets are not suitable), or

- UE request (if the UE wants to STOP collecting the training data or if the UE determines that configured resources/resource sets are not suitable).

Proposal 8: RAN2 should wait RRC parameter list from RAN1 before further discussing signaling or procedure for option B.

Observation 1: The UE may not have a model available for inference configuration provided to the UE due to

- The trained model (for the inference configuration) is not available at the UE, i.e., the UE-side has trained a model for the inference configuration, but it is not available at the UE/ device, or

- UE does not have a trained model (for the inference configuration), i.e., UE-side has not trained a model for the inference configuration.

Proposal 9: The UE indicates to the network the cause of inapplicability as either:

- Trained model not available: For the case when the UE side has a trained model for the inference configuration, but the model is not available at the UE/device.

- No trained model: For the case when the UE side does not have a trained model for the inference configuration.

- Observation 2:  UE behavior for non-applicable periodic CSI-ReportConfig is not clear, i.e., what is the UE's expected behavior if periodic CSI-ReportConfig becomes applicable after earlier signaled as inapplicable in RRCReconfigurationComplete/UAI?

Proposal 10: The UE autonomously activates applicable periodic CSI-ReportConfig (full inference configuration) upon sending the UE Assistance Information (when a periodic CSI-ReportConfig for inference becomes applicable after earlier signaled as inapplicable in RRCReconfigurationComplete / UAI).

- Observation 3: In the current implementation of the applicability reporting [7]

- The UE is expected to send full applicability/inapplicability information of the inference configuration configured by the previous RRCReconfiguration message (note that the UE has already provided the complete applicability/inapplicability information when configured by previous RRCReconfiguration(s) messages).

Proposal 11: The following applicability/inapplicability information is provided in the RRCReconfigurationComplete message

- Full applicability/inapplicability information for the inference configuration provided by the latest RRCReconfiguration message, and

- Change in applicability/inapplicability information for the inference configuration provided by the previous RRCReconfiguration message.

Observation 4: The legacy CSI-ReportConfig can be differentiated from the CSI-ReportConfig for AI/ML-based beam management based on Information Elements (IEs) in the CSI-ReportConfig.

Proposal 12: OtherConfig for enabling report in the UAI is a setup message (i.e., a FLAG), configuring the UE to report changes in applicability/inapplicability information.

Proposal 13: CA and NR-DC are considered for the specification work.

Observation 5: CSI-ReportConfig includes the serving cell index. The gNB-CU assigns a unique CSI-ReportConfigID to each cell or beam it wants to monitor for CSI reports.

Proposal 14: Additional reporting of the serving cell index is not required, as the UE reports the CSI-ReportConfigID in the applicability report.

Proposal 15: For the NR-DC, if the applicability report is sent over SRB1, the UE will send the applicability report for the SCG using UL Information Transfer MRDC (similar to the legacy measurement reporting for SCG).

Proposal 16: The SN can determine the cells based on the reported CSI-ReportConfigID in the applicability report for SCG.

LCM for CSI Prediciton

Proposal 17: The UE can request measurement configuration for data collection of AI/ML based CSI Prediction (similar to the beam management use case). The request can contain one or more of the following:

- An indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Proposal 18: The UAI message is used by the UE to request data collection measurement configuration for CSI prediction (similar to the beam management use case). It is up to UE implementation when to send the request.

Observation 6: Objective of performance monitoring is to identify data drift, it will be hard to make any monitoring decision unless prediction accuracy is observed over enough number of samples to get a reliable statistic. Monitoring latency could be tens or even hundreds of milliseconds.

Observation 7: The buffering burden can be relaxed if L3 signalling or mechanism is used for performance monitoring.

Proposal 19: Support RRC based mechanism for UE assisted monitoring report for CSI Prediction.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- RAN2#129 meeting report

- RAN2#129bis meeting report

- RAN1#121bis meeting report

- R1-2503478, Introduction of AI/ML beam prediction, endorsed TS 38.214 CR

- RAN1#121 meeting report

- R1-2502831, “Specification support for CSI prediction,” RAN1#120bis