3GPP TSG-RAN-WG2 Meeting #130	R2- 2504357

St Julian’s, Malta, May. 19th – 23rd , 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Discussion on DL coverage enhancement

Document for	:	Discussion and Decision

Introduction

In the RAN2 #129bis meeting, RAN2 made some agreements/conclusions as follows [1].

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In addition, at the post129bis email discussion, it was proposed the followings by rapporteur.

[Proposals for easy agreement] Proposal 1 If a new SMTC periodicity is finally adopted and included in system information broadcast, RAN2 introduces the support in a backwards compatible manner. FFS signalling details (e.g., new field or extension of SMTC4). Proposal 2 RAN2 includes the mapping between a MBS broadcast session (TMGI) and ISA ID(s) extending MBS-SessionInfoList to avoid signaling overhead (duplication of TMGIs). Proposal 3 For the service continuity discussion in MBS NR NTN, RAN2 takes the following scenarios as a baseline: - Multiple frequencies, served by the same satellite, illuminating the same geographical area. Each frequency provides a different MBS service. These services are not being broadcast in the serving cell. - Multiple frequencies, served by the same or different satellites, but not covering the same geographical area (they are neighbour cells). Each frequency provides a different MBS service. These services are not being broadcast in the serving cell. [Proposals for discussion] Proposal 4 Companies are encouraged to address the following topics in their contributions: - Maximum number of SMTCs that need to be supported (e.g., 6). - Association of SMTC with location/beam information (e.g. serving cell SSB index, reference location). - Nature of SIBXX defining the intended service area: cell or area specific.

In this paper, we further discuss remaining issues for SMTC configuration.

Discussion

It is FFS about signalling details of SMTC configuration and details of assistance information on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode.

SMTC configuration consists of periodicity, offset, and duration. For new SMTC configuration, just additional periodicities are needed, and it may be enough to reuse SMTC4 for the base of new configuration, but new SMTC configuration (SMTC5) have a flexibility of NW configuration. NW can configure the same or different parameter values in each list, and it is also possible to omit the parameter values in new configuration if same parameter value is configured for both configurations. So, we think new SMTC configuration (i.e. SMTC5) should be introduced for NTN DL CE.

Proposal.1: New SMTC configuration (i.e. SMTC5) is introduced for NTN DL CE.

If the cells are in an ideal honeycomb structure, 6 SMTC patterns are needed to measure the surrounding cells. Based on the current agreements, it requires that the number of SMTCs to be measured at one time should not exceed 4.

Therefore, introducing assistant information is considered to select the SMTC configuration. For example, serving cell SSB Index and Reference Location are considered to associate the SMTC configurations. However, the area of each SSB Index is determined based on the satellite beams, which can be not only circular-shape but also elliptical-shape, and it may not be properly configured.

In the current spec, location-based measurement reporting / conditional reconfiguration mechanism is specified, and the location of the serving cell and the target cell can be recognized by UE using SIB19. So, if the relation info between neighbour cell (physical cell id and frequency) and SMTC pattern is informed to the UE, and the UE can determine the SMTC patterns to use. Therefore, we think the location-based SMTC selection is appropriate. In addition, all necessary information can be notified by SIB, this can be adopted not only to connected mode UEs but also to idle/inactive mode UEs.

Proposal.2: Maximum number of SMTC configuration is 6.

Proposal.3: The location-based SMTC selection is introduced to help UE to perform SMTC selection.

Conclusion

In this paper, we made following proposals;

Proposal.1: New SMTC configuration (i.e. SMTC5) is introduced for NTN DL CE.

Proposal.2: Maximum number of SMTC configuration is 6.

Proposal.3: The location-based SMTC selection is introduced to help UE to perform SMTC selection.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #129bis

- [Post129bis][305][R19 NR NTN] RRC CR (Ericsson)