3GPP TSG-RAN WG2 Meeting #130                                    R2-2504361

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.3.5

Source:				Samsung

Title:					Discussion on Performance Monitoring for AI/ML Mobility

Document for:		Discussion & Decision

1	Introduction

In this contribution, we would like to share our view on performance monitoring for AI/ML mobility.

2	Discussion

2.1	Model/Functionality Management

2.1.1	NW-side Model

Regarding the model/functionality management for NW-side model, the following agreement was made in Rel-19 AI/ML for air WI.

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

Since we do not see any issue specific to AI/ML for mobility for the management procedure of NW-side model, we would like to propose the following.

Proposal 1: For AI/ML for mobility, for NW-side model, the model/functionality management decision can be up to NW implementation and transparent to UE.

2.1.2	UE-side Model

For UE-side model case, based on the discussions in the AI/ML for air WI so far, both RAN1 and RAN2 are focusing solely on functionality management for UE-side model case, assuming that model-level management is handled by the UE itself and is transparent to the NW. Specifically, for a particular AI-enabled functionality, the UE can simply report that it is applicable if there is at least one available model to use for that functionality. Consequently, the network can manage the functionality at this level without needing to know which AI model the UE employs for the functionality. The same principle can be adopted also in AI/ML for mobility.

Proposal 2: For AI/ML for mobility, for UE-side model, the model management decision can be up to UE implementation and transparent to NW.

Regarding the functionality management for UE-side model, the following agreement was made in Rel-19 AI/ML for air WI.

Agreement (in RAN2)

- For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

Agreement (in RAN1) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In the RAN1 agreement on the BM use case, Type 1 refers to the method where NW makes the functionality management decision based on the monitoring results from UE or NW itself, while Type 2 refers to a method where UE makes the decision by itself based on the internal monitoring results. Given the remaining time, it is likely that only Type 1 will be supported at least for BM use case in Rel-19 AI/ML for air WI. On the other hand, for positioning use case, RAN2 is still discussing the UE autonomous fallback as it offers the advantage of reducing management delay compared to NW-side decision.

Based on this, for AI/ML for mobility, we think NW-decision can serve as the baseline, but we should remain open to discussing the UE-side decision.

Proposal 3: For AI/ML for mobility, for UE-side model, the functionality management decision can be made by NW as a baseline. FFS. Whether to support UE-side decision.

2.2	Performance Monitoring

2.2.1	NW-side Model

Regarding the performance monitoring for NW-side model, RAN2 made the following agreement for BM use case in Rel-19 AI/ML for air WI.

Agreements for NW-sided model for Beam Management

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

In our view, also in AI/ML for mobility, the gNB has the responsibility on monitoring its NW-side model performance. For the monitoring, gNB should have both predicted and the actual measurement results. For the predicted measurement results, gNB can obtain it as outcome of its NW-side model inference. However, the actual measurement results should be provided by UE and we think the existing measurement/report configuration can be reused for that. For example, for frequency domain prediction, gNB can predict the measurement results for the cell in the target frequency based on the serving frequency measurement results from UE. When the gNB need to perform the monitoring (i.e., need to obtain the actual measurement in the target frequency), it can configure UE to perform the measurement in that target frequency and to report the results with the legacy measurement configuration with measurement gap.

Proposal 4: For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement.

2.2.2	UE-side Model

Regarding the performance monitoring for UE-side model, RAN1 made the following agreement for BM use case in Rel-19 AI/ML for air WI.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

For the UE-side model, the performance monitoring (i.e., calculation of performance metric) can be done by either UE (Option 2) or NW (Option 1) and the agreement above shows that RAN1 is focusing on the method to support UE-side monitoring. In our view, in case of UE-side model, both predicted and actual result are available at UE-side. Thus, it seems reasonable for UE to perform the performance monitoring and report it to NW for the management decision by NW. In that sense. we would like to propose that UE-side performance monitoring can be the baseline.

Proposal 5: For AI/ML for mobility, for UE-side model, UE can perform the performance monitoring (e.g., performance metric calculation) and report the monitoring results to NW as baseline.

For the performance monitoring metric, “L3 RSRP difference” can be used for temporal and frequency domain prediction since we already evaluate the prediction accuracy with the metric. For measurement event prediction, the prediction accuracy is evaluated based on “F1 score” and thus we can use it as performance monitoring metric. One more point to think about is whether “L3 RSRP difference” can be also used as a metric for measurement event prediction, considering the relation that the lower “L3 RSRP difference” will lead to higher “F1 score” in the indirect method.

Proposal 6: For temporal and frequency domain prediction, “L3 RSRP difference” can be used for performance monitoring metric.

Proposal 7: For measurement event prediction, “F1 score” can be used for performance monitoring metric.
FFS. “L3 RSRP difference” can be also used for the indirect prediction.

If L3 RSRP difference is used for the performance monitoring metric, the definition of “L3 RSRP difference” should be clear without any ambiguity. In temporal domain prediction case A, UE can predict an RSRP value multiple times for one real value at the same time instance as the observation/prediction windows move forward (i.e., as long as the time instance of the real value belongs to prediction window). Also, it is obvious that if the prediction performance tends to get better as the observation window gets close to the time instance of the real RSRP value. Similar to case A, in temporal domain prediction case B, if multiple RSRP values need be predicted with the same input sequence (e.g., when MRRT = 2/3, 4/5, …), the prediction accuracy will depend on the temporal gap between the measurement timing of the input and that of the real RSRP value (For measurement reduction actual RSRP values are not measured normally, but here it is assumed that actual RSRP values are acquired as commanded by NW for monitoring purpose). Therefore, in calculating L3 RSRP difference, the temporal gap between the measurement time of the input used for RSRP prediction and that of real RSRP value must be carefully considered.

Proposal 8: For temporal domain prediction cases A and B, the definition of L3 RSRP difference should take into account the temporal gap between the measurement time of the input used for RSRP prediction and that of the real RSRP value.

In frequency domain prediction, even though NW configures monitoring-related procedures to UE for performance monitoring purposes, UE may not be able to calculate L3 RSRP difference because UE does not have an ability to measure signals on multiple frequency bands simultaneously. How to define L3 RSRP difference in the above-mentioned scenario should be studied.

Proposal 9: For frequency domain prediction, RAN2 can study how to calculate L3 RSRP difference when UE is not able to measure signals on multiple frequency bands simultaneously.

Regarding the report of monitoring results by UE, we can consider the following two options.

- Option 1 (periodical reporting)
- UE periodically reports the calculated performance metric (e.g., L3 RSRP difference) to NW
- NW can make the management decision based on the calculated performance metric from UE.

- Option 2 (event-triggered reporting)
- UE reports the monitoring event (e.g., prediction accuracy becomes lower than some threshold) to NW once it is detected.
- NW can make the management decision based on the monitoring event report from UE.

In terms of signaling overhead, the option 2 has a benefit compared to the option 1, since with the option 2, the UE will trigger the monitoring report only when any event to report is detected. On the other hand, the option 1 can give more flexibility to NW from a management perspective while simplifying UE operations. Thus, RAN2 can study both options for the monitoring report.

Proposal 10: For monitoring report from UE to NW in UE-side performance monitoring, RAN2 can study the following two options.

- Option 1 (periodical reporting)
- UE periodically reports the calculated performance metric (e.g., L3 RSRP difference) to NW
- NW can make the management decision based on the calculated performance metric from UE.

- Option 2 (event-triggered reporting)
- UE reports the monitoring event (e.g., prediction accuracy becomes lower than some threshold) to NW once it is detected.
- NW can make the management decision based on the monitoring event report from UE.

Another issue to address is how the UE can acquire the actual measurement results for performance monitoring in use cases involving measurement reduction (e.g., temporal domain Case B, frequency domain prediction). In such scenarios, the UE does not carry out actual measurements at certain times or frequencies to minimize its measurement overhead. Consequently, when the UE conducts inference, it cannot access the actual measurement results needed to compute the performance metric. Therefore, there must be a designated time duration (e.g., a monitoring window) during which the UE performs both actual measurements and inference for prediction concurrently.

For the specific spec. impact, we would like to propose the following based on the discussion above.

Proposal 11: For UE-side performance monitoring for UE-side model, RAN2 can study the following spec. impact.

- Monitoring results

  - Option. 1: Performance monitoring metric (e.g., RSRP difference)

  - Option. 2: Event report (e.g., monitoring failure event)

- Enhanced or new measurement/report configuration from NW to UE

  - Indication to request UE to report the monitoring result

  - (Temporal Case B, frequency domain) Monitoring window

  - Monitoring report configuration 
(For Option. 1) monitoring report periodicity
(For Option. 2) monitoring event configuration

- Enhanced or new report from UE to NW

  - Report of monitoring results

3	Conclusion

Proposal 1: For AI/ML for mobility, for NW-side model, the model/functionality management decision can be up to NW implementation and transparent to UE.

Proposal 2: For AI/ML for mobility, for UE-side model, the model management decision can be up to UE implementation and transparent to NW.

Proposal 3: For AI/ML for mobility, for UE-side model, the functionality management decision can be made by NW as a baseline. FFS. Whether to support UE-side decision.

Proposal 4: For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement.

Proposal 5: For AI/ML for mobility, for UE-side model, UE can perform the performance monitoring (e.g., performance metric calculation) and report the monitoring results to NW as baseline.

Proposal 6: For temporal and frequency domain prediction, “L3 RSRP difference” can be used for performance monitoring metric.

Proposal 7: For measurement event prediction, “F1 score” can be used for performance monitoring metric.
FFS. “L3 RSRP difference” can be also used for the indirect prediction.

Proposal 8: For temporal domain prediction cases A and B, the definition of L3 RSRP difference should take into account the temporal gap between the measurement time of the input used for RSRP prediction and that of the real RSRP value.

Proposal 9: For frequency domain prediction, RAN2 can study how to calculate L3 RSRP difference when UE is not able to measure signals on multiple frequency bands simultaneously.

Proposal 10: For monitoring report from UE to NW in UE-side performance monitoring, RAN2 can study the following two options.

Proposal 11: For UE-side performance monitoring for UE-side model, RAN2 can study the following spec. impact.

4	Reference