3GPP TSG-RAN WG2 Meeting #130	R2-2504365

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.3

Source:	InterDigital, Ericsson, Nokia, Sony, Vodafone, KT, Turkcell

Title:	Discussion on neighboring cell measurement with LR

Document for:	Discussion

# 1. Introduction

In RAN2#126 meeting, the following agreement was made:

Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

In RAN4#112bis, RAN4 made the following agreements

Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON.

# 2. Discussion

2.1 The issue of neighboring cell measurement with MR

In RAN2#126 meeting, RAN2 agreed to perform neighboring cell measurement by MR, but RAN2 have not considered to allow the LR to perform Intra-frequency measurements. This decision has the following Limitations:

- This limitation is not power efficient because intra-frequency cell reselection by MR is triggered frequently due to irregular cell coverage and different beam qualities of the cell. Due to these issues, MR relaxation cannot be done and the potential UE power saving is limited.

- This agreement is not clear when the offloading condition (e.g., case #1) is sastified, neighboring cell measurement can be allowed by LR (i.e., in case of serving cell measurement via LR is available). There is not a clear technical issue preventing a UE from performing intra-frequency neighboring cell measurement while performing serving cell measurement via (OFMD-based) LR.

- If EMR (early measurement reporting, i.e., measidleconfig) is configured, the MR cannot perform relaxed measurement according to section 5.2.4.9 of 38.304, hence the power saving gain is limited. According to [6], RAN4 concluded that EMR carriers should not be relaxed in Rel-16 power saving WI. To achieve further power saving gain in Rel-19, RAN2 can consider to use EMR procedure with relaxed measurement once Intra-frequency based measurement are performed by (OFMD-based) LR.

Based on above reasons, RAN2 should discuss whether these limitations are reasonable.

2.2 The necessity of neighboring cell measurement with LR

2.2.1. MR RRM relaxation with cell coverage

Figure 1. Urban environments radio condition [1]

According to [1], in urban environments the radio conditions may change rapidly and dramatically, e.g. when turning around a corner. This implies that LP-WUS usage may be limited to a small part of the cell. In case of multi-beam operation the cell coverage may also be beam dependent, i.e. beams of the same cell provide different quality. This may be another reason why intra-frequency cell reselection is triggered, and the principle is the UE should be camping on the best quality cell. As a result, the UE is configured and required to perform intra-frequency measurements in most of the cell and these intra-frequency measurements cannot be relaxed.

For this reason it is beneficial to discuss neighboring cell measurements performed by OFDM-based LR, i.e. the LR also performs intra-frequency measurements and ranking and the MR is only woken up when cell reselection is initiated.

Observation 1: In many practical deployments MR neighboring cell measurement relaxation cannot be done limiting the potential power saving even if LP-WUS is configured and used.

However, the conditions to relax neighboring cell measurements should be independent of the actual LP-WUS coverage and entry/exit thresholds for LP-WUS (see e.g. [1]), depending on the fulfilment of low-mobility and/or not-at-cell-edge conditions. For example, if the UE is under LP-WUS coverage but the UE is moving fast and towards cell-edge, then the neighboring cell measurements should not be relaxed.

Based on the issues above, RAN2 can discuss the need and feasibility to support neighboring cell measurement using OFDM-based LR. We can send an LS to ask RAN4 to investigate the feasibility of the neighboring cell measurement using OFDM-based LR.

Proposal 1: RAN2 to discuss intra-frequency neighboring cell measurements using OFDM-based LR.

Proposal 2: Ask RAN4 whether it is feasible to support neighboring cell measurements using OFDM-based LR.

2.2.2. MR neighboring cell relexation with EMR configuration

According to current IDLE specification [5], MR RRM relaxation of neighboring cell measurement is not allowed when frequencies are configured with idle/inactive (EMR) configuration.

#### 5.2.4.9 Relaxed measurement ##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8]; - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8]; … NOTE 1: It is up to UE implementation when to start performing relaxed measurements in RRC Idle/Inactive if multiple methods are configured. NOTE 2: It is up to UE implementation which relaxation method to perform based on the "allowed" cases as specified in TS 38.133 [8] for RRC Idle/Inactive if multiple methods are configured. The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

Considering the Case#1 and IDLE specification, when the serving cell measurements are fully offloaded by LR, the UE needs to activate MR to perform neighboring cell measurement if the UE is configured with EMR configuration (e.g., idle/inactive configuration). As a result, MR cannot be offloaded/relaxed (i.e., case#1 or case#3) and power consumption of the UE would be increased [2]. Regarding the same issue, some companies [3] proposed that a necessity of discussion on neighboring cell measurement relxation with MR when a UE is configured with idle/inactive measurement. Also, another company mentioned [4] to study cell reselection with LR.

Observation 2: According to IDLE spec, RRM relaxation of neighboring cell measurements cannot be allowed with idle/inactive configuration.

Proposal 3: RAN2 to discuss neighboring cell measurements using OFDM-based LR when EMR is configured.

# 3. Conclusion

Observation 1: In urban enviroments, MR RRM relaxation of neighboring cell measurements cannot be operated (e.g., triggered reselection) due to urban radio conditions.

Observation 2: According to IDLE spec, RRM relaxation of neighboring cell measurements cannot be allowed with idle/inactive configuration.

Proposal 1: RAN2 to discuss intra-frequency neighboring cell measurements using OFDM-based LR.

Proposal 2: Ask RAN4 whether it is feasible to support neighboring cell measurements using OFDM-based LR.

Proposal 3: RAN2 to discuss neighboring cell measurements using OFDM-based LR when EMR is configured.

# 4. References

[1] R2-2501094, LP-WUS and RRM measurements, Ericsson, RAN2#129.

[2] R2-2500944, Discussion on RRM measurement relaxation and offloading, InterDigital, RAN2#129.

[3] R2-2500994, RRM measurement relaxation in RRC_IDLE/INACTIVE, Nokia, RAN2#129.

[4] R2-2410378, Discussion on RRM aspects for LP-WUS/WUR, Sony, RAN2#128.

[5] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[6] R4-2009132, LS on RRM relaxation in UE power saving.