3GPP TSG-RAN2 Meeting #130			R2-2504373

St. Julian, Malta, 19th-23rd May

Agenda item:	8.7.4.1

Title:	Further consideration on LCP enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting [1], RAN2 discusses some issues related to LCP and formulates agreements as follows.

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

This contribution mainly discusses some remaining issues on LCP.

Discussion

  - Bj adjustment for priority-adjusted LCH data

It is observed that the token bucket UE variable Bj, which regulates the PBR conformance for logical channels, may occasionally become negative even when delay-critical data still resides in the transmission buffer. This phenomenon presents a challenge for ensuring both timely data delivery and long-term fairness across logical channels.

The PBR is configured based on GFBR in QoS profile, which is supposed to be alignment with the server ice request, hence, Bj adjustment should always follow the restriction of PBR. In scenarios where the PBR is adequately configured from a long-term throughput perspective, temporary insufficiency of Bj may still occur. This may result from early uplink data transmission, transient traffic bursts, or a short interval after the last Bj increment. In such cases, negative Bj values do not necessarily reflect long-term overuse of resources but rather momentary demand peaks.

Under these circumstances, ignoring negative Bj values for priority-adjusted logical channels could be beneficial, allowing the UE to continue transmission of critical data without undue delay. However, unconditionally overriding Bj constraints risks violating the intended fairness enforced by the PBR mechanism.

Observation 1: The reason for a negative Bj mainly stems from the increment of Bj is too small, usually caused by the time to the last Bj increment is too short or the last LCP allocates too much resource for LCH j.

To balance latency performance with long-term fairness, it is proposed that in case UE has delay-critical data, UE may enlarge the time T for Bj increment. Then the Bj can be recalculated as BLj = (Bj + PBR × TLong), where TLong denotes a longer time window, potentially on the order of several seconds. When Bj becomes negative but delay-critical or priority-adjusted data remains in the buffer, the UE may draw from BLj, thereby enabling transmission beyond the short-term token limitation. Once BLj is applied and activated, the original Bj is temporarily suspended, and no increment to BLj is permitted until the completion of TLong, ensuring that the long-term data rate remains bounded.

This approach maintains fairness over extended time periods while enabling timely servicing of transient priority traffic. Alternatively, the handling of such conditions may be left to UE implementation, provided that fairness and QoS compliance are preserved at the system level.

Observation 2: Fairness issue caused by ignoring negative Bj can be solved by compensating the additional Bj increment later, e.g., using less time T for Bj increment, or a long-term token bucket after UE ignoring negative Bj.

Proposal 1: To avoid the negative Bj impact on delay-critical data when PBR is sufficient, RAN2 may consider to enlarge the time T for Bj increment and compensate the irregular Bj increment later.

  - Impact for SR

Generally, when there exists pending DSR but no pending SR, UE should trigger an SR to request a potential UL grant. And for SR triggered by priority-adjusted LCH data, it’s beneficial to send the SR as fast as possible to avoid potential data discarding.

Therefore, to avoid discarding, the SR resource configuration which has the nearest PUCCH occasion to send SR should be prioritized. As SR resource has different periodicity, an SR configuration with periodicity of 10 slots might be able send DSR earlier than another SR configuration with 80 slots periodicity.

However, SR resources are configured per MAC entity and the LCH only binds with one SR configuration in legacy, therefore it’s not possible for DSR to trigger the SR that binds with another higher priority LCH.

One possible solution is that if priority-adjusted LCH data triggers SR, UE should choose the SR configuration that binds with the LCH whose priority is the same with the priority-adjusted LCH data. However, it’s a bit complicated as UE has to look up in LCHs to find the LCH with proper priority. And the UE behavior is also in doubt if there are multiple or no LCH with the same priority. Besides, chance is that due to SR configuration is kind of shared among LCHs, the LCH whose priority is the same with priority-adjusted LCH data may have the same SR configurations with the original LCH. Thus, the attempt to find a proper SR configuration with higher priority is not granted to be successful.

Therefore, to make sure that pending DSR will trigger an SR configured with short periodicity while maintain simple and easy for UE implantation, it’s suggested to configure an additional SR resource to the LCH that may have delay-critical data so that if DSR is triggered, the additional SR configuration with short periodicity will be utilized and UE doesn’t need to search SR configuration among LCHs.

Another option is that, once pending DSR triggers an SR, the MAC entity can choose the SR configuration with the shortest periodicity and triggers a pending SR. This option has basically the same functionality but may have more impacts on specifications.

The prohibit timer may also have some impact, but it’s too complicated for UE implementation.

Observation 3: Using SR configuration with short periodicity is helpful to send DSR in time and avoid potential data discarding.

Observation 4: Looking up for a proper SR configuration among LCHs is complicated and there is no guarantee that a proper SR configuration exists, e.g., the LCH is already configured with the SR configuration that has the shortest periodicity or the no LCH has SR resource configuration with shorter periodicity.

Proposal 2: NW can configure an additional SR resource configuration with shorter periodicity per LCH for pending DSR.

  - Priority determination of UL grant.

According to current specifications, the priority of an UL grant in intra-UE prioritization is based on the highest LCH priority among those multiplexed in the corresponding MAC PDU. As per agreements in previous meetings, the additional priority configured for an LCH may be used during the LCP procedure. It is also agreed that fallback to the default priority may occur in the 2nd round of LCP.

However, for consistency and to ensure delay-sensitive traffic is prioritized appropriately, also concerns on UE complexity, it is proposed to clarify that the additional priority shall be applied for UL grant prioritization whenever the corresponding LCH carries delay-critical data, regardless of whether fallback occurred during the second round of LCP or whether the transmission is an initial transmission or a retransmission.

Proposal 3: The additional LCH priority is applied in UL grant prioritization if the LCH has delay-critical data, regardless of whether the priority falls back in the 2nd round of LCP or it is an initial transmission or retransmission.

  - PSI impact on LCP enhancement.

When PSI-based discarding is activated, low importance PDU sets are configured with shorter discard timers, making them more likely to be discarded.

Besides, their low importance also indicates that they are less critical compared to other data. Therefore, to ensure fairness and scheduling efficiency under congestion, it is proposed that these low-importance PDU sets shall not be prioritized and also not be reported in DSR.

Proposal 4: In case PSI-based discard is activated, the additional priority is not applied to low importance PDU set in both rounds of LCP for that LCH.

  - PSIHI impact on LCP enhancement.

With regarding to the definition of delay-critical PDU SDU, which define both PDCP SDU and PDCP SDU belonging to a PDU set whose remaining time is less than the same threshold, i.e., remainingTimeThreshold.

Therefore, it is reasonable to extend the application of additional priority to PDU sets containing at least one SDU whose remaining time is below the defined threshold. But if there is no delay-critical SDU within the PDU set, the priority should not be affected. As the delay-critical SDUs in the delay-critical PDU set are already delivered to the lower layers, the remaining SDUs would not trigger any discarding.

Observation 5: Regarding the definition of delay-critical data, PDU set with at least one PDCP SDU whose remaining time is less than the same threshold for PDCP SDU is also considered as delay-critical data.

Proposal 5a: In the 1st round of LCP, PSIHI has no specification impact on LCH priority determination as PDU set and PDCP SDU use the same remaining time threshold.

Proposal 5b: In the 2nd round of LCP, if pdu-SetDiscard is configured, the priority of LCH should fall back to the original priority if there is neither PDCP SDU nor PDCP SDU within a PDU set whose remaining is less than the threshold.

Conclusion

Based on the abovementioned analysis and observations, the following is proposed.

Bj enhancement:

Observation 1: The reason for a negative Bj mainly stems from the increment of Bj is too small, usually caused by the time to the last Bj increment is too short or the last LCP allocates too much resource for LCH j.

Observation 2: Fairness issue caused by ignoring negative Bj can be solved by compensating the additional Bj increment later, e.g., using less time T for Bj increment, or a long-term token bucket after UE ignoring negative Bj.

Proposal 1: To avoid the negative Bj impact on delay-critical data when PBR is sufficient, RAN2 may consider to enlarge the time T for Bj increment and compensate the irregular Bj increment later.

Intra-UE prioritization:

Observation 3: Using SR configuration with short periodicity is helpful to send DSR in time and avoid potential data discarding.

Observation 4: Looking up for a proper SR configuration among LCHs is complicated and there is no guarantee that a proper SR configuration exists, e.g., the LCH is already configured with the SR configuration that has the shortest periodicity or no LCH has SR resource configuration with shorter periodicity.

Proposal 2: NW can configure an additional SR resource configuration with shorter periodicity per LCH for pending DSR.

Proposal 3: The additional LCH priority is applied in UL grant prioritization if the LCH has delay-critical data, regardless of whether the priority falls back in the 2nd round of LCP or it is an initial transmission or retransmission.

PSI and PSIHI impact on LCP:

Proposal 4: In case PSI-based discard is activated, the additional priority is not applied to low importance PDU set in both rounds of LCP for that LCH.

Observation 5: Regarding the definition of delay-critical data, PDU set with at least one PDCP SDU whose remaining time is less than the same threshold for PDCP SDU is also considered as delay-critical data.

Proposal 5a: In the 1st round of LCP, PSIHI has no specification impact on LCH priority determination as PDU set and PDCP SDU use the same remaining time threshold.

Proposal 5b: In the 2nd round of LCP, if pdu-SetDiscard is configured, the priority of LCH should fall back to the original priority if there is neither PDCP SDU nor PDCP SDU within a PDU set whose remaining is less than the threshold.

References

[1]	R2-2502985, Report from session on R19 XR, Session chair (Huawei)