3GPP TSG-RAN2 Meeting #130			R2-2504374

St. Julian, Malta, 19th-23rd May

Agenda item:	8.7.4.2

Title:	Further consideration on DSR enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting [1], RAN2 discusses some issues related to DSR and formulates agreements as follows.

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

This contribution mainly discusses some remaining issues on DSR.

Discussion

  - Enhanced DSR configuration

In the last meeting, a potential concern arises regarding the possibility of triggering a DSR where no delay-critical data exists with remaining time below the configured DSR triggering threshold. This scenario may occur, for instance, if the triggering threshold is configured to a value that precedes all reporting thresholds, raising questions about the necessity and implications of such triggering behavior.

Upon examination, it is observed that the DSR triggering mechanism introduced in Rel-18, mandates that a DSR is only triggered and remains pending when at least one LCG have available delay-critical data whose remaining time is lower than the configured triggering threshold. Consequently, if no such data exists at the time of evaluation, the DSR will not be generated or maintained in a pending state.

However, for enhanced DSR, it is recognized that the network may intentionally configure a triggering threshold that is lower than any reporting threshold. This configuration allows the network to acquire early insight into upcoming delay-critical data, thereby enabling more proactive resource allocation decisions before any data approaches its discard deadline. From this perspective, the triggering threshold serves as a flexible mechanism for advance signaling, rather than being strictly tied to imminent data expiration.

Observation 1: NW may configure a triggering threshold lower than all reporting threshold to avoid any possible discarding and have a clear insight into the potential delay-critical data in the buffer queue.

As a result of these observations, it is concluded that there is no need to impose restrictions on the relative configuration of reporting and triggering thresholds. The behavior remains consistent with the Rel-18 design principles, and the configuration strategy can be left entirely to network implementation based on specific service requirements and performance objectives.

Proposal 1: There is no restriction and no specification impact on the time relationship of configured and reporting thresholds, which is entirely up to NW implementation.

  - DSR data volume calculation

When considering the calculation of DSR data volume, C-PDUs and SDU/Data PDUs to be retransmitted should be paid with additional attention.

C-PDUs, by design, are important but not configured with discard timer. Therefore, they cannot trigger DSR. Similarly, SDUs or Data PDUs in the HARQ buffer, or RLC/PDCP for retransmission also are not necessarily associated with discard timer, therefore fall outside the scope of DSR triggering, as they may not consist of delay-critical data alone.

Proposal 2a: As legacy, C-PDU and SDU/Data PDU to be retransmitted alone will not trigger DSR.

In principle, C-PDUs can be accounted for within the data volume reported under the smallest configured reporting threshold, which is acceptable due to their minimal impact on buffer dynamics. However, ambiguity remains regarding whether SDU and data PDUs to be retransmitted consistently correspond to delay-critical traffic, particularly when such retransmissions may involve both delay-critical and non-delay-critical data.

Proposal 2b: If DSR is triggered, due to its importance, C-PDU can be reported in the smallest reported threshold.

Furthermore, it is noted that gNB implementations typically assign high scheduling priority to retransmission data, for example, in RLC-AM. This prioritization ensures their timely delivery irrespective of DSR-triggered scheduling, thereby diminishing the practical benefit of explicitly including retransmission data in early DSR reporting thresholds.

An additional consideration arises from the potential for misinterpretation at the NW side. Specifically, the inclusion of retransmission SDU/data PDUs in the smallest configured threshold could be erroneously construed as the presence of data with critically low remaining time, leading to unintended scheduling bias and undermining the intended semantics of the reporting threshold hierarchy.

Another concern is that the retransmitting SDU/data PDUs may also contain some data with sufficient remaining time or not assigned with the discard timer.

Observation 2a: Since it's unclear whether the SDUs/data PDUs to be retransmitted always have low remaining time, and they may not be as critical as C-PDUs, putting all of them in the smallest configured reporting threshold could mislead the network about the presence of truly delay-critical data.

Given these factors, it is proposed that C-PDUs and retransmission PDUs be included in the smallest reported threshold only as a form of informational indication, without altering their non-triggering status for DSR initiation. Alternatively, for UE implementation complexity concerns, there should not be too much difference on which threshold is selected.

Observation 2b: In UE implementation, the data volume of C-PDU and retransmitting SDU/data PDUs only needs to added to the smallest reported threshold after UE calculates the data volume for that threshold.

In either case, it should be confirmed that neither C-PDUs nor retransmission SDU/Data PDUs are to trigger DSR, in alignment with their operational characteristics and protocol behavior.

Proposal 2c: The retransmitting SDU/data PDU is also reporting the smallest reported threshold.

Another concern is the impact of PSI, from our perspective, if there are congestion and PSI-based discarding is activated, the intention to report data with low importance is doubtful as they may easily be discarded and NW are unlikely to allocate sufficient resource for low importance data in congestion. Therefore, the gain of reporting low importance data in DSR is very unclear.

Observation 3: In case of UL congestion and PSI-based discarding is activated, the benefit of reporting low importance data is doubtful as such data may be discarded before the next UL grant.

Proposal 3: The data in low importance PDU set is not considered in data volume calculation if PSI-based discarding is activated.

  - Non-delay-critical-reporting Data

The agreement from RAN2#129bis allows the UE to include non-delay-reporting data ahead of delay-reporting data in the data volume calculation for DSR, based on UE capability indication. However, how the UE determines the relative ordering of such data is an implementation issue with potential specification impact. To ensure fairness and avoid inconsistent behaviour across UE from various vendors, it is important that the ordering logic used for DSR reporting reflects the actual internal data queueing and transmission behaviour of the UE.

For example, UEs may use PDCP COUNT as a basis for determining the ordering between SDUs, since it provides a monotonically increasing sequence number. However, other mechanisms (e.g., internal queue position, timestamp, or logical channel configuration) may also be valid, as long as they are applied consistently. Mandating a specific method such as PDCP COUNT may limit implementation flexibility unnecessarily, especially given the diversity of UE architectures.

Therefore, it is proposed that the specification emphasize consistency with actual data transmission behaviour as the principle, while allowing flexibility in the specific method used to determine the ordering.

Proposal 4: Capture “UE should ensure the ordering of data in buffer is consistent with actual data transmission behaviour by its implementation, when UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.” in PDCP running CR.

  - UE behavior when enhanced DSR is larger than UL grant.

Enhanced DSR can be much longer than Rel-18 legacy DSR. Per current specifications, UE should keep pending SR and wait for next potential UL grant that if current UL grant cannot accommodate the entire enhance DSR.

However, current procedure may increase delay and cause data discarding. Therefore, it’s preferred that UE can send a simplified DSR when UL grant cannot accommodate the entire enhance DSR.

Proposal 5a: If UL grant cannot accommodate the intact enhanced Rel-19 DSR, UE may use legacy DSR instead.

Proposal 5b: If UL grant cannot accommodate even the intact legacy DSR, UE may only report delay status on LCG(s) with least remaining time.

Conclusion

Based on the abovementioned analysis and observations, the following is proposed.

Threshold configuration:

Observation 1: NW may configure a triggering threshold lower than all reporting threshold to avoid any possible discarding and have a clear insight into the potential delay-critical data in the buffer queue.

Proposal 1: There is no restriction and no specification impact on the time relationship of configured and reporting thresholds, which is entirely up to NW implementation.

Data volume calculation:

Proposal 2a: As legacy, C-PDU and SDU/Data PDU to be retransmitted alone will not trigger DSR.

Proposal 2b: If DSR is triggered, due to its importance, C-PDU can be reported in the smallest reported threshold.

Observation 2a: Since it's unclear whether the SDUs/data PDUs to be retransmitted always have low remaining time, and they may not be as critical as C-PDUs, putting all of them in the smallest configured reporting threshold could mislead the network about the presence of truly delay-critical data.

Observation 2b: In UE implementation, the data volume of C-PDU and retransmitting SDU/data PDUs only needs to added to the smallest reported threshold after UE calculates the data volume for that threshold.

Proposal 2c: The retransmitting SDU/data PDU is also reporting the smallest reported threshold.

Observation 3: In case of UL congestion and PSI-based discarding is activated, the benefit of reporting low importance data is doubtful as such data may be discarded before the next UL grant.

Proposal 3: The data in low importance PDU set is not considered in data volume calculation if PSI-based discarding is activated.

Non-delay-critical-reporting data:

Proposal 4: Capture “UE should ensure the ordering of data in buffer is consistent with actual data transmission behaviour by its implementation, when UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.” in PDCP running CR.

UE behavior when UL grant cannot accommodate DSR:

Proposal 5a: If UL grant cannot accommodate the intact enhanced Rel-19 DSR, UE may use legacy DSR instead.

Proposal 5b: If UL grant cannot accommodate even the intact legacy DSR, UE may only report delay status on LCG(s) with least remaining time.

References

[1]	R2-2502985, Report from session on R19 XR, Session chair (Huawei)