3GPP TSG-RAN WG2 Meeting #130	R2-2504385

St.Julians, Malta,  May 19th – 23rd , 2025

# Agenda item:	8.4.4

# Source:	CMCC

# Title:	Discussion on LP-WUS operation in CONNECTED mode

# Document for:	Discussion, Decision

- Introduction

In the last RAN2 meeting, RAN2 discussed LP-WUS operations in RRC_CONNECTED mode, following is the related agreements:

RAN2#129bis meeting agreements [1]:

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

In this contribution, we will discuss the potential RAN2 impacts with LP-WUS/WUR in RRC_CONNECTED state.

- Discussion

  - Short DRX cycle with Option 1-2

In last two RAN2 meetings, the coexistence of short DRX cycle and LP-WUS operation was discussed, and RAN2 agreed for Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. While for Option 1-2, only working assumption is reached as following:

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

For option 1-2, it is agreed that LP-WUS monitoring is performed at least outside legacy C-DRX Active Time and a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring [2]. What’s more, the drx-onDurationTimer is not started with Option 1-2 LP-WUS [3]. Based on the agreements, it can be noticed that for Option 1-2, UE’s PDCCH monitoring has no relationship with DRX cycle, regardless it’s Short DRX cycle or Long DRX cycle.

Observation 1: For Option 1-2, UE’s PDCCH monitoring has no relationship with DRX cycle, regardless it’s Short DRX cycle or Long DRX cycle.

Besides, RAN2 made conclusion on UE’s  periodic CSI/L1-RSRP reporting behaviour for Option 1-2 [3]:

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

That’s to say, DRX is mainly related to UE’s periodic CSI/L1-RSRP reporting for Option 1-2.

Observation 2: For Option 1-2, DRX is mainly related to UE’s periodic CSI/L1-RSRP reporting.

Based on the two observations, we prefer RAN2 to confirm the working assumption to progress the LP-WUS discussion.

Proposal 1: RAN2 confirms the working assumption that “for option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used” to progress the LP-WUS discussion.

  - LP-WUS in CA/DC

During the running CR discussion, how to support LP-WUS operation in DC scenario was discussed in RAN2, and following is the related agreements made in RAN2#129bis meeting:

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

Meanwhile, RAN1 also discussed LP-WUS operation in DC scenario, and agreed to only configure LP-WUS on Pcell/PSCell [4] for the case without dual DRX groups:

Agreement - For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

For the case that DC/CA with dual DRX groups, there’s no consensus in RAN1:

Conclusion For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

Considering the limited time of LP-WUS WI, the energy saving gain of LP-WUS with dual DRX group is not clear and there could be potential spec impact, it is proposed to not support LP-WUS with dual DRX groups.

Proposal 2: LP-WUS with dual DRX groups is not supported in Rel-19.

As to the LP-WUS operation cell, though in current spec, cross carrier scheduling is supported and there’s no limitation to schedule PCell by SCell,  the scheduling of control information is on PCell while cross carrier scheduling is mainly used for the scheduling of traffic data. When it comes to LP-WUS, it is a kind of control information. And considering the potential deployment, it’s more possible to deploy LP-WUS in low frequencies like 2.6GHz to ensure the coverage, Therefore, we prefer that LP-WUS monitoring is only configured on the SpCell (PCell or PSCell) per cell group, and LP-WUS indication is applicable to all serving cells in the cell-group. Therefore, we think RAN2 should confirm RAN1’s agreements that  LP-WUS is only configured on PCell/PSCell.

Proposal 3: For NR-DC (without dual DRX group), the LP-WUS can only be configured to be monitored on the PCell and PSCell:

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell;

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group.

  - Option 1-3

Both Option 1-1 and Option 1-2 are used to trigger the MR into active time for PDCCH monitoring,  which means that with only Option 1-1 or Option 1-2, once MR is activated, the UE need to always monitor all PDCCH occasions. While Option 1-3 works inside UE’s active time, with LP-WUS indication, UE can skip some PDCCH occasions monitoring for further power saving. Based on the discussion in SI phase, Option 1-3 main focuses on XR service, which the service is periodic, but there is jitter for each packet, to reduce the effects introduced by jitter, the C-DRX on-duration may be configured to cover the jitters. But it results in UE’s extra power assumption for PDCCU monitoring. With Option 1-3, UE can start PDCCH monitoring when the traffic arrives and achieve energy saving.

Figure 1: Example for Option 1-3

Based on RAN1’s definition/description of Option 1-3, LP-WUS indication is received in UE’s active time. Currently, the active time includes the following cases TS 38.321 [5]:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

It’s not clear whether Option 1-3 LP-WUS indication is applied to the timers mentioned above. In our understanding, since Option 1-3 is to solve the jitter issue of XR service, and C-DRX cycle  and on-duration configuration has taken XR service periodicity and jitter into account, Option 1-3 applied to drx-onDurationTimer is sufficient.

Proposal 4: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Besides, considering that Option 1-3 and Option 1-1/1-2 focuses on different phases of PDCCH monitoring, therefore, If Option 1-3 is supported, it can be configured with Option 1-1 or Option 1-2.

Proposal 5: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Support of LP-WUS use options

Following is the working assumption made in RAN1#118 meeting [6]:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

As to Option 1-3, though it can be used with Option 1-1 or Option 1-2, but the power saving gain is not as large as the other two options.  Considering RAN1’s working assumption, it can be deprioritized.

Proposal 6: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

  - Configuration and (de)activation of LP-WUS monitoring

In RAN1#116 meeting [7], gNB RRC signalling based LP-WUS enable/disable is agreed and whether additional options on top of it is for further study.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Considering the motivation of using LP-WUS in CONNECTED mode is to reduce the PDCCH monitoring power consumption while guarantee the latency performance, it is not necessary to adapt the on/off of LP-WUS monitoring frequently based on fast L1/L2 enable/disable signalling. Besides, UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring. For example, when a UE is in lower battery status, UE can report to gNB that it prefers to activate the LP-WUS monitoring function. As to the condition based activation/deactivation mechanism, similar performance can be achieved by proper network configuration based on UE reported information, including UE assistant information and SR/BSR. Therefore, in our understanding, Option 3 and Option 4 can be achieved by network implementation.

Proposal 7: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 8: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 9: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 10: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

  - Co-existence with other power saving mechanisms

Co-existence with DCP

In last meeting, RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE, while Option 1-2 and Option 1-3 are more flexible to trigger PDCCH monitoring in drx off period or within DRX active time, therefore, there’s no need to configure DCP and LP-WUS for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 11: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

- Conclusions

In this contribution, we discuss the LP-WUS operation in RRC_CONNECTED mode, and following are the proposals and observations:

Proposals:

Proposal 1: RAN2 confirms the working assumption that “for option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used” to progress the LP-WUS discussion.

Proposal 2: LP-WUS with dual DRX groups is not supported in Rel-19.

Proposal 3: For NR-DC (without dual DRX group), the LP-WUS can only be configured to be monitored on the PCell and PSCell:

- For the case when UE is configured with CA without dual DRX groups in RRC CONNECTED mode, LP-WUS can be configured only on PCell

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, LP-WUS can be configured only on PCell/PSCell per cell-group

Proposal 4: UE only monitors the Option 1-3 LP-WUS indication within the legacy drx-onDurationTimer duration.

Proposal 5: If Option 1-3 is supported, it can be configured together with Option 1-1 or Option 1-2.

Proposal 6: RAN2 confirms RAN1’s working assumption that Option 1-3 is deprioritized for RRC_CONNECTED UE.

Proposal 7: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 8: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 9: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 10: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

Proposal 11: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

Observations:

Observation 1: For Option 1-2, UE’s PDCCH monitoring has no relationship with DRX cycle, regardless it’s Short DRX cycle or Long DRX cycle.

Observation 2: For Option 1-2, DRX is mainly related to UE’s periodic CSI/L1-RSRP reporting.

- References

- Report of 3GPP TSG RAN WG2 meeting #129bis;

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- Report of 3GPP TSG RAN WG2 meeting #128;

- Report of 3GPP TSG RAN WG1 meeting #120bis;

- TS 38.321; Medium Access Control (MAC) protocol specification; (Release 18);

- Report of 3GPP TSG RAN WG1 #118

- Report of 3GPP TSG RAN WG1 #116.