3GPP TSG-RAN WG2 Meeting #130	R2-2504386

St.Julians, Malta,  May 19th – 23rd , 2025

# Agenda item:	8.5.2

# Source:	CMCC

# Title:	Discussion on on-demand SSB for SCell

# Document for:	Discussion, Decision

- Introduction

In RAN2#129bis meeting, RAN2 reached the following agreements on on-demand SSB for SCell [1]:

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this paper, we further discuss open issues related to OD-SSB transmission configuration/indication and measurements.

- Discussion

  - OD-SSB activation/deactivation

Based on RAN2 discussion, both explicit and implicit activation/deactivation of OD-SSB can be indicated by MAC CE, while the detail is still FFS.

Meanwhile, OD-SSB activation/deactivation is also discussed in RAN1, and in last RAN1 meeting, following agreements on the configuration of OD-SSB were achieved as following [2]:

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

Observation 1: Based on RAN1 agreements, Number N of on-demand SSB bursts to be transmitted after on-demand SSB can be configured by RRC, in other words, implicit deactivation of OD-SSB can be indicated in RRC configuration.

Observation 2: Both RRC and MAC CE can be used for OD-SSB activation/deactivation.

In our opinion, if Number N of on-demand SSB bursts to be transmitted after on-demand SSB is configured in RRC configuration, there’s no need to use MAC CE for implicit deactivation. And if Number N is not configured in RRC configuration, both explicit or implicit activation/deactivation by MAC CE can be used.

Proposal 1: If Number N of on-demand SSB bursts to be transmitted after on-demand SSB is configured in RRC configuration, only explicit activation/deactivation is indicated to the UE by MAC CE.

Proposal 2: If Number N of on-demand SSB bursts to be transmitted after on-demand SSB is not configured in RRC configuration, both explicit and implicit activation/deactivation can be used to indicate UE by MAC CE.

Besides, there’s two cases for further discussion:

Case a: OD-SSB may be activated by the configuration or indication of  number N, it is not clear, whether OD-SSB can be deactivated during period of the N SSB bursts.

Case b: OD-SSB MAC CE is used for OD-SSB parameter updating after OD-SSB is transmitted.

For Case 2, RAN1 discussed when OD-SSB is activated after SCell activation in RAN1#120bis meeting[2]:

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Proposal 3: For Scenario#3B, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Proposal 4: It can be further discussed  whether OD-SSB can be deactivated during period of the N OD-SSB bursts activated by the configuration or indication of  number N.

  - OD-SSB configuration

In last RAN2 meeting, RAN2 agreed that OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB, in other words, multiple configuration of OD-SSB is supported. As to the parameters that can be configured with multiple values is not clear, and following are the related RAN1 agreements:

Besides, RAN1 also discussed OD-SSB related configuration and reached consensus as following:

RAN1#117 meeting Agreement[3] - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

RAN1#118bis meeting Agreement[4] For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

RAN1#120Agreement [5] - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

RAN1#120bis Agreement [2] For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

Based on RAN1 progress, the following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB

Proposal 5: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB

Proposal 6: Periodicity of the on-demand SSB, SSB positions within an on-demand SSB burst and Number N of on-demand SSB bursts to be transmitted after on-demand SSB can be configured with multiple value.

  - Measurement considering OD-SSB

ServingCellMO configuration

Considering the overlapping of OD-SSB and AO-SSB in time domain and frequency domain, following cases are identified by RAN1:

RAN1#118bis meeting Agreement [4] For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

RAN1#120 meeting Agreements [5] Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

In last RAN2 meeting, RAN2 mainly discussed the case that AO-SSB and OD-SSB are on different frequency, and reached the following working assumption that When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

As agreed in RAN1, AO-SSB and OD-SSB are located in the same BWP if their frequency locations are different, though the bandwidth of BWP can be configured from a few megabits Hz to tens or even hundreds of megabits Hz, and supporting multiple OD-SSBs with the different frequencies for a given SCell may provides more flexibility for network scheduling, it should be noticed that it is already agreed to support multiple values for some parameters, which results in the increasing of signalling overhead for OD-SSB configuration. If multiple OD-SSBs with the different frequencies for a given SCell is supported, the signalling overhead will increase exponentially, which is not expected. What’s more, it may introduce extra workload in RAN4 to decide the requirements on OD-SSB switching.

Observation 3: With multiple OD-SSBs with the different frequencies for a given SCell, not only the network scheduling flexibility, but also the signalling overhead will be increased.

Based on the analysis above, we prefer to not support multiple OD-SSBs with the different frequencies for a given SCell, and based on this, RAN2 confirms the working assumption on servingCellMO.

Proposal 7: Multiple OD-SSBs with the different frequencies for a given SCell is not supported.

Proposal 8: RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Besides, how to configure servingCellMO in case 1 that AO-SSB and OD-SSB have same center frequency location should also be discussed. In our opinion, the same servingCellMO can be used in this case.

Proposal 9: For case 1 that AO-SSB and OD-SSB have the same center frequency location, the same servingCellMO is used.

Measurement of OD-SSB

How to configure OD-SSB is discussed in both RAN2 and RAN4, and RAN2 agreed to leave it to RAN4:

Agreements on OD-SSB 1. RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

In the beginning of OD-SSB discussion, following scenarios are identified by RAN1:

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Since the discussion in the early stage is mainly on Scenario 2 that OD-SSB is activated for a deactivated SCell, therefore, RAN4’s mainly discussed on OD-SSB measurement for deactivated SCell, and following are the related RAN4 agreements for OD-SSB measurement in Case#1 and Case#2 with SCell deactivated[6].

Case#1 Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). Agreement: Case#2 Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

Based RAN4 agreements, no matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB before SCell activation.

Observation 4: No matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB before SCell activation.

In last RAN1 meeting, RAN1 discussed OD-SSB parameter updating for Scenario#3B via MAC CE, therefore, the measurement on OD-SSB after SCell activation should also be discussed. Since the SCell the activated, we think the current SMTC mechanism should be reused. Based on TS 38.331, SMTC is configured in the measurement objective, and SMTC configuration includes SMTC duration, periodicity and offset, while the parameters are related to SSB pattern. When it comes to OD-SSB, no matter it has the same or different frequency location with AO-SSB, the periodicity and offset of the two kinds of SSBs may be different, therefore, multiple SMTCs may be needed.

Proposal 10: Current SMTC based measurement is used for OD-SSB measurement after SCell activation.

Proposal 11: RAN2 needs to discuss whether multiple SMTC configuration is needed for OD-SSB measurement after SCell activation.

- Conclusion

In this contribution, we discuss configuration/indication of OD-SSB transmission and measurement related issues. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: Based on RAN1 agreements, Number N of on-demand SSB bursts to be transmitted after on-demand SSB can be configured by RRC, in other words, implicit deactivation of OD-SSB can be indicated in RRC configuration.

Observation 2: Both RRC and MAC CE can be used for OD-SSB activation/deactivation.

Observation 3: With multiple OD-SSBs with the different frequencies for a given SCell, not only the network scheduling flexibility, but also the signalling overhead will be increased.

Observation 4: No matter in Case#1 or Case#2, the same measurement mechanism is used, that is UE will measure OD-SSB based on SSB pattern within the fast measurement window, and measured based on measCycleSCell as legacy if there’s OD-SSB or AO-SSB before SCell activation.

Proposals:

Proposal 1: If Number N of on-demand SSB bursts to be transmitted after on-demand SSB is configured in RRC configuration, only explicit activation/deactivation is indicated to the UE by MAC CE.

Proposal 2: If Number N of on-demand SSB bursts to be transmitted after on-demand SSB is not configured in RRC configuration, both explicit and implicit activation/deactivation can be used to indicate UE by MAC CE.

Proposal 3: For Scenario#3B, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Proposal 4: It can be further discussed  whether OD-SSB can be deactivated during period of the N OD-SSB bursts activated by the configuration or indication of  number N.

Proposal 5: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Time location of on-demand SSB burst

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB

Proposal 6: Periodicity of the on-demand SSB, SSB positions within an on-demand SSB burst and Number N of on-demand SSB bursts to be transmitted after on-demand SSB can be configured with multiple value.

Proposal 7: Multiple OD-SSBs with the different frequencies for a given SCell is not supported.

Proposal 8: RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Proposal 9: For case 1 that AO-SSB and OD-SSB have same center frequency location, the same servingCellMO is used.

Proposal 10: Current SMTC based measurement is used for OD-SSB measurement after SCell activation.

Proposal 11: RAN2 needs to discuss whether multiple SMTC configuration is needed for OD-SSB measurement after SCell activation.

- References

- Report of 3GPP TSG RAN WG2 meeting #129bis;

- Report of 3GPP TSG RAN WG1 meeting #120bis;

- Report of 3GPP TSG RAN WG1 meeting #117;

- Report of 3GPP TSG RAN WG1 meeting #118bis;

- Report of 3GPP TSG RAN WG1 meeting #120;

- R4-2502598 WF on RRM requirements for Netw_Energy_NR_enh_Part1.