3GPP TSG-RAN WG2 Meeting #130	R2-2504388

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.5.4

Source:	CMCC

Title:	Discussion on Common signalling adaptation

Document for:	Discussion

Introduction

In the RAN#102 meeting, the new WI of enhancements of network energy savings for NR was set up [1], one of the objectives is to adapt the common signal/channel transmission, which has been discussed during R18 SI and proved to be beneficial for energy saving, the related objective is as following,

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

And following are the agreements related to common signalling adaptation in the last two RAN2 meetings[2]:

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In this contribution, we provide our view on paging adaptation related issues.

Discussion

Paging adaptation

One open issue about separate PEI identified in the running CR discussion [3] is whether to introduce a separate pei-ConfigBWP for paging adaptation. In our opinion, it’s necessary to introduce a separate pei-ConfigBWP for R19 separate PEI, in this way, the collision of legacy PEI and R19 separate PEI which may introduce confusion for the UE, can be avoided easily.

Proposal 1: A separate pei-ConfigBWP is introduced for the separate PEI.

Besides, since paging adaptation can also configure with legacy paging parameters, i.e., 4 POs per PF, RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

Proposal 2: RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

PRACH adaptation

For adaptation of PRACH in time-domain, additional PRACH resources are configured, and there could be overlapping between additional RO and legacy valid RO in time domain or frequency domain, and based on RAN1#120 meeting agreements[3], if the additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid. The overlapping of additional RO and legacy RO may result in the RA-RNTI collision issue. We are open to discuss whether to solve RA-RNTI collision issue. From our point of view, there are many solutions to solve this issue, such as:

	Solution 1: It is up to network implementation to solve this.

	Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

	Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 4

For Solution 1, there is no any spec impact, but it cannot solve RA-RNTI collision issue efficiently in some time. As to  Solution 2, it is the most flexible solution among the three solutions, but it will bring the most workload to determine the two offsets and related IEs to indicate them. In terms of Solution 3, it is a kind of compromise of Solution 1 and Solution, in which a new formula is introduced similar to 2-step RACH, and a fixed offset is used avoid RA-RNTI collion for additional ROs and legacy ROs, but no new IE is needed to be  introduced.

Proposal 3: The following solutions can be discussed in RAN2 to solve the RA-RNTI collision issue caused by PRACH adaptation:

Solution 1: It is up to network implementation to solve this.

	Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

	Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 4

Conclusions

In this contribution, we analyse the paging adaptation related issues, and the following proposals and observation are made:

Proposal 1: A separate pei-ConfigBWP is introduced for the separate PEI.

Proposal 2: RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

Proposal 3: The following solutions can be discussed in RAN2 to solve the RA-RNTI collision issue caused by PRACH adaptation:

Solution 1: It is up to network implementation to solve this.

	Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

	Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 4

References

- RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;

- Draft RAN2 129bis meeting report;

- Running RRC CR NES.