3GPP TSG RAN WG2 Meeting #130	R2-2504396

Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#129bis meeting [1], the following agreements have been made:

Agreements FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. Agreements on CFRA Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. Agreements on paging ID length A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. Send an LS to SA2 to take this into account for their design.

Then in this contribution, we would like to provide our views on remaining issues in A-IoT paging.

Discussion

## 2.1 General analysis

### 2.1.1 Multi-reader scenario

In RAN2#129 meeting, the following agreement are made for multi-reader scenario [2]:

RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

Some companies have the concern that it may not be sufficient to mitigate the impact of multi-reader scenario and thinks that reader ID is also needed in A-IoT paging message. From our point of view, there are two cases for the multi-reader scenario as given in Figure 1.

Figure 1. Schematic diagram of the multi-reader scenario.

To be specific, for case 1, where different readers perform inventory procedures simultaneously, the devices located in overlapping areas covered by different readers may simultaneously receive R2D messages from different gNB-readers. Note that there are frequent R2D and D2R message transmissions in an inventory process and the A-IoT system is an asynchronous communication system, the interference of the signals received by the devices located in the overlapping coverage area may be extremely severe. To this end, we do not think they can communicate normally and it seems that introducing reader ID will not bring any benefits for case 1.

For case 2, where different readers perform inventory procedures interleavingly, the interference can be mitigated. Note that the gNB-reader 3 suspends its inventory procedure when gNB-reader 4 performs the inventory procedure as shown in Figure 1. However, the reason why case 2 happens and its benefits are not clear, because compared with the case where gNB-reader 3 completes the inventory procedure and then gNB-reader 4 starts to perform the inventory, it seems that there is no improvement in efficiency, but only an increase in the complexity of the protocol. To this end, we do not think it is necessary to support case 2, and any optimization for case 2 is not need. Therefore, reader ID is not needed as well for case 2.

Based on above analysis, we think that the best method is to avoid the occurrence of multi-reader scenarios or mitigating the impact as much as possible through following implementation methods:

- In R19, only D1T1-B where CW node is out of the Topology 1 is supported. So it is very likely that when one reader transmits R2D messages and receives D2R messages, adjacent readers transmit CW waves in actual deployment scenarios. Therefore, multi-reader scenario may also be naturally avoided as much as possible in practical scenarios.

- For Topology 1, each TRP acts as one reader as discussed in RAN3. To this end, it can be achieved through gNB implementation to control the transmission power of each TRP, thereby avoiding overlapping coverage between adjacent readers. With this, the device(s) is less likely to receive the another (different) service request from different reader.

In terms of reader ID, there is no need to support it for multi-reader scenario in R19.

Observation 1: When different readers perform inventory procedures simultaneously, the interference of the signals received by the devices located in the overlapping coverage area may be extremely severe.

Observation 2: For the case where different readers perform inventory procedures interleavingly, it seems that there is no improvement in efficiency, but only an increase in the complexity of the protocol.

Observation 3: In R19 WID, only the case that multiple paging IDs are included in one A-IoT paging message is listed as a forward compatibility issue. For multi-reader scenario and/or parallel service case, neither of them has been agreed as the direction for forward compatibility yet.

Observation 4: There are several implementation methods to avoid the occurrence of multi-reader scenario or mitigate its impacts.

Proposal 1: In R19, reader ID is not included in A-IoT paging message for multi-reader scenario.

### 2.1.2 Device behavior when new service is received during an ongoing procedure

During RAN2#129bis meeting, the following agreement is made:

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

During the post email discussion after RAN2#129 meeting and online discussion in RAN2#129bis meeting, the following options of device behavior are in table:

- Option a: ignore the new service request, if device gets a new service request while one procedure is still ongoing.

- Some companies think that Option a is a simpler solution compared to Option b. While we think that Option a is closely related to the scheme for an A-IoT device to determine whether the previous procedure is ongoing or not, such as: timer-based scheme, explicit message or implicit message, etc. For timer-based scheme, we are not sure whether it can work well for device 1. For message-based scheme, an A-IoT device may become unreachable for a period of time, if it misses such message(s) and timer-based scheme may also be needed as a supplemantary solution.

- Option b: terminate the ongoing procedure and respond to the latest request.

- We think Option b is a simple and efficient scheme, because once an A-IoT device is paged by a new A-IoT paging message with new transaction ID, it considers the previous ongoing procedure is terminated and responds to the latest A-IoT paging message, that is implicit message (i.e. A-IoT paging message with new transaction ID) is used for determing whether one onging procedure is terminated.

- Note that some companies think that Option a is forward compatible, while Option b brings some concerns. However, we think that with the reserved bits in R2D messages, as introduced in section 2.1.3, both Option a and Option b can be forward compatible.

- Option c: depend on device implementation.

- If Option c is supported, it is hard for the gNB-reader to predict the behavior of devices which are performing the ongoing procedure because different devices may have different implementations, that is, some devices continue the ongoing procedure, while others terminate it. Therefore, Option c is asked to be avoided.

Observation 5: In terms of the device behavior when new service is received during an ongoing procedure, Option a has a greater specification impact than Option b, because RAN2 needs to discuss how the device determines whether the previous procedure is terminated. Leaving it to the device implementation will make it hard for the gNB-reader to predict the behavior of the devices.

Proposal 2: In terms of the device behavior when it receives a new A-IoT paging during an ongoing procedure, terminating the ongoing procedure and responding to the latest request (i.e. Option b) is preferred. Leaving it to device implementation should be avoided.

### 2.1.3 Forward compatibility

In terms of forward compatibility, only the case that multiple paging IDs are included in one A-IoT paging message is listed in R19 WID. For multi-reader scenario and/or parallel service case, we do not think either of them has been agreed as the direction for forward compatibility yet, and they are still in discussion.

In RAN2#129bis meeting, it has been agreed that:

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

From our point of view, the features and/or scenarios of the future release are still undetermined, because the objectives and core part of R20 A-IoT WID are still under discussion. Therefore, RAN2 should just reserve enough bit(s) which are expected to be used for future features when designing the format of R2D messages (e.g. A-IoT paging message, R2D command message, etc.) and do not touch any details of future features and scenarios which should be discuss and determined in RAN.

Observation 6: In R19 WID, only the case that multiple paging IDs are included in one A-IoT paging message is listed.

Observation 7: The objectives and core part of R20 A-IoT WID are still undetermined in RAN.

Proposal 3: In terms of the forward compatibility of A-IoT paging message, RAN2 should only focus on reserved bit(s) and do not touch any details of future features and scenarios.

### 2.1.4 Paging identifier

#### 2.1.4.1 Visibility of paging identifier to A-IoT MAC layer

In RAN2#129 meeting, it has been agreed that [2]:

2.The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer

In RAN2#129 meeting, some companies think that the paging identifier may need to be visible to A-IoT MAC layer in gNB-reader so that sub-grouping scheme can be performed. From our perspective, we think that it is an optimization scheme for the A-IoT procedure and totally transparent to A-IoT devices. And we do not find much benefits of this scheme.

Apart from this, since AS ID can also be used for the purpose of D2R scheduling and R2D reception, it seems that the identifier transmitted from CN, which is being discussed by RAN3 and SA2, should be visible to at least the A-IoT MAC layer in gNB-reader, so that the gNB-reader can map it to the corresponding AS ID or vice versa. Since whether a new ID pair in NG-AP is needed and the relationship between the ID pair and paging identifier is still undetermined, we can conclude that, from the perspective of specification impact, there is no clear necessity for the paging identifier to be visible to the A-IoT MAC layer at this stage, and may come back if necessary.

Observation 8: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure, which may be achieved by reader implementation, and is totally transparent to A-IoT device.

Observation 9: Whether gNB-CN device ID is needed and the relationship between gNB-CN device ID and paging identifier have not determined by RAN3/SA2.

Proposal 4: From the perspective of specification impact, there is no clear necessity for the paging identifier to be visible to the A-IoT MAC layer at this stage, and may come back if necessary.

#### 2.1.4.2 Details of paging identifier

In RAN2#129bis meeting, the following agreements have been made:

A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. Send an LS to SA2 to take this into account for their design.

Then, we think that we can just wait for SA2 and CT4 reply on the paging identifier length before designing the detailed specifics of paging identifier field.

Proposal 5: RAN2 is asked to wait for SA2 and CT4 reply on the paging identifier length before designing the detailed specifics of paging identifier field.

### 2.1.5 Transaction ID

In RAN2#129 meeting, the following agreements have been made [2]:

3.The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4.1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

- As for the generation of transaction ID, it is not in RAN2 scope. And as proposed in section 2.1.1, there is also no need to include or reflect reader ID in the transaction ID.

- With the consideration that In terms of the transaction ID, it is only used in A-IoT radio interface by RAN2 and is totally transparent to CN and physical layer. Thus, we think that we can determine the size of transaction ID by ourselves. In Re-19, since only gNB-reader is in scope and multi-reader scenario can be avoided by implementation as introduced in Section 2.1.1, the size of transaction ID does not need to be too long. Based on above analysis, we think 4-bit transaction ID is enough.

Observation 10: The method for gNB-reader generating transaction ID is not in RAN2 scope.

Proposal 6: There is no need to include/reflect reader ID in the transaction ID.

Proposal 7: RAN2 is asked to agree to a 4-bit transaction ID.

### 2.1.6 Resource indication

In RAN1#120bis meeting, it has been agreed that [3]:

Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH.

Based on the latest agreement in RAN1, it is clear that parameters for the D2R transmission resources should be included in A-IoT MAC layer message, while the details of such parameters have not been agreed yet. To this end, what RAN2 needs to do is wait for RAN1 agreement on detailed parameters and design the field of shared resources for A-IoT MSG1 transmission and the field of dedicated resources for first D2R message transmission in A-IoT paging message, accordingly. Note that RAN2 has agreed that A-IoT paging message can include a number of A-IoT MSG1 resources, while we think that the indication of this number should also take RAN1’s agreement into consideration, because the number of time-domain resources and the frequency-domain information may need to be indicated separately.

Observation 11: According to RAN1 agreement, it is clear that parameters for the D2R transmission resources should be included in A-IoT MAC layer message.

Observation 12: The indication of number of A-IoT MSG1 resources should also take RAN1’s agreement into consideration, because the number of time-domain resources and the frequency-domain information may need to be indicated separately.

Proposal 8: RAN2 is asked to wait for RAN1 agreement on detailed parameters of D2R transmission resources, and design the field of shared resources for A-IoT MSG1 transmission and the field of dedicated resources for first D2R response transmission in A-IoT paging message, accordingly.

## 2.2 General design for A-IoT paging message

In RAN2#129bis meeting, it has been agreed that:

Agreements on CFRA Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. Agreements on paging ID length A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

Based on above analysis and agreements in RAN2#129bis meeting, there are six contents which may be included in an A-IoT paging message, that is, message type, CBRA or CFRA indication, transaction ID, paging identifier, reserved bit(s) for future features and resource indication. For message type, CBRA or CFRA indication and reserved bit(s), they are mandatory and are of fixed lengths. For transaction ID, we think that it may be absent for A-IoT paging message which triggers CFRA, since one A-IoT device should always respond to the A-IoT paging which only targets to itself, regardless of the transaction ID. Therefore, we think that the transaction ID is with fixed length and whether it exists in A-IoT paging message or not can be indicated by the agreed 1-bit CBRA/CFRA indication. As for paging identifier, it is optional because it has been agreed that no identifier can be contained in A-IoT paging message to page all A-IoT devices. And the length of paging ID is variable. Finally, in terms of the resource indication whose details depend on RAN1, RAN2 is asked to discuss whether we need to treat each parameter (such as: chip duration, number of FDMed occasions, etc.) agreed by RAN1 as a separate field, and discuss respectively whether it is mandatory or optional.

Based on above analysis, we think that the following principle is asked to be supported by RAN2 at first:

- For each optional field in A-IoT paging message, one indication is needed to indicate whether it (maybe together with its length indication) exists in the A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length information indication, which indicates the length directly or indicates the length option, is needed in the A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication to reduce the overhead of A-IoT device(s) receiving the A-IoT paging message.

Observation 13: There are six contents which may need to be included in an A-IoT paging message, that is, message type, CBRA or CFRA indication, transaction ID, paging identifier, reserved bit(s) for future features and resource indication.

- Proposal 9: RAN2 is asked to agree that transaction ID can be absent in A-IoT paging message which triggers CFRA.

- Proposal 10: At least for the design of A-IoT paging message, RAN2 is asked to agree to the following designing principles:

- For each optional field in A-IoT paging message, one indication is needed to indicate whether it (maybe together with its length indication) exists in the A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length information indication is needed to indicate its length or the length option in the A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication.

Proposal 11: For resource indication, RAN2 is asked to discuss whether we need to treat each parameter (such as: chip duration, number of FDMed occasions, etc.) agreed by RAN1 as a separate field, and discuss respectively whether it is mandatory or optional, after RAN1 provides the parameter list.

According to above principle, we think the following fields may be included in one A-IoT paging message:

- M: Message type (mandatory)

- B/F: 1-bit CBRA or CFRA indication (mandatory)

- T: Transaction ID (optional)

- I: Indication which indicates whether paging identifier and its length information is present or absent (mandatory)

- L&P: Length information of Paging identifier and Paging identifier field (optional)

- RI: Resource indication of shared resources for A-IoT MSG1 transmission or dedicated resources for first D2R message transmission (details depend on RAN1)

- R: Reserved bit(s) (mandatory)

Then, an overview diagram of the format of A-IoT paging message can be given in Figure 2.

Figure 2. Overall diagram of the format of A-IoT paging message

We further shown the specific format of A-IoT paging messages which pages all devices, a group of devices and one specific device in Figure 3-5, respectively.

Figure 3. Schematic diagram of the format of A-IoT paging message which pages all devices

Figure 4. Schematic diagram of the format of A-IoT paging message which pages group of devices

Figure 5. Schematic diagram of the format of A-IoT paging message which pages one specific device

- Proposal 12: The following fields may be included in the A-IoT paging message:

- M: Message type (mandatory)

- B/F: 1-bit CBRA or CFRA indication (mandatory)

- T: Transaction ID (optional)

- I: Indication which indicates whether paging identifier and its length information is present or absent (mandatory)

- L&P: Length information of Paging identifier and Paging identifier field (optional)

- RI: Resource indication of shared resources for A-IoT MSG1 transmission or dedicated resources for first D2R message transmission (details depend on RAN1)

- R: Reserved bit(s) (mandatory)

- Proposal 13: RAN2 is asked to take the overall format of A-IoT paging message shown in Figure 2 and specific format of A-IoT paging message shown in Figure 3 to Figure 5 as a start point.

## 2.3 Subsequent A-IoT paging

In RAN2, it has been discussed and agreed that one or more subsequent A-IoT paging messages can be sent by gNB-reader to trigger the A-IoT device(s) which fails to complete the procedure initiated by initial A-IoT paging message. In fact, with the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices. To this end, we think that there should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than D2R transmission resources field. RAN2 can further discuss whether the D2R transmission resources field can be different between subsequent paging message and initial paging message.

Observation 14: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices.

- Proposal 14: There should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than resource indication field.

Proposal 15: RAN2 is asked to discuss whether resource indication field can be different between subsequent paging message and initial paging message.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on A-IoT paging as following:

Observation 1: When different readers perform inventory procedures simultaneously, the interference of the signals received by the devices located in the overlapping coverage area may be extremely severe.

Observation 2: For the case where different readers perform inventory procedures interleavingly, it seems that there is no improvement in efficiency, but only an increase in the complexity of the protocol.

Observation 3: In R19 WID, only the case that multiple paging IDs are included in one A-IoT paging message is listed as a forward compatibility issue. For multi-reader scenario and/or parallel service case, neither of them has been agreed as the direction for forward compatibility yet.

Observation 4: There are several implementation methods to avoid the occurrence of multi-reader scenario or mitigate its impacts.

Observation 5: In terms of the device behavior when new service is received during an ongoing procedure, Option a has a greater specification impact than Option b, because RAN2 needs to discuss how the device determines whether the previous procedure is terminated. Leaving it to the device implementation will make it hard for the gNB-reader to predict the behavior of the devices.

Observation 6: In R19 WID, only the case that multiple paging IDs are included in one A-IoT paging message is listed.

Observation 7: The objectives and core part of R20 A-IoT WID are still undetermined in RAN.

Observation 8: Sub-grouping performed by the reader is an optimization scheme for the A-IoT procedure, which may be achieved by reader implementation, and is totally transparent to A-IoT device.

Observation 9: Whether gNB-CN device ID is needed and the relationship between gNB-CN device ID and paging identifier have not determined by RAN3/SA2.

Observation 10: The method for gNB-reader generating transaction ID is not in RAN2 scope.

Observation 11: According to RAN1 agreement, it is clear that parameters for the D2R transmission resources should be included in A-IoT MAC layer message.

Observation 12: The indication of number of A-IoT MSG1 resources should also take RAN1’s agreement into consideration, because the number of time-domain resources and the frequency-domain information may need to be indicated separately.

Observation 13: There are six contents which may need to be included in an A-IoT paging message, that is, message type, CBRA or CFRA indication, transaction ID, paging identifier, reserved bit(s) for future features and resource indication.

Observation 14: With the consideration of the unavailability of A-IoT devices caused by their insufficient energy or fading in PRDCH, from the reader's perspective, the subsequent A-IoT paging message it sends may be treated as initial A-IoT paging messages by some A-IoT devices.

Proposal 1: In R19, reader ID is not included in A-IoT paging message for multi-reader scenario.

Proposal 2: In terms of the device behavior when it receives a new A-IoT paging during an ongoing procedure, terminating the ongoing procedure and responding to the latest request (i.e. Option b) is preferred. Leaving it to device implementation should be avoided.

Proposal 3: In terms of the forward compatibility of A-IoT paging message, RAN2 should only focus on reserved bit(s) and do not touch any details of future features and scenarios.

Proposal 4: From the perspective of specification impact, there is no clear necessity for the paging identifier to be visible to the A-IoT MAC layer at this stage, and may come back if necessary.

Proposal 5: RAN2 is asked to wait for SA2 and CT4 reply on the paging identifier length before designing the detailed specifics of paging identifier field.

Proposal 6: There is no need to include/reflect reader ID in the transaction ID.

Proposal 7: RAN2 is asked to agree to a 4-bit transaction ID.

Proposal 8: RAN2 is asked to wait for RAN1 agreement on detailed parameters of D2R transmission resources, and design the field of shared resources for A-IoT MSG1 transmission and the field of dedicated resources for first D2R response transmission in A-IoT paging message, accordingly.

- Proposal 9: RAN2 is asked to agree that transaction ID can be absent in A-IoT paging message which triggers CFRA.

- Proposal 10: At least for the design of A-IoT paging message, RAN2 is asked to agree to the following designing principles:

- For each optional field in A-IoT paging message, one indication is needed to indicate whether it (maybe together with its length indication) exists in the A-IoT paging message.

- For each field (whether optional or mandatory) with variable length, a length information indication is needed to indicate its length or the length option in the A-IoT paging message.

- For the mandatory field(s) with fixed length, it(they) can be tightly arranged without any additional indication.

Proposal 11: For resource indication, RAN2 is asked to discuss whether we need to treat each parameter (such as: chip duration, number of FDMed occasions, etc.) agreed by RAN1 as a separate field, and discuss respectively whether it is mandatory or optional, after RAN1 provides the parameter list.

- Proposal 12: The following fields may be included in the A-IoT paging message:

- M: Message type (mandatory)

- B/F: 1-bit CBRA or CFRA indication (mandatory)

- T: Transaction ID (optional)

- I: Indication which indicates whether paging identifier and its length information is present or absent (mandatory)

- L&P: Length information of Paging identifier and Paging identifier field (optional)

- RI: Resource indication of shared resources for A-IoT MSG1 transmission or dedicated resources for first D2R message transmission (details depend on RAN1)

- R: Reserved bit(s) (mandatory)

- Proposal 13: RAN2 is asked to take the overall format of A-IoT paging message shown in Figure 2 and specific format of A-IoT paging message shown in Figure 3 to Figure 5 as a start point.

Figure 2. Overall diagram of the format of A-IoT paging message

Figure 3. Schematic diagram of the format of A-IoT paging message which pages all devices

Figure 4. Schematic diagram of the format of A-IoT paging message which pages group of devices

Figure 5. Schematic diagram of the format of A-IoT paging message which pages one specific device

- Proposal 14: There should be no difference between the subsequent A-IoT paging message and initial A-IoT paging message for at least the fields other than resource indication field.

Proposal 15: RAN2 is asked to discuss whether resource indication field can be different between subsequent paging message and initial paging message.

Reference

- “RAN2 chair’s notes”, RAN2#129bis, Wuhan, China, RAN2 Chair (InterDigital).

- “RAN2 chair’s notes”, RAN2#129, Greece, Athens, RAN2 Chair (InterDigital).

- “RAN1 chair’s notes”, RAN2#120bis, Wuhan, China, RAN1 Chair.