3GPP TSG-RAN WG2#130		      					R2-2504412

St. Julians, Malta, May 19 – 23, 2025

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Remaining issue of Conditional intra-CU LTM

Document for:	Discussion

Introduction

The following is the RAN2#129-bis agreement of Conditional intra-CU LTM [1]:

Agreements on C-LTM 1. Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. 2. C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer. 3. RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. 4. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

It is necessary to consider the remaining issues. So, this paper discusses a scenario in which multiple execution conditions are triggered simultaneously.

Discussion

Open issue RRC-3

According to the Editor’s Note in the List of RRC FFSs for mobility [2], FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

2.3	RRC-3

Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

Rapporteur comment: Current procedural text leaves this case to the UE implementation (similar to CHO), so the proposal is to keep current text as it is and delete the FFS.

Differences between CLTM and CHO

According to the TS38.331 [3] below, CHO has specifications for cases where multiple execution conditions are triggered simultaneously, and it is up to UE implementation which candidate cell to perform handover to.

5.3.5.13.5	Conditional reconfiguration execution

The UE shall:

1> if more than one pair of triggered PCell and associated triggered PSCell exist:

2> select one of the triggered PCell(s) and the associated triggered PSCell(s) as the selected cells for

conditional reconfiguration execution;

[…]

NOTE:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.

However, there are differences between CLTM and CHO;

Table 1 Differences between CLTM and CHO

|  | Type of execution conditions | Timing of TA value acquisition procedure |
| --- | --- | --- |
| CHO | L3 execution condition | Upon cell switch execution (i.e., Upon access to the target cell) |
| CLTM | L3 execution condition and/or L1 execution condition | Before cell switch execution or Upon cell switch execution (i.e., Upon access to the target cell) |

In the case of CHO, NW can configure only L3 execution condition for a CHO candidate cell. The timing for acquisition the TA value is only upon cell switch execution, and there is no mechanism for acquiring the TA value in advance.

In CHO, the UE evaluates multiple execution conditions at the same layer (i.e., with L3 measurements), so there is no difference in the evaluation stability.

For CHO, the UE acquires TA values when accessing the target cell.

While, in the case of CLTM, NW can configure L3 execution condition or L1 execution condition for a CLTM candidate cell, and there is a mechanism to acquire the TA value before cell switch execution. However, if the UE does not have a valid TA value upon cell switch execution, the UE acquires the TA value upon cell switch execution same as CHO.

In CLTM, the UE may evaluate multiple execution conditions at different layers (i.e., with L1 and L3 measurements), so the evaluation for L3 execution condition would be more stable than that for L1 execution.

Rel-19 CLTM supports the early TA acquisition procedure.

However, even in CLTM, when the timing of the TA value acquisition procedure is upon access to the target cell and multiple execution conditions are triggered simultaneously at the same layer, it can be considered the same as in CHO. So, in this case, which CLTM candidate cell the UE selects to execute the LTM is up to UE implementation, as proposed by the rapporteur.

Timing of TA value acquisition procedure

CLTM has a significant advantage over CHO in that RACH-less cell switch can be executed by acquiring TA values in advance. This is because RACH-less cell switch can enable lower latency cell switch. To maximize this advantage, if multiple execution conditions are triggered simultaneously, the UE should prioritize the CLTM candidate cell with the valid TA value regardless of execution condition type.

CLTM has a significant advantage over CHO in that RACH-less cell switch can be executed by acquiring TA values in advance.

RAN2 should agree that the UE prioritizes the CLTM candidate cell with the valid TA value over the one without the TA value regardless of execution condition type, if multiple execution conditions are triggered simultaneously.

Execution conditions in different layers

It is also the CLTM specific issue that execution conditions in different layers may be triggered simultaneously. Since there has been no discussion about multiple execution conditions being triggered at different layers, it should be considered as a new scenario.

In the case, there are three types of UE behaviour as below:

- Type 1: The UE accesses the CLTM candidate cell triggering the L1 execution condition.

- Type 2: The UE accesses the CLTM candidate cell triggering the L3 execution condition.

- Type 3: It is up to UE implementation to select the CLTM candidate cells.

- According to the TS38.300[4] below, after driving beam quality at the physical layer (i.e., L1 measurement), cell quality is derived from multiple beams at the RRC level (i.e., L3 measurement). Therefore, compared to the L1 execution condition (i.e., Type 1), the L3 execution condition (i.e., Type 2), which uses the results of the L3 measurement, is considered to have higher reliability and stability. Consequently, when the execution conditions of L1 and L3 are simultaneously fulfilled, the UE should prioritize accessing the CLTM candidate cell that fulfills the execution condition of L3 (Type 2) over L1 (Type 1).

- Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams.

- On the other hand, it is also possible to consider Type 3, where the selection of the CLTM candidate cell is up to UE implementation, as in CHO.

RAN2 should agree that the UE prioritizes the CLTM candidate cell triggering the L3 execution condition over the one triggering the L1 execution condition, if multiple different type of execution conditions is triggered simultaneously where the UE has valid TA values (or the UE does not have valid TA values for any of them).

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the observations and proposals below:

Observation 1	In CHO, the UE evaluates multiple execution conditions at the same layer (i.e., with L3 measurements), so there is no difference in the evaluation stability.

Observation 2	For CHO, the UE acquires TA values when accessing the target cell.

Observation 3	In CLTM, the UE may evaluate multiple execution conditions at different layers (i.e., with L1 and L3 measurements), so the evaluation for L3 execution condition would be more stable than that for L1 execution.

Observation 4	Rel-19 CLTM supports the early TA acquisition procedure.

Observation 5	CLTM has a significant advantage over CHO in that RACH-less cell switch can be executed by acquiring TA values in advance.

Proposal 1	RAN2 should agree that the UE prioritizes the CLTM candidate cell with the valid TA value over the one without the TA value regardless of execution condition type, if multiple execution conditions are triggered simultaneously.

Proposal 2	RAN2 should agree that the UE prioritizes the CLTM candidate cell triggering the L3 execution condition over the one triggering the L1 execution condition, if multiple different type of execution conditions is triggered simultaneously where the UE has valid TA values (or the UE does not have valid TA values for any of them).

References

- R2-2502981 “Report from session on R18 SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#129bis

- R2-250xxxx “List of RRC FFSs for mobility”, Ericsson, RAN2#130

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP