3GPP TSG RAN WG2 #130			R2-2504418

St Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[5]. In this contribution, we provide our views on the first scope on on-demand SSB SCell operation.

Agreements on OD-SSB SCell@RAN2#129bis

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

Agreements on OD-SSB SCell@RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Agreements on OD-SSB@RAN2#127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell@RAN2#127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- Discussion

2.1	Signaling for OD-SSB

Agreement@RAN1#119 For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

RAN1 agreed that whether/how to support deactivation of OD-SSB by RRC is up to RAN2 [6]. While RAN2 has already supported RRC to indicate initial activation/deactivation state of OD-SSB configuration, in our understanding, it is not decided yet whether to support RRC indicating deactivation of OD-SSB which is already activated. We think, since the similar framework in legacy (SCell deactivation, SP CSI-RS) does not support RRC based deactivation, it seems straightforward to follow the legacy concept also for OD-SSB. It is enough to define the functionality of RRC for already activated OD-SSB as OD-SSB configuration release.

Proposal 1:

- Not support RRC based deactivation of OD-SSB that is already activated.

2.1.1	Issue of MAC-1

In the TS38.321 CR rapporteur’s summary, the following remaining issue was identified [7].

Issue 1: How to define single MAC CE format for both explicit and implicit OD-SSB deactivation

Although RAN1 is still discussing what parameter sets can be indicated via MAC CE and whether these parameters should be configured via RRC (before MAC CE indication) for OD-SSB activation, RAN2 can assume that a lot of parameters related to OD-SSB properties (e.g., SSB burst periodicity, SSB index) are RRC-configured as OD-SSB configurations, and MAC CE can indicate which one(s) to be activated or deactivated.

As it was agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE, corresponding SCell index(s) also should be indicated.

In the previous meeting, it was concerned that, when one OD-SSB#1 on SCell#A is already activated and transmitted and NW wants to activate or deactivate another OD-SSB#2 on SCell#B, then indication for OD-SSB#2 via MAC CE might affect the state of OD-SSB#1 (e.g., re-activate and reset the number of OD-SSB burst or deactivate OD-SSB#1).

Our position is that, at least, independent indication per OD-SSB/SCell should be achieved. If OD-SSB configured with the finite number of N (meaning it to be deactivated implicitly) cannot be explicitly deactivated, then the above issue can be solved by UE simply ignoring the explicit deactivation indication for the activated OD-SSB to be implicitly deactivated. Otherwise, we think that 1 bit indication for either of activation or deactivation per serving cell is not sufficient and it is better to introduce indications representing which SCell(s) is intended by the MAC CE signalling.

Additionally, if OD-SSB is associated with a BWP rather than a cell, (e.g., OD-SSB is NCD-SSB config-ured per BWP), OD-SSB config ID may not be unique per serving cell, thus corresponding BWP index should also be indicated.

Proposal 2: (MAC-1)

- For signaling design of MAC CE based indication of OD-SSB, the following fields can be considered:

  - If OD-SSB is configured per BWP not per serving cell, BWP index

  - If OD-SSB configured with number of N (to be deactivated implicitly) can be deactivated explicitly via MAC CE, 1 bit indication per SCell to represent which SCel is intended on top of 1 A/D bit per SCell.

2.2	Impact on L3 measurement

In the TS38.331 CR rapporteur’s summary, the following remaining issues on TS38.331 were identified [8].

3) FFS on L3 frame work (i.e., based on different measurement configuration or based on OD-SSB pattern ignoring SMTC when OD-SSB pattern is transmitted. 4) If the od-ssb-PositionsInBurst of OD-SSB and AO-SSB is different, how the UE adapts the ssb-ToMeasure of MeasObjectNR after OD-SSB is activated.

2.2.1	Issue of RRC-ODSSB-3

There have been some discussions about whether to extend the current L3 measurement framework by introducing OD-SSB. We believe that a new mechanism for L3 measurement to follow dynamic changes in SSB properties can be supported.

Since RAN1 agreed that multiple OD-SSB properties including OD-SSB burst periodicity, are RRC-configured, and MAC CE indicates one of them, therefore, OD-SSB burst periodicity to be measured would dynamically change over the legacy operation.

The current L3 measurement configuration can set the periodicities from 5ms to 160ms and the SSB indexes to be measured as measurement object, which may or may not be aligned with the actual SSB burst periodicity and SSB index completely. This means which SSB occasions to be measured are determined by a semi-static manner.

If we consider when NW determines to activate OD-SSB (and what property of OD-SSB), one criterion should be that UE’s speed and channel condition in a given cell. For example, if a UE is in relatively high speed, the UE should be provided with OD-SSB whose periodicity is relatively shorter to follow the channel fluctuation so that NW can obtain accurate measurement result from the UE in a timely manner. In this situation, it is preferable that all indicated OD-SSB should be L3-measured for accurate and timely evaluation of the SCell strength. If the update of measurement object requires the legacy scheme, RRC Reconfiguration signaling, there seems to be less benefit by introducing OD-SSB, because anyway gNB has to send RRC signaling for applying OD-SSB TX and corresponding measurement. Even using the legacy scheme, it is indeed possible to set two measurement configurations where one corresponds to short SSB and the other is longer SSB in one frequency, while this is an inefficient way from UE measurement budget and energy consumption perspective, so this should be avoided.

Observation 1: (RRC-ODSSB-3)

- It is expected that NW would determine whether and what property to activate OD-SSB on deactivated SCell for faster / more accurate L3 measurement according to UE’s condition (e.g., UE’s speed).

Proposal 3: (RRC-ODSSB-3)

- Introduce a mechanism that measurement object, if this includes time and frequency of OD-SSB, is dynamically changed according to the indicated OD-SSB.

On the details of how to enhance the current L3 framework according to OD-SSB indication, the following two directions can be considered.

(1) Measurement configurations to be used are indicated together with OD-SSB indication (via MAC CE)

(2) Measurement configurations to be used are associated with an OD-SSB, and only after OD-SSB indication, the corresponding configurations are applicable.

As discussed above, since it seems to be an appropriate operation that measurement occasions for OD-SSB TX are aligned with the actual OD-SSB TX patterns, we think direction (2) would be sufficient. The direction (1) could lead to unnecessary flexibility, putting a burden on UEs to prepare for possible combinations of measurement configurations and OD-SSB TX patterns.

On which parameters to be enhanced in measurement configurations, we think, at least, SMTC configurations to set periodicity and duration, should be enhanced, as RAN1 agreed that multiple SSB burst periodicities and SFN offsets were configured in each OD-SSB configuration. Whether the other parameters, e.g., frequency and PCI, should be configurable in each OD-SSB configs is now under discussion, so RAN2 will consider the necessary L3 measurement parameters associated with each OD-SSB as agreed in RAN1.

2.2.1	Issue of RRC-ODSSB-4

On issue of RRC-4, we see that RAN1 agreed that SSB positions within OD-SSB burst can be configured differently from that within AO-SSB burst in the case of different frequencies, while it is not yet agreed whether to allow different SSB positions between OD-SSB and AO-SSB with same center frequency [9]. We suggest that RAN2 start discussions to address this issue if such a situation is agreed in RAN1.

Agreement@RAN1#120bis For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. • SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases o The case where center frequency of AO-SSB and OD-SSB are different o Case 1 • Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions

Proposal 4 : (RRC-ODSSB-4)

- RAN2 to start discussion on the following issue after RAN1 agrees to support that AO-SSB and OD-SSB with same center frequency can have different ssb-PositionsInBurst.

  - If the od-ssb-PositionsInBurst of OD-SSB and AO-SSB is different, how the UE adapts the ssb-ToMeasure of MeasObjectNR after OD-SSB is activated.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SSB SCell operation for network energy saving.

Proposal 1:

- Not support RRC based deactivation of OD-SSB that is already activated.

Proposal 2: (MAC-1)

- For signaling design of MAC CE based indication of OD-SSB, the following fields can be considered:

  - If OD-SSB is configured per BWP not per serving cell, BWP index

  - If OD-SSB configured with number of N (to be deactivated implicitly) can be deactivated explicitly via MAC CE, 1 bit indication per SCell to represent which SCel is intended on top of 1 A/D bit per SCell.

Observation 1: (RRC-ODSSB-3)

- It is expected that NW would determine whether and what property to activate OD-SSB on deactivated SCell for faster / more accurate L3 measurement according to UE’s condition (e.g., UE’s speed).

Proposal 3: (RRC-ODSSB-3)

- Introduce a mechanism that measurement object, if this includes time and frequency of OD-SSB, is dynamically changed according to the indicated OD-SSB.

Proposal 4 : (RRC-ODSSB-4)

- RAN2 to start discussion on the following issue after RAN1 agrees to support that AO-SSB and OD-SSB with same center frequency can have different ssb-PositionsInBurst.

  - If the od-ssb-PositionsInBurst of OD-SSB and AO-SSB is different, how the UE adapts the ssb-ToMeasure of MeasObjectNR after OD-SSB is activated.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127 meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- 3GPP, RAN1#119 meeting, Chairman’s notes

- R2-2504281, Summary of Comments to 38.321 CR for NES	, InterDigital

- R2-2503608, Report of [POST129b][111][NES] (Ericsson),	Ericsson

- 3GPP, RAN1#120bis meeting, Chairman’s notes