3GPP TSG RAN WG2 #130			R2-2504420

St Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on adaptation of common signal and channel

Agenda Item:	8.5.4

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[5]. In this contribution, we provide our views on adaptation of common signal and channel.

Agreements on OD-SSB@RAN2#129bis

1.	For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

2.	For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

3.	Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

4.	Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

5.	For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

6.	Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Agreements on OD-SSB@RAN2#129

On Paging adaptation

1.	For N, values smaller than T/32 are not supported.

2.	The maximum possible value for Ns is 8.

3.	Introduce a separate PEI configuration.

4.	Paging adaptations are configured semi-statically and updated via system information update notification.

5.	A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

on SSB adaptation

1.	RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

2.	RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

3.	For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

- 5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

- 5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

Agreements on adaptation@RAN2#127bis

Agreements on paging adaptation

1.	Select option-b as baseline for R19 NES paging enhancement.

2.	R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

3.	Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

4.	Legacy UE is not barred.

5.	Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

- Discussion

2.1	PRACH adaptation

RAN1 agreed that additional PRACH resources are semi-statically provided, and DCI is used to indicate its availability. About the discussion of signaling, some companies mention that additional PRACH resources should be available via semi-static signaling only, i.e., without DCI. However, we rather believe DCI-based provision should be supported. This is because, there seems no introduction of new parameters applied to additional PRACH to generate more confined patterns, which means additional PRACH finally turns out to be almost same as legacy PRACH resources. In this sense, if additional PRACH resources are provided by semi-static signaling, we can say the legacy SIB1 can provide almost all PRACH patterns that is combination of legacy and additional. We cannot see any motivation to support what the legacy framework already supports, so additional PRACH resources should be activated only via DCI.

Proposal 1:

- For PRACH adaptation mechanism, it is preferable not to support additional PRACH resources that are available via semi-static signaling (i.e., SIB1) only, i.e., support additional PRACH resources that are only available when DCI is indicated.

2.2.1	Issue of RRC-RACH adaptation-1

In the TS38.331 CR rapporteur’s summary, the following remaining issues on TS38.331 were identified [8].

1) FFS: How to configure additional RACH resource for RACH adaptation in RRC, with consideration of RAN1 input.

RAN1 discussed and then made the following relationship and restrictions of parameters for additional PRACH resources with legacy PRACH so far [6][7].

Agreement@120 - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported Agreement@119 At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

We can consider two ways in general to define parameter sets for introducing an enhancement, one way is to define only new parameters to be configured for additional PRACH, which can be different from those for the legacy PRACH under the legacy PRACH configurations. Another way is to define new parameter sets that specify entire additional PRACH resources.

When it comes to this enhancement for PRACH adaptation, the independent parameters of additional ones are limited, such as PRACH config. index, msg1-FDM and msg1-frequencyStart. Therefore, we prefer to define new RRC parameters of additional PRACHs under those of the legacy IEs (i.e., RACH-ConfigGeneric). This can reduce the impact of specification and potential signaling overhead in system information. If we take the way to define whole sets for additional PRACH, most of the parameters except for the above parameters should be set to the same value as legacy and we will have a lot of field descriptions for these parameters in TS38.331 to limit the value to be the same as legacy.

Proposal 2: (RRC-RACH adaptation-1)

- Prefer to define new RRC parameters of additional PRACH that can be configured differently from the legacy ones only (e.g., PRACH config. index, msg1-FDM and msg1-frequencyStart) under the corresponding legacy IEs (e.g., RACH-ConfigGeneric)

2.2.2	Issue of MAC-2

In the TS38.321 CR rapporteur’s summary, the following remaining issue on TS38.321 was identified [9].

Issue 2: Whether there is a need to capture selection/availability of additional RACH resources for RACH adaptation.

For additional PRACH operation, the enhanced behaviours have been agreed to support, such as new PRACH mask to identify available subset of additional PRACH and DCI based indication to inform availability of additional PRACH resources. In the legacy MAC spec., it is specified that MAC layer identifies available PRACH resources considering the RRC parameters (e.g., ra-ssb-OccasionMaskIndex) and selects one of them. In this sense, the aspects mentioned above should be captured in MAC spec. for additional PRACH.

Proposal 3: (MAC-2)

- At least, the following aspects should be captured in MAC spec. for additional PRACH resources.

  - MAC layer would apply new PRACH mask configured by RRC to additional PRACH resources

  - MAC layer can use additional PRACH resources when lower layer indicates that they are available until indicated that they are unavailable.

- Conclusion

In this contribution, we provided the following observations and proposals of adaptation of common signal and channel for network energy saving.

Proposal 1:

- For PRACH adaptation mechanism, it is preferable not to support additional PRACH resources that are available via semi-static signaling (i.e., SIB1) only, i.e., support additional PRACH resources that are only available when DCI is indicated.

Proposal 2: (RRC-RACH adaptation-1)

- Prefer to define new RRC parameters of additional PRACH that can be configured differently from the legacy ones only (e.g., PRACH config. index, msg1-FDM and msg1-frequencyStart) under the corresponding legacy IEs (e.g., RACH-ConfigGeneric)

Proposal 3: (MAC-2)

- At least, the following aspects should be captured in MAC spec. for additional PRACH resources.

  - MAC layer would apply new PRACH mask configured by RRC to additional PRACH resources

  - MAC layer can use additional PRACH resources when lower layer indicates that they are available until indicated that they are unavailable.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- 3GPP, RAN1#119 meeting, Chairman’s notes

- 3GPP, RAN1#120 meeting, Chairman’s notes

- R2-2503608, Report of [POST129b][111][NES] (Ericsson),	Ericsson

- R2-2504281, Summary of Comments to 38.321 CR for NES	, InterDigital