3GPP TSG-RAN WG2 Meeting #130	R2-2504421

St Julian's, Malta, 19th – 23rd May, 2025

Agenda Item:	8.8.4

Source:    TCL.

Title:         Discussion on Supporting of MBS Broadcast Service

Document for:	Discussion and Decision

# Introduction

3GPP is discussing providing mmulticast/broadcast service (MBS) features over NR NTN system, leveraging the large coverage area of NTN compared to TN for MBS deployment. During 3GPP RAN#102, a new Rel-19 satellite access related WID was approved [1] targeting enabling MBS features over NTN in the 5G specifications. The major focus this WID, is to enhancements NTN/MBS to address the scenario when intended service area of MBS broadcast is smaller than the coverage of an NTN cell or a satellite beam. During last meetings the following agreements has been made on this regard:

Agreements: 1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH. 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. 4. The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE

This report discusses the enhancements for supporting of MBS broadcasting over NTN focusing on signalling of intended MBS broadcasting area taking into account these new agreements.

# Discussion

In RAN2#127 meeting it was agreed that MBS intended service area IDs can be included into MCCH.  While for the mapping between broadcast service session and intended service area, it was argued whether if it shall also be included in MCCH or provided in a new SIB or in an existing SIB. Regarding SIBs option, some companies have showed a concern on whether the size of the MBS SIB can fit for such mapping information if an existing SIB is used. Others argued that there will be a signaling duplication and delay because the UE may need to decode more than one SIB, one for intended service area IDs another for the mapping, if we consider introducing a new SIB. During RAN2#128 the following agreements has been made：

Agreements: 1. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 2. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

And during RAN2 #129 meeting [2]

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In last RAN2 meeting, it is agreed

Agreements: 1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts). 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH. 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behaviour applies

In practical scenarios, a UE typically initiates session release or establishment based on its knowledge of network identities (e.g., PLMN, tracking areas, and/or mapped cell identities) rather than explicit location information. For example, during connected or inactive-state mobility, a UE's location is defined by a set of network identities (e.g., cells or tracking areas), and based on these identities, the UE may perform actions such as releasing or establishing a radio bearer. Without knowledge of the identities defining the boundary of the intended service area, it is difficult for the UE to determine whether it is inside or outside the intended area. Consequently, the UE may not know when to (re)acquire MCCH, initiate the broadcast MRB establishment procedure upon entry, or release MCCH/MRB upon leaving the intended area. Additionally, if the network does not provide the UE with an association between the geographical area descriptions (covering the entire satellite footprint) and the network identities (e.g., PLMN IDs, tracking areas, or mapped cell IDs) in advance, the UE cannot determine whether a specific region within the satellite coverage is an intended service area for MBS. Based on the above discussion, we make the following observations:

Observation 1: A UE initiates actions based on network identities. Without knowledge of the identities defining the boundary of the intended service area, it is difficult for the UE to determine whether it is inside or outside the intended area. As a result, the UE may not know when to (re)acquire MCCH, initiate MRB establishment upon entry, or release MCCH/MRB upon leaving the intended area.

Observation 2: Without prior knowledge of the association between geographical area descriptions (covering both intended and unintended MBS service areas under satellite coverage) and network identities, the UE cannot determine whether a specific region within the satellite footprint is an intended service area (or an unintended one for MBS).

Based on these observations, we propose:

  - Proposal 1:

The new SIBxx needs to include mapping between network identities (PLMNs, tracking areas, or mapped cell identities) and geographical area definitions to enable UEs to identify MBS intended service areas.

  - Proposal 2:

The new SIBxx should also provide mapping between network identities and geographical definitions of the complete satellite coverage area (including both MBS intended and non-intended regions), allowing UEs to determine it is within an intended service area or not.

# Conclusion

In this contribution, we have discussed the enhancements for supporting MBS broadcasting over NTN focusing on the intended MBS broadcasting area, the signalling design for controlling MBS broadcasting over NTN and the service continuity issues. Based on the above discussion, we following proposals:

  - Proposal 1:

The new SIBxx needs to include mapping between network identities (PLMNs, tracking areas, or mapped cell identities) and geographical area definitions to enable UEs to identify MBS intended service areas.

  - Proposal 2:

The new SIBxx should also provide mapping between network identities and geographical definitions of the complete satellite coverage area (including both MBS intended and non-intended regions), allowing UEs to determine it is within an intended service area or not.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Netherlands, March 18-22, 2024.

- R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN, Session chair (ZTE)