3GPP TSG RAN WG2 #130       	                            R2-2504422

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source:	     KDDI Corporation

Title:	Discussion on L1 event triggered measurement reporting for LTM

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN #106, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1]:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

In RAN2#129, the following agreement was made:[2]

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we will discuss open issues form RAN2#129bis and share our views on L1 event triggered MR.

- Discussion

2.1 Unified format for all beams reported in MAC CE measurement reports

Option 1 can distinguish between four different beam types (type1, type2, type3, type4), which allows for detailed understanding and management of beam states and behaviours. This contributes to improved network operation and control flexibility, making it easier to adapt to various scenarios.

On the other hand, Option 3 is simpler and reduces implementation costs and burden, but it does not differentiate between type1 and type3, making it less suitable for detailed control and analysis. The following table is the detail analysis of two options. Considering future scalability and flexibility, selecting Option 1 is regarded as more appropriate.

| Item | Option 1: Introduce Four Codepoints | Option 3: Introduce Three Codepoints (no distinction between type1 and type3) |
| --- | --- | --- |
| Overview | Explicitly represent {type1, type2, type3, type4} with separate codepoints | Represent {type1_or_type3, type2, type4} with three codepoints (no distinction between type1 and type3) |
| Implementation Complexity | High: requires defining and managing four codepoints | Low: fewer codepoints, simplifying implementation |
| Representation Clarity | High: each beam type can be clearly distinguished | Low: type1 and type3 are not distinguished, so detailed information is lost |
| Network Operation & Management | Appropriate: allows detailed understanding and control of beam states | Simplified: no distinction, reducing operational burden and simplifying management |
| Necessity & Benefits | Useful when accurate differentiation of beam types is needed | Useful when the distinction is not important or for simplifying operations |
| Discussion Outlook | Easier to gain consensus among companies and offers higher future scalability | Cost-effective and quick to implement, suitable for rapid deployment |

Proposal 1: Confirm Working assumption Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} for detailed beam management and future scalability.

2.2 Support for mTRP in serving cells for LTM event-triggered reporting

To address diverse measurement needs: In 5G NR networks, beamforming and multi-viewpoint measurements using multiple TRPs are essential. Supporting mTRP enables the network to obtain measurement results from different TRPs simultaneously, thereby promoting improved interference management and enhanced beam control accuracy.

The efficiency of event-triggered reports: Since LTM event-triggered reports are designed to report measurement results based on specific conditions or events, enabling flexible application of mTRP to the serving cells and beams allows for more real-time and accurate understanding of the network conditions.

Proposal 2: Support mTRP in serving cells for LTM event-triggered reports.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: Confirm Working assumption Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} for detailed beam management and future scalability.

Proposal 2: Support mTRP in serving cells for LTM event-triggered reports.

- References

[1] RP-250339 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #129bis Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)