3GPP TSG-RAN WG2 Meeting #130	R2-2504424

St. Julians, Malta, May. 19th – 23rd, 2025

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Remaining open issues on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#129-bis meeting, agreements on conditional LTM design have been made as the followings [1]:

Agreements on Conditional LTM - Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer. - RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. - For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

In this paper, we discuss remaining open issues regarding conditional intra-CU LTM.

- Discussion

  - Co-existence between network-triggered LTM and CLTM

Regarding the co-existence between network-triggered LTM and CLTM, the following agreement was reached at RAN2#129 meeting.

- The LTM-candidate IE is re-used to model an CLTM candidate cell configuration. - For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

However, there are still unclear aspects related to the coexistence. For instance, it is not yet clearly determined whether a single candidate LTM configuration can be applied to both LTM types, or whether the network can trigger the UE to perform cell switch to a CLTM candidate cell.

Network-triggered LTM and CLTM share many similarities in terms of functionality and procedures. The key difference is that in network-triggered LTM, the network determines the target cell based on the measurement results reported by the UE, whereas in CLTM, the UE determines the target cell based on the execution conditions configured by the serving cell.

Therefore, network-triggered LTM candidate cells and CLTM candidate cells do not need to be considered as separate sets; rather, CLTM candidate cells can be regarded as a subset of network-triggered LTM candidate cells. In other words, a CLTM candidate cell can simultaneously serve as both a CLTM candidate and a network-triggered LTM candidate. In this case, the network may request the UE to report measurement results for CLTM candidate cells through measurement report configuration and may also request UE to perform a cell switch to a CLTM candidate cell based on the measurement reports.

Proposal 1: The network can trigger the UE to perform cell switch to a CLTM candidate cell by transmitting an LTM cell switch command MAC CE.

  - CLTM TAT Maintenance

Although there have been many discussions on CLTM TAT operation, there has been no discussion on how the TA should be reacquired before or after the expiration of the CLTM TAT. Some companies seem to assume that the CLTM TAT is maintained by both the UE and the source cell. However, based on the contributions submitted at the previous meeting, it seems that not all companies share a common view on this issue. To reduce unnecessary confusion, it needs to clearly define whether the source cell maintain the CLTM TAT. If the CLTM TAT is maintained by only the UE, then the UE should send the source cell a request for TA reacquisition. If the CLTM TAT is maintained by the source cell as well, then the source cell can trigger the PDCCH-ordered early RACH procedure for TA reacquisition without such overhead. Moreover, the corresponding TA can be used for RACH-less LTM cell switch when the source cell requests the UE to perform LTM cell switch to the CLTM candidate cell for which CLTM TAT is running as proposed in 2.1.

Proposal 2: The source cell maintains the CLTM TAT for a candidate cell TA and may re-trigger the PDCCH-ordered early RACH procedure before the expiration of the CLTM TAT.

  - RACH-less cell switch

If the previous proposals are adopted, a scenario may be considered where the UE receives an (Enhanced) LTM CSC MAC CE requesting a cell switch to a CLTM candidate cell while the CLTM TAT for that candidate cell is running. If the LTM CSC MAC CE includes a valid TA, the UE can apply it and perform a RACH-less LTM cell switch. However, if no valid TA is provided, the UE should determine whether to perform a RACH-less or a RACH-based LTM cell switch.

Although the target cell is defined as CLTM candidate cell and CLTM TAT for that target cell is running, if the UE receives an (Enhanced) LTM CSC MAC CE requesting a cell switch to a CLTM candidate cell, the UE shall consider this cell switch as a network-triggered LTM cell switch, not CLTM cell switch. In network-triggered LTM, after transmitting the LTM CSC MAC CE containing a valid TA, the source cell requests the target cell to allocate UL resources for a RACH-less switch. If the source cell does not have a valid TA, then it does not request the target cell to allocate UL resources for a RACH-less switch. Therefore, the UE performs RACH-less LTM cell should perform a RACH-based cell switch if a valid TA is not included in the received (Enhanced) LTM CSC MAC CE although the CLTM TAT for the target candidate cell is running.

Although the target cell is defined as a CLTM candidate cell and the CLTM TAT for that candidate cell is running, if the UE receives an (Enhanced) LTM CSC MAC CE requesting a cell switch to that candidate cell, the UE shall treat this as a network-triggered LTM cell switch, not a CLTM cell switch. In network-triggered LTM, after transmitting the (Enhanced) LTM CSC MAC CE containing a valid TA, the source cell requests the target cell to allocate UL resources for a RACH-less switch. If the source cell does not have a valid TA, it does not request UL resource allocation for a RACH-less switch. Therefore, the UE should perform a RACH-based LTM cell switch if a valid TA is not included in the received (Enhanced) LTM CSC MAC CE, even if the CLTM TAT for the target candidate cell is running.

Proposal 3: When the UE receives an (Enhanced) LTM CSC MAC CE requesting a cell switch to a CLTM candidate cell for which the CLTM TAT is running, it performs a RACH-less LTM switch only if the (Enhanced) LTM CSC MAC CE contains a valid Timing Advance Command. Otherwise, the UE performs a RACH-based LTM switch.

- Conclusions

In this paper, we discuss CLTM related issues. Following are proposals made in this paper:

Proposal 1: The network can trigger the UE to perform cell switch to a CLTM candidate cell by transmitting an LTM cell switch command MAC CE.

Proposal 2: The Enhanced LTM Cell Switch MAC CE always contains a NCC value field

Proposal 3: When the UE receives an (Enhanced) LTM CSC MAC CE requesting a cell switch to a CLTM candidate cell for which the CLTM TAT is running, it performs a RACH-less LTM switch only if the (Enhanced) LTM CSC MAC CE contains a valid Timing Advance Command. Otherwise, the UE performs a RACH-based LTM switch.

- References

- Chair notes RAN2#129-bis

- Running MAC CR for Mob Ph4