3GPP TSG-RAN WG2 Meeting #130	R2-2504434

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, we agreed that:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In this paper, we provide our view for the remaining issues.

Discussion

QoS Flow Identification

If a new MAC CE is introduced to indicate QoS flow level bit rate signalling, how to identify the QoS flow should be discussed first. For this purpose, considering that QoS Flow ID is unique within a certain PDU session/DRB/Logical Channel, we can consider the following three alternatives for this purpose:

- Alt. 1: PDU session ID + QoS Flow ID;

- Alt. 2: DRB ID + QoS Flow ID;

- Alt. 3: LCID + QoS Flow ID.

Among the three options, since the MAC CE is processed by MAC entity and then the bit rate indication is provided to upper layers, it is preferable to use LCID, which is visible for MAC entity, as part of the QoS flow indication, i.e., Alt. 3, to ease the MAC processing.

Proposal 1: RAN2 to use [LCID + QoS Flow ID] to signal/identify a targeted QoS flow in the new MAC CE for UL rate control.

Multi-QoS Flow Extension

Since an XR service normally entails multiple QoS flows, and among the multiple QoS flows, it is likely that more than one flow are supposed to be throttled simultaneously, when congestion happens. With this in mind, we see the benefit by featuring the new MAC CE with multi-QoS flow support, to reduce the signalling overhead (otherwise multiple MAC CEs are needed) and the associated latency.

Proposal 2: The new MAC CE is designed to support the signalling for multiple QoS flows.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 to use [LCID + QoS Flow ID] to signal/identify a targeted QoS flow in the new MAC CE for UL rate control.

Proposal 2: The new MAC CE is designed to support the signalling for multiple QoS flows.