# 3GPP TSG-WG2 Meeting #130                                            R2-2504440

St.Julians, Malta, May 19th – 23rd, 2025

Source: 			CMCC

Title:  		    Discussion on LCM for positioning

Agenda item:	8.1.2.3

Document for: 	Discussion

Introduction

With the past RAN2 meetings, RAN2 discussed general procedures for LCM for UE sided model for AI positioning case 1. The following agreements have been reached.

#129 - Introduce AI/ML positioning Case 1 as a new positioning method. 2: Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR. 3: FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation. 4: The content of error cause is discussed while drafting stage3 CRs. 5: As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list. 6: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed. #129bis - LMF is responsible for functionality management - UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. - Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation". - An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. - For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

In the contribution, we will discuss the remaining controversial issues.

Discussion

Performance monitor

For model performance monitoring of AI/ML positioning Case 1, at least the UE side performs monitoring calculation is supported, the LMF side performs monitoring is still FFS until RAN1#120bis. For model performance monitoring of AI/ML positioning Case 1, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. The target UE may signal the monitoring outcome to the LMF.

In RAN1 #120bis, it has been agreed that the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.” From RAN2’ perspective, we should confirm to support the UE side performs monitoring for AI/ML positioning Case 1 and discuss the signaling to support it. For the label data of location is generated by LMF can be transmitted by the LPP ProvideAssistanceData message. The monitoring outcome can be transmitted by the UE via LPP ProvideLocationInformation message, including at least an indication that the target UE cannot perform the Case 1 positioning method.

Proposal 1: RAN2 is suggested to support the UE side performs monitoring for AI/ML positioning Case 1.

Proposal 2: The label data (e.g., label and quality indicator of label) of location generated by LMF can be transmitted by the LPP ProvideAssistanceData message.

Proposal 3: The monitoring outcome can be transmitted by the UE via LPP ProvideLocationInformation message, including at least an indication that the target UE cannot perform the Case 1 positioning method.

As per RAN1 progress, for both UE initiated and LMF initiated case, the DL PRS configuration and existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD are needed to be provided by LMF to UE.

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Conclusion

According to the above discussion, the following observations and proposals are given:

Performance monitoring

Proposal 1: RAN2 is suggested to support the UE side performs monitoring for AI/ML positioning Case 1.

Proposal 2: The label data (e.g., label and quality indicator of label) of location generated by LMF can be transmitted by the LPP ProvideAssistanceData message.

Proposal 3: The monitoring outcome can be transmitted by the UE via LPP ProvideLocationInformation message, including at least an indication that the target UE cannot perform the Case 1 positioning method.

Proposal 4: For data collection for UE sided POS model training, at least DL PRS configuration, existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD need to be provided by LMF to UE.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] R1-2501627 LS on AI/ML Posting case 3b, RAN1.