3GPP TSG-RAN WG2 Meeting #130	R2-2504441

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.2

Source:	CMCC

Title:	Discussion on functionality management for UE sided model

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting RAN2#129bis [1], RAN2 discussed LCM framework and required signaling and protocol for AI Mobility and achieved the following agreements.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.    FFS when the UE can perform inference

- Agreements

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

This contribution will focus on applicability procedure, inference configuration and reporting and functionality management for RRM measurement prediction with UE-sided model and for measurement event prediction with UE-sided model.

# 2 Discussion

2.1 General applicability procedure for UE sided model

For AI/ML for NR air interface, the legacy UE capability framework serves as the baseline to report UE’s supported AI/ML-enabled Feature/FG. RRCReconfiguration message is used to carry the full inference configuration and/or inference related info which are used for UE’s decision on applicability/inapplicability of functionality. And the initial applicability/inapplicability reporting is carried in RRCReconfigurationComplete message. When there are changes for applicability/inapplicability of functionality, the subsequent applicability/inapplicability reporting could be carried in UAI. The basic procedure for UE capability and applicability functionality reporting in AI/ML for NR air interface WI is shown in Fig 1.

Fig 1. Basic procedure for applicability functionality reporting

Proposal 1: The UE capability and applicable functionality reporting defined for AI/ML based beam management could be reused as the baseline for AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model. RAN2 to capture Fig 1 the signalling procedure in TR 38.744.

The legacy UE capability framework could also be reused as the baseline for AI Mobility. Therefore, for RRM measurement prediction and Measurement events prediction use cases, this information is indicated in UE AS capability in RRC (e.g., UECapabilityEnquiry/UECapabilityInformation).

According to the agreements achieved in last meeting [1], the spec impact of the related AI Mobility cases/subcases will be addressed first, and functionality granularity will be discussed later. Therefore, the further discussions on UE capability details (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be carried during the WI phase.

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

Proposal 2: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature and/or supported functionalities for RRM measurement prediction and measurement event prediction with UE-sided model.

Proposal 3: The further discussions on UE capability details (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be carried during the WI phase.

For AI/ML for NR air interface, in Step 3, RRCReconfiguration message is used to carry the full inference configuration and/or inference related info which are used for UE’s decision on applicability/inapplicability of functionality. Specifically, in AI/ML for NR air interface, RAN1 provides the following Option A) and Option B) in Step 3 (the configurations provided from NW to UE) for the applicability report. Since the configurations of Option A) and Option B) in Step 3 are different, the content of UE applicability reporting in Step 4 and the configurations in RRCReconfiguration message in Step 5 are also different for Option A) and Option B) accordingly.

- For Option A):

- In Step 3: One or more of the full inference configuration sent in CSI-ReportConfig;

- In Step 4: UE sends the initial applicability report in RRCReconfigurationComplete for each full inference configuration;

- In Step 5: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3;

- For Option B):

- In Step 3: One set or multiple sets of inference related parameters via OtherConfig for applicability report only (not for inference);

- In Step 4: UE reports applicability for B) set(s) of inference related parameters;

- In Step 5: NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration;

According to the progress of AI/ML for NR air interface WI, both Option A) and Option B) are agreed in RAN2. For AI for Mobility, RRCReconfiguration message in Step 3 could be used to carry the full inference configuration (Option A) and/or inference related info (Option B) of AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model.

Proposal 4: For AI for Mobility, RRCReconfiguration message could be used to carry the full inference configuration (Option A) and/or inference related info (Option B) of AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model.

According to the agreements achieved in last meeting [1], UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message for AI/ML for mobility. Specifically, in AI/ML for NR air interface, the initial applicability/inapplicability reporting of the functionality in Step 4 is carried in RRCReconfigurationComplete message. And UAI is used in Step 6 to carry the subsequent applicability/inapplicability reporting. Therefore, AI/ML for mobility could follow the principle of AI/ML for NR air interface.

Proposal 5: In applicability procedure of AI/ML for mobility, RRCReconfigurationComplete message could be used to carry the initial applicability/inapplicability reporting of the functionality.

Proposal 6: In applicability procedure of AI/ML for mobility, UAI could be used to carry the subsequent applicability/inapplicability reporting.

2.2 Inference configuration and reporting

According to the agreements achieved in last meeting [1], the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. And in the discussion in Section 2.1 of the contribution, the inference configuration could be carried in RRCReconfigurationComplete message in the applicability procedure. Therefore, Section 2.2 will focus on discussion on the content of the inference configuration and reporting.

Considering there are two study goals in AI for Mobility, including measurement overhead reduction and HO performance improvement, the content of inference configuration and reporting of these two study goals maybe different. For example, for measurement overhead reduction, at least the measurement reduction rate and model input and output related info should be considered in inference related info; for HO performance improvement, Prediction Window, model input and output related info should be considered in inference related info.

Observation 1: There are two study goals in AI for Mobility, including measurement overhead reduction and HO performance improvement, the content of inference configuration and reporting of these two study goals maybe different.

During the discussion in last meeting, RAN2 agreed that both cell level and L3 beam level measurement prediction will be studied in AI for Mobility. And the following sub-use cases should be considered for cell level and L3 beam level measurement prediction:

- For cell level measurement prediction:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results.

- For L3 beam level measurement prediction:

  - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results.

  - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results.

  - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results.

RRM measurement prediction with UE sided model

For RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered as follow:

- Temporal domain case A (2nd goal: HO performance improvement):

  - The length of Prediction Window

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - L1 sampling period: e.g., sampling period is 40ms@FR1, sampling period is 80ms@FR2, which could be used to specify the sampling period of actual RRM measurements and the measurement period of predicted RRM measurements for periodic reporting;

  - The threshold of RRM measurement results, which is used for event triggered inference and/or reporting;

  - Associated ID (FFS), which is introduced in AI/ML for Air Interface, and indicates the network-side additional condition

- Temporal domain case B (1st goal: measurement overhead reduction):

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - MRRT related info: e.g., a threshold of MRRT, the MRRT actually used by UE could not exceed the threshold;

  - L1 sampling period

  - Associated ID (FFS)

- Frequency domain (1st goal: measurement overhead reduction),

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - Frequency related info: e.g., the measured frequency and the predicted frequency;

  - L1 sampling period

  - Associated ID (FFS)

Proposal 7: For Temporal domain case A in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - L1 sampling period: e.g., sampling period is 40ms@FR1, sampling period is 80ms@FR2, which could be used to specify the sampling period of actual RRM measurements and the measurement period of predicted RRM measurements for periodic reporting;

  - The threshold of RRM measurement results, which is used for event triggered inference and/or reporting;

  - Associated ID (FFS), which is introduced in AI/ML for Air Interface, and indicates the network-side additional condition

Proposal 8: For Temporal domain case B in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - MRRT related info: e.g., a threshold of MRRT, the MRRT actually used by UE could not exceed the threshold;

  - L1 sampling period

  - Associated ID (FFS)

Proposal 9: For Frequency domain in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - Frequency related info: e.g., the measured frequency and the predicted frequency;

  - L1 sampling period

  - Associated ID (FFS)

In the evaluation for RRM measurement prediction, the following agreements were achieved for Temporal domain case A [2]. RAN2 will not define the threshold of OW length, even though too long the OW could not yield significant benefits.

Agreements

Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold, further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.  RAN2 will not define the actual threshold and fast fading assumption.

Proposal 10: The length of Observation Window is not needed in inference related info from NW to UE, which can be up to UE implementation.

Proposal 11: L1 sampling period needs to be considered in inference related info from NW to UE, which could be used to specify the sampling period of actual RRM measurements and the measurement period of predicted RRM measurements for periodic reporting.

Proposal 12: The threshold of RRM measurement results needs to be considered in inference related info from NW to UE, which is used for event triggered inference and/or reporting.

Measurement event prediction

For measurement event prediction, the following content of inference related info could be further discussed:

- For indirect prediction:

  - The length of Prediction Window

  - L1 sampling period

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

- For direct prediction:

  - The length of Prediction Window

  - Probability threshold related info, e.g., the minimum value of the threshold which is used to determine the event prediction;

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

Proposal 13: For indirect prediction in measurement event prediction, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - L1 sampling period

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

Proposal 14: For direct prediction in measurement event prediction, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - Probability threshold related info, e.g., the minimum value of the threshold which is used to determine the event prediction;

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

2.3 Functionality management

In AI/ML for NR air interface, Functionality management refers to activation/deactivation/fallback/switching of AI/ML functionality, which could be indicated via 3GPP signalling (e.g., RRC, MAC-CE, DCI) for functionality-based LCM. Based on the description in TR 38.843[3], we could observe the management decision could be taken by the network or by the UE.

Specifically, there are two options for functionality/model management in AI/ML for NR air interface.

- Option 1: Decision by the network

1a) Network-initiated

1b) UE-initiated and requested to the network

- Option 2：Decision by the UE

2a) Event-triggered as configured by the network, UE’s decision is reported to the network

2b) UE-autonomous, UE’s decision is reported to the network

2c) UE-autonomous, UE’s decision is not reported to the network

According to the progress AI/ML for NR air interface WI, we could observe that Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Observation 2: There are two options for functionality/model management in AI/ML for NR air interface. According to the progress AI/ML for NR air interface WI, Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management [3]           Figure 7.2.1.1-2: Network decision, UE-initiated AI/ML management [3]

Figure 7.2.1.1-3: UE decision, event-triggered as configured by the network [3]              Figure 7.2.1.1-4: UE autonomous, decision reported to the network [3]

Proposal 15: Option 1a (Network decision, network-initiated AI/ML management) could be considered as the starting point for the study of functionality management for AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model.

# 3 Conclusion

In this paper, we provide our consideration on applicability procedure, inference configuration and reporting and functionality management for RRM measurement prediction with UE-sided model and for measurement event prediction with UE-sided model. We have the following observation and proposals:

General applicability procedure for UE sided model

Proposal 1: The UE capability and applicable functionality reporting defined for AI/ML based beam management could be reused as the baseline for AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model. RAN2 to capture Fig 1 the signalling procedure in TR 38.744.

Proposal 2: UE capability framework (i.e. UECapabilityEnquiry/UECapabilityInformation) could be used to report UE’s supported AI/ML-enabled Feature and/or supported functionalities for RRM measurement prediction and measurement event prediction with UE-sided model.

Proposal 3: The further discussions on UE capability details (e.g., functionality granularity, content, structure of the related UE capabilities, etc.) could be carried during the WI phase.

Proposal 4: For AI for Mobility, RRCReconfiguration message could be used to carry the full inference configuration (Option A) and/or inference related info (Option B) of AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model.

Proposal 5: In applicability procedure of AI/ML for mobility, RRCReconfigurationComplete message could be used to carry the initial applicability/inapplicability reporting of the functionality.

Proposal 6: In applicability procedure of AI/ML for mobility, UAI could be used to carry the subsequent applicability/inapplicability reporting.

Inference configuration and reporting

Observation 1: There are two study goals in AI for Mobility, including measurement overhead reduction and HO performance improvement, the content of inference configuration and reporting of these two study goals maybe different.

Proposal 7: For Temporal domain case A in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - L1 sampling period: e.g., sampling period is 40ms@FR1, sampling period is 80ms@FR2, which could be used to specify the sampling period of actual RRM measurements and the measurement period of predicted RRM measurements for periodic reporting;

  - The threshold of RRM measurement results, which is used for event triggered inference and/or reporting;

  - Associated ID (FFS), which is introduced in AI/ML for Air Interface, and indicates the network-side additional condition

Proposal 8: For Temporal domain case B in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - MRRT related info: e.g., a threshold of MRRT, the MRRT actually used by UE could not exceed the threshold;

  - L1 sampling period

  - Associated ID (FFS)

Proposal 9: For Frequency domain in RRM measurement prediction with UE sided model, the content of inference related info from NW to UE could be considered:

  - Inference output related info: e.g., predicted L3 cell level results or predicted L3 filtered beam level results;

  - Inference input related info: e.g., L1 beam level measurement results or L3 cell level measurement results;

  - Frequency related info: e.g., the measured frequency and the predicted frequency;

  - L1 sampling period

  - Associated ID (FFS)

Proposal 10: The length of Observation Window is not needed in inference related info from NW to UE, which can be up to UE implementation.

Proposal 11: L1 sampling period needs to be considered in inference related info from NW to UE, which could be used to specify the sampling period of actual RRM measurements and the measurement period of predicted RRM measurements for periodic reporting.

Proposal 12: The threshold of RRM measurement results needs to be considered in inference related info from NW to UE, which is used for event triggered inference and/or reporting.

Proposal 13: For indirect prediction in measurement event prediction, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - L1 sampling period

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

Proposal 14: For direct prediction in measurement event prediction, the content of inference related info from NW to UE could be considered:

  - The length of Prediction Window

  - Probability threshold related info, e.g., the minimum value of the threshold which is used to determine the event prediction;

  - Inference output related info, e.g., predicted A1-A6 events;

  - Associated ID (FFS)

Functionality management

Observation 2: There are two options for functionality/model management in AI/ML for NR air interface. According to the progress AI/ML for NR air interface WI, Option 1a is taken as baseline, Option 1b and 2b would be further discussed, and Option 2c is not considered in Rel-19.

Proposal 15: Option 1a (Network decision, network-initiated AI/ML management) could be considered as the starting point for the study of functionality management for AI/ML based RRM measurement prediction and AI/ML based measurement event prediction with UE-sided model.

# Reference

- Draft_RAN2_129bis_Meeting_Report_v2, Apr 2025.

- Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

- 3GPP TR 38.843: " Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".