3GPP TSG-RAN WG2 Meeting #130		R2-2504445

St Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining Issues of L1 Event Triggered Measurement Report

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

Based on agreements RAN2 made, event-triggered L1 measurement enhancements for LTM has implemented in the MAC/RRC running CRs [1][2].

In addition, RAN1 sent LS (R1-2501500) [3] on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells to RAN2. This LS include following RAN1 agreement that the semi-persistent CSI-RS resource is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting.

Agreement - Confirm the following working assumption made in RAN1#118bis Working Assumption(RAN1#118bis) In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

For this issue, RAN2 only made following agreements in RAN2#129bis meeting.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

In this contribution, we will further address the above issues on the event triggered L1 measurement reporting and support of semi-persistent CSI-RS activation/deactivation for LTM.

Discussion

Semi-persistent CSI-RS resource activation/deactivation MAC CE

In Rel-19, CSI-RS resources are common measurement for all kind of LTM features, so we think RAN2 first need to make a high-level agreement that this CSI-RS resources including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE and it is used for evaluation of conditional LTM. We think it is not big deal to support SP CSI-RS resource could be associated with any LTM MR signalling i.e. no impact on the current running CRs, only some association between CSI resource configuration and report configuration need to be checked. We agree that association of this SP CSI-RS activation with the conditional execution condition for LTM candidate cells seems not a realistic scenario but it could be supported any specification changes.

Proposal 1: RAN2 confirm that CSI-RS resources for LTM including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE, and this resources are used for evaluation of the conditional LTM.

According to the RAN1 LS [3], RAN2 needs to discuss how to support the measurement procedure for semi-persistent CSI-RS resource which could be activated/deactivated by source cell. For this purpose, we think following RAN2 need to conclude how to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM).

For the first issue, we think all Rel-19 LTM features are supported after LTM cell switch (i.e. subsequent LTM) without additional RRC reconfiguration. CSI-RS resource measurement for LTM is supported in Rel-19 and we think it should be supported after LTM cell switch, this measurement of semi-persistent CSI-RS resource are also kind of this CSI-RS resource measurement for LTM. Then, we think it is general for UE to measure the activated semi-persistent CSI-RS resources after LTM cell switch unless NW explicitly signal the deactivation of semi-persistent CSI-RS resources.

However, the legacy semi-persistent CSI-RS resources are only measured in the source cell when it is activated i.e. if semi-persistent CSI-RS resources are activated and the HO is triggered by gNB then UE should stop to measure the activated semi-persistent CSI-RS resources.

In order to support the CSI resource measurement for subsequent LTM, the CSI-RS resource configuration for LTM should be common for all candidate cells, then UE could continuously monitor/measure the CSI-RS resources after LTM cell switch.

For example, following scenario could be considered for this issue:

- Step 1: Semi-persistent CSI-RS resources in candidate cell #1/#2/#3 are activated by the source cell and UE measure/report the activated semi-persistent CSI-RS resources results.

- Step 2: Based on MR, network triggers the LTM cell switch to candidate cell #1.

- Step 3: After cell switch, UE could measure these semi-persistent CSI-RS resources because there was no explicit MAC CE for deactivation i.e. UE consider these resources as activated.

We also think it is also possible that these semi-persistent CSI-RS resources are considered as deactivated when the LTM cell switch is executed, and it could be simple in terms of UE procedure perspective. We summarize two options with some observation for each option.

- Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

  - Assume the activated SP CSI-RS resources are not valid after cell switch

  - We think this requires the RAN3 impact for both F1/Xn signaling for all candidate cells to indicate the deactivation of the all SP CSI-RS resources (i.e. because of cell switch)

  - NW could provide the semi-persistent CSI-RS activation MAC CE for activation after cell switch based on further signaling between the source cell and the LTM candidate cells

  - Need some rule or signaling  in both RAN2/3 specifications that the activated semi-persistent CSI-RS resources are considered as deactivated upon LTM cell switch

    - The success of LTM cell switch indication is provided to all candidate cells in order that candidate cell could provide the new state of semi-persistent CSI-RS resources again after cell switch. Some new indication could be added in F1/Xn message.

- Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

  - Each candidate cell has provided the state of semi-persistent CSI-RS resources which are associated with the candidate cell, source cell could know the state of these resources

    - In order to support subsequent LTM for this semi-persistent CSI measurement, this state information of semi-persistent CSI-RS resources for each candidate cell could be shared to all candidate cells, then target cell could just associate the activated semi-persistent CSI resource with the report configuration.

    - It means the state of semi-persistent CSI resources should be shared all candidate cells not only for the source cell. (this requires RAN3 impact both F1/Xn signaling)

    - However, it makes the signaling complicate to support.

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

    - If companies want to handle the activated SP resources for target cell and other candidate cells different, this kind of approach could be considered as well.

From our understanding, anyway RAN3 impact is expected to handle the SP CSI-RS resource activation/deactivation after LTM cell switch regardless of RAN2 to determine either stop or continue the measuring of this resources. Then, we think it would be better to support more feature (i.e. subsequent LTM measurement), so we prefer the Option 2 above.

Proposal 2: UE continues to measure the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM).

Note that RAN3 already introduced the Xn and F1 signaling for Rel19 in [4][5] which includes the CSI-RS coordination so if RAN2 make the decision on this feature, then RAN2 need to send LS to RAN3 to request the change of F1/Xn signaling.

Proposal 3: RAN2 send LS to RAN3 for the issue of handling the activated SP CSI-RS resources after LTM cell switch.

Regarding the RRC configuration of the semi-persistent CSI-RS resources, the current CSI-RS resource configuration is only configured in the candidate configuration (i.e. LTM-Candidate) and how to associate between the measurement CSI-RS resources and candidate cells in LTM-CSI-ResourceConfig is put as FFS. From our understanding, this CSI-RS resource configuration including semi-persistent CSI resources should be common and the CSI-RS resource configuration should include all resources in candidate cells. Therefore, it should be visible in LTM-CSI-ResourceConfig, the detail signaling should be further determined based on RAN1/2 discussion.

In short, actual resource configuration is per candidate given by ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 and ltm-NZP-CSI-RS-ResourceToAddModList-r19 in LTM-Candidate-r18. In addition we need follow singlaing structure for CSI-RS resource configurations for Rel-19 LTM.

LTM-CSI-RS-ResourceSet-r19  NZP-CSI-RS-ResourceId in LTM-CSI-ResourceConfig-r18 refers to ltm-NZP-CSI-RS-ResourceToAddModList-r19.

Proposal 4: CSI-RS resource configuration for LTM including semi-persistent CSI resources should be common, i.e. semi-persistent CSI resources for all candidate cells should be configured in LTM-CSI-ResourceConfig.

Truncated L1 MR MAC CE details

RAN2 made a lot of agreements to support (Truncated) L1 MR MAC CE, but the details are not specified yet, e.g.: (below issues have added in the MAC open issue list)

- Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE?

- In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE?

From our understanding, the ideal design of truncated event-triggered L1 MR MAC CE would be the same of the non-truncated, the event-triggered L1 MR MAC CE format. However, current rule to add the beam contents for the truncated event-triggered L1 MR MAC CE is somewhat different with the non-truncated, the event-triggered L1 MR MAC CE i.e. the triggered beam should be prioritized than other beams.

For easy implementation and easy description in specification, the generalized MAC CE format and rule between the event-triggered L1 MR MAC CE and the truncated event-triggered L1 MR MAC CE is required.

So, we want to propose following event-triggered L1 MR MAC CE format with description:

- Introduce the two group of beams (i.e. triggered beam and non-triggered beam), so there would be two reference beam and RSRP based on the best beam quality.

  - Group of triggered beam: Type 1 and Type 2

  - Group of non-triggered beam: Type 3 and Type 4

- Priority of Types (Type 1 and Type 2 -> Type 3 -> Type 4)

- Based on the above rule, both the event-triggered L1 MR MAC CE and the truncated event-triggered L1 MR MAC CE added the contents

- If the truncated event-triggered L1 MR MAC CE is needed, just truncate the MR MAC CE based on the size i.e. no need to re-formulate the MAC CE and contents.

Proposal 5: For the generalized rule to create the (truncated) event-triggered L1 MR MAC CE, apply the below rules:

- Introduce the two group of beams (i.e. triggered beam and non-triggered beam), so there would be two reference beam and RSRP based on the best beam quality.

  - Group of triggered beam: Type 1 and Type 2

  - Group of non-triggered beam: Type 3 and Type 4

- Priority of Types (Type 1 and Type 2 -> Type 3 -> Type 4)

- Based on the above rule, both the event-triggered L1 MR MAC CE and the truncated event-triggered L1 MR MAC CE added the contents

- If the truncated event-triggered L1 MR MAC CE is needed, just truncate the MR MAC CE based on the size i.e. no need to re-formulate the MAC CE and contents.

For the event-triggered L1 MR procedure, if UE has at least one beam to satisfy the event condition UE triggers the event-triggered L1 MR procedure. It means if there are enough UL grant to include the event-triggered L1 MR MAC CE, UE generate the event-triggered L1 MR MAC CE and include it to the UL grant. Otherwise, UE request the UL grant by dedicated SR resource. Based on this SR, source cell provides the UL grant for the event-triggered L1 MR MAC CE but the source cell has no idea the size of this MAC CE.

We think it should be confirmed that UE generates the (truncated) event-triggered L1 MR MAC CE when UE receives the UL grant in case at least one triggered beam is exist.

In addition, we think UE continues to evaluate the event condition for the L1 MR event after UE request SR to the source cell, and this evaluation should be continued at least before receiving the UL grant. It means UE should evaluate the L1 event MR procedure even UE send the truncated event-triggered L1 MR MAC CE to the source cell because the L1 MR procedure is not completed in this case. UE should include all latest beam information when UE receives the UL grant.

Proposal 6: UE continues to evaluate the event condition for the L1 MR event after UE request SR to the source cell and the L1 MR procedure is completed i.e. L1 event evaluation should be continued during L1 MR procedure.

Proposal 7: UE generates the (truncated) event-triggered L1 MR MAC CE when UE receives the UL grant, and include the latest beam’s measurement result at this time.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: CSI-RS resources for LTM including semi-persistent CSI-RS resources are reported by UCI, event-triggered L1 measurement report MAC CE, and this resources are used for evaluation of the conditional LTM.

Proposal 2: UE continues to measure the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM).

Proposal 3: RAN2 send LS to RAN3 for the issue of handling the activated SP CSI-RS resources after LTM cell switch.

Proposal 4: CSI-RS resource configuration for LTM including semi-persistent CSI resources should be common, i.e. semi-persistent CSI resources for all candidate cells should be configured in LTM-CSI-ResourceConfig.

Proposal 5: For the generalized rule to create the (truncated) event-triggered L1 MR MAC CE, apply the below rules:

- Introduce the two group of beams (i.e. triggered beam and non-triggered beam), so there would be two reference beam and RSRP based on the best beam quality.

  - Group of triggered beam: Type 1 and Type 2

  - Group of non-triggered beam: Type 3 and Type 4

- Priority of Types (Type 1 and Type 2 -> Type 3 -> Type 4)

- Based on the above rule, both the event-triggered L1 MR MAC CE and the truncated event-triggered L1 MR MAC CE added the contents

- If the truncated event-triggered L1 MR MAC CE is needed, just truncate the MR MAC CE based on the size i.e. no need to re-formulate the MAC CE and contents.

Proposal 6: UE continues to evaluate the event condition for the L1 MR event after UE request SR to the source cell and the L1 MR procedure is completed i.e. L1 event evaluation should be continued during L1 MR procedure.

Proposal 7: UE generates the (truncated) event-triggered L1 MR MAC CE when UE receives the UL grant, and include the latest beam’s measurement result at this time.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Introduction of event-triggered measurement reporting for RRC spec	Huawei	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[3]	R1- 2501500	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells	Rel-19	NR_Mob_Ph4-Core, 	Source: RAN1	To: RAN2	CC: RAN3.

[4]	R3-252520	(BL CR to 38.423) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm	38.423 NR_Mob_Ph4-Core Rel-19.

[5]	R3-252520	BL CR to 38.473) Xn support for inter-CU LTM	Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm	38.473 NR_Mob_Ph4-Core Rel-19.