3GPP TSG-RAN WG2 Meeting #130		R2-2504446

St Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.6.4

Source:	Samsung

Title:	Remaining issues of Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In this contribution, we will further address remaining issues to support the conditional intra-CU LTM in detail.

Discussion

Issue 1: RRC issues

As indicated by WID, the C-LTM is limited to the non-DC case. Thus, the C-LTM for SCG is not supported in Rel-19.

Proposal 1: RAN2 is kindly asked to confirm that C-LTM for SCG is not supported in Rel-19.

Issue 2: RACH-less C-LTM for the cells in the same TAG

In CLTM, the UE can derive the TA value of one candidate cell (e.g., Cell A) based on the LTM candidate Timing Advance Command MAC CE. The RACH-less cell switch toward Cell A can be performed as long as the TAT for such cell is still running when the CLTM execution is triggered. However, for other cells belonging to the same TAG as Cell A (e.g., Cell B/C…), the UE cannot perform RACH-less cell switch. The reason is that the UE cannot know that Cell B/C is in the same TAG as Cell A.

Actually, if the UE can know the TAG information of each candidate cell, it has more chance to perform the RACH-less CLTM, i.e., it can perform RACH-less CLTM for the cells in the same TAG as long as the TA value of one cell is derived.

This is different from Rel18 design. In Rel18, the UE performs RACH-less cell switch when receiving the TA value via the cell switch command MAC CE. Thus, it needn’t to reuse the received TA value for other cells. However, in Rel19 CLTM, the UE determines the cell switch by itself even the TA value is not derived. If the UE can know more applicable cells for one received TA value, it can perform RACH-less cell switch to more cell, and speed up the cell switch procedure.

Proposal 2: RAN2 is kindly asked to discuss whether to support the RACH-less CLTM for the cells in the same TAG.

If the above proposal is agreeable, the gNB can inform the TAG information of each candidate cell to the UE when configuring the candidate cells. Moreover, such TAG information should be uniquely assigned for each candidate cell.

Issue 3: Criteria to determine that RACH-less C-LTM is ongoing upon C-LTM execution condition is met

Text in Current CR

2>	if the ltm-Candidate-TimeAlignmentTimer associated with the Target Configuration ID is running in the first available CG occasion for initial CG transmission according to clause 5.8.2 in case two TAGs are not configured for the CLTM candidate cell:

3>	process the stored Timing Advance Command associated with the ltm-Candidate-TimeAlignmentTimer (see clause 5.2);

3>	consider the RACH-less CLTM cell switch to be ongoing;

2>	if the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the Target Configuration ID for the TAG associated with the selected SSB or selected CSI-RS is running in the first available CG occasion for initial CG transmission according to clause 5.8.2 in case in case two TAGs are configured for the CLTM candidate cell:

3>	process the stored Timing Advance Command associated with the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (see clause 5.2);

3>	consider the RACH-less CLTM cell switch to be ongoing;

2>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the LTM target configuration in the first available CG occasion for initial CG transmission according to clause 5.8.2:

3>	process the measured Timing Advance (see clause 5.2);

3>	consider the RACH-less CLTM cell switch to be ongoing;

Current text (highlighted in yellow) considers that RACH-less CLTM cell switch to be ongoing if UE has valid TA. However, RACH-less CLTM is supported only based on CG. If CG resources are not configured for RACH less LTM, there is no question of performing or determining whether to perform RACH less or not. So, ‘RACH-less CLTM cell switch can be considered ongoing’, only if UE has valid TA and CG resources for LTM cell switch is configured.

Proposal 3: ‘RACH-less CLTM cell switch can be considered ongoing’, only if UE has valid TA and CG resources for LTM cell switch is configured.

It is not clear why the text highlighted in ‘green’ is needed. Even with this condition, candidate TAT timer can expire during C-LTM execution. So, this additional check is not necessary in our view. Also, there is no RAN2 agreement to introduce this check and such check is not there in R18 LTM. RAN2 agreement is to check TA validity when the C-LTM execution condition is met. So, suggest to remove the condition ‘in green’.

Proposal 4: Remove ‘in the first available CG occasion for initial CG transmission according to clause 5.8.2’ in the condition to determine RACH less C-LTM.

Issue 4: Handling expiry of ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 during RACH less C-LTM execution.

ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 based on which TA is applied during the RACH less C-LTM execution may expire while RACH less C-LTM execution is ongoing. In this case, after expiry of this timer, UE can not use CG resource. As a result C-LTM execution cannot be completed. The simplest approach would be to allow the UE to fallback to RACH based C-LTM.

Proposal 5: if ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (based on which TA is applied during the RACH less C-LTM execution) expires during the RACH less C-LTM execution, UE falls back to RACH based C-LTM. UE stops using the CG resources for RACH less LTM.

Issue 5: ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 handlingltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG2 for the LTM candidate cell (say Cell A) is running.

Case 1: UE receives new LTM configuration which does not include Cell A’s LTM candidate configuration.

Case 2: UE receives new LTM configuration which include Cell A’s LTM candidate configuration without conditional LTM execution condition.

In above cases, it is reasonable to stop the running ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2. These timers are associated with candidate configuration index and in the new configuration candidate configuration index may point to a different LTM configuration of different candidate.

Proposal 6: UE stops the ltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG2 for a LTM candidate cell A when the LTM candidate cell A configuration is removed in the latest received RRCReconfiguration message.

Issue 6: Removal of L3 Measurement configuration for CLTM

While executing ReconfigurationWithSync, UE autonomously removes any report configuration with reportType as condTriggerConfig and the related measurement objects and measurement identities as highlighted below, except for SCPAC.

- 5.3.5.3          Reception of an RRCReconfiguration by the UE

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2> if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2> if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA, CPC, or subsequent CPAC was configured:

3> remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

3> remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3> for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4> if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5> remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4> if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4> if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5> remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4> remove the entry with the matching measId from the measIdList within the VarMeasConfig;

CLTM can be based on L3 measurements also and subsequent CLTM is supported. So the UE shouldn’t autonomously remove the corresponding measurement configurations while executing ReconfigurationWithSync.ReconfigurationWithSync for this case can be legacy L3 handover, CHO,LTM, CLTM etc. For e.g. UE currently keeps the CLTM configuration while executing a legacy L3 handover. So it is important that the UE keeps associated L3 measurement configuration also and doesn’t autonomously release it. Such measurement configurations can be released by the network, for e.g. when the network releases the CLTM configuration.

To avoid the autonomous release, new conditions need to be added as below

2> if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2> if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA, CPC, or subsequent CPAC was configured:

3> remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

3> remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3> for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4> if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included or;

4> if the reportConfigId is not associated with any measId within the MCG VarMeasConfig which is indicated in a CLTM-ExecutionConditions IE associated with the MCG:

5> remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4> if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4> if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included; or

4> if the measObjectId is not associated with any measId within the MCG VarMeasConfig which is indicated in a CLTM-ExecutionConditions IE associated with the MCG:

5> remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4> remove the entry with the matching measId from the measIdList within the VarMeasConfig;

Proposal 7: Add additional conditions to check whether the reportConfig/measObject/measId is used for CLTM before removing those configurations while executing ReconfigurationWithSync.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 is kindly asked to confirm that C-LTM for SCG is not supported in Rel-19.

Proposal 2: RAN2 is kindly asked to discuss whether to support the RACH-less CLTM for the cells in the same TAG.

Proposal 3: ‘RACH-less CLTM cell switch can be considered ongoing’, only if UE has valid TA and CG resources for LTM cell switch is configured.

Proposal 4: Remove ‘in the first available CG occasion for initial CG transmission according to clause 5.8.2’ in the condition to determine RACH less C-LTM.

Proposal 5: if ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (based on which TA is applied during the RACH less C-LTM execution) expires during the RACH less C-LTM execution, UE falls back to RACH based C-LTM. UE stops using the CG resources for RACH less LTM.

Proposal 6: UE stops the ltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG2 for a LTM candidate cell A when the LTM candidate cell A configuration is removed in the latest received RRCReconfiguration message.

Proposal 7: Add additional conditions to check whether the reportConfig/measObject/measId is used for CLTM before removing those configurations while executing ReconfigurationWithSync.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Rapporteur RRC for inter-CU and conditional LTM	Ericsson	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.