3GPP TSG-RAN2 Meeting #130	R2-2504459

St.Julians, Malta,  May 19th – 23rd , 2025

Source: 			ZTE Corporation

Title: 	Discussion on functionality management for AI mobility

Agenda item:	8.3.2

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we discussed the LCM and spec impact for AI mobility and achieved the following agreements:

1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management - Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) - The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

In this document, we’d like to share our view on the applicability procedure, inference configuration and report for UE sided model.

- Discussion

### 2.1. Applicability procedure

The applicability procedure in AI-phy beam management is as follows:

- Associated id

In the AI-phy, associated id is used to enable the consistency of network-side additional condition across training and inference. While, for AI-mobility, since the generalization performance is well based on the existing simulation results, some companies think that the network-side additional condition / associated Id is not needed in the AI-mobility. In our view, at least for spatial domain prediction, the cell result is predicted based on a sub set of SSB, which is related to inter beam prediction. The selection of SSB used for the AI input and the beam pattern impacts the prediction accuracy, similar in AI-phy. So, the associated id is needed for at least spatial domain measurement prediction.

Proposal 1: The associated id is needed for at least spatial domain measurement prediction.

### 2.2. Inference configuration

Similar in AI-phy, the network can configure the UE the inference configuration to indicate how to perform inference at the UE side. And in the last meeting, we have agreed that the inference configuration for AI mobility can be based on the RRM measurement object.

- For all use cases

For all use cases (i.e. temporal domain case A, temporal domain case B, frequency domain prediction), the network can indicate which measurement object the UE shall perform inference on. For each measurement object, multiple cells can be detected, in our understanding, there is no need for the UE to perform inference on each detected cell. This is because (1) the motivation for RRM measurement prediction and measurement event prediction is for PCell change and SCell change, the UE do not need to perform inference on the cell(s) cell(s) whose results are not good; (2) the computation resource consumption can be reduced. Considering the computation resource may be different at different UE and the UE side model may be different (e.g. the number of cell(s) for AI input and AI output may be fixed for cluster-based AI model), it is hard to set an exact number of cell that UE shall perform inference on. A simply way is that: for each measurement object, the network configures the minimum number of cell that the UE shall perform inference on (i.e. the number N), and the UE shall at least perform inference on the top-N cell(s) based on the current channel quality. Taking an example of temporal domain prediction case A, if N = 3, the UE shall at least perform inference on the top-3 cell based on the current actual measurement results.

Proposal 2: For AI-mobility, the network can indicate the frequency that the UE shall perform inference on and the minimum number of cell(s) on one frequency that the UE shall perform inference.

- For temporal domain prediction case A

The motivation of temporal domain prediction case A is to improve handover performance via taking measures in advance. In this case, the network can indicate the UE that how early in advance the network expects to obtain the channel quality (i.e. prediction window length) and/or which future time instance that the network is interested in. For the inference configuration in AI-phy BM-Case 2, RAN1 has achieved the following agreement:

For inference, for BM-Case 2 of UE-side model, - The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC. - time gap is [10ms, 20ms, 40ms, 80ms, 160ms] - N = [1, 2, 4, 8]

The same configuration can be used in the temporal domain prediction case A.

Proposal 3: For temporal domain case A, the network can configure the UE the time gap between two consecutive future time instance and the number of future time instance.

Besides, as mentioned above, the motivation of temporal domain prediction case A is to improve handover performance, it seems no need to perform inference if the cell quality of the serving cell is good enough. One solution is up to network implementation, the network determines whether to configure inference configuration for temporal domain case A based on reported measurement results. While, in this way, upon the cell quality of the serving cell gets worse, the applicability reporting procedure are needed, which means the inference cannot be performed immediately. Another simpler solution is that the UE performs inference when a pre-configured condition is met. For example, the network can configure a threshold for the cell quality of the serving cell. If the cell quality of the serving cell is lower than a threshold, the UE performs temporal domain prediction case A. Another example of the pre-configured condition is that: the UE performs temporal domain prediction case A when entering condition for the corresponding event is fulfilled based on the actual measurement results. The details of pre-configured condition can be further discussed.

Proposal 4: For temporal domain case A, the network can configure the UE a condition to determine when to perform inference. FFS the pre-configured condition.

- For temporal domain prediction case B

The motivation of temporal domain prediction case B is to reduce measurement via using predicted results. Considering the measurement reduction results in the performance deterioration, the network should have some control. These controls can be:

- the control over the MRRT, which is defined as skipped measurement time instances / total measurement time instances;

- the control over the skipping pattern, which can be described as the number of continuous time instance for actual measurement and the number of continuous time instance that actual measurement is skipped (i.e. for prediction).

For the first approach, the UE can determine the detailed skipping pattern, and the network is unaware of which time instance is for prediction and which time instance is for actual measurement. So, the network needs to transmit reference signal at each time instance. While, for the second approach, the network can know the exact time instance for actual measurement, and the network can send the reference signaling at only time instances for actual measurement. In this way, besides UE measurement effort, the network reference signaling resource and network energy can be saved.

Proposal 5: For temporal domain case B, the network can configure the UE the skipping pattern, which is defined as the number of continuous time instance for actual measurement and the number of continuous time instance that actual measurement is skipped (i.e. for prediction).

- For frequency domain prediction

For UE side model, only prediction at co-located scenario is supported, i.e. the cell(s) for actual measurement and the cell(s) for prediction needs to be co-located. While, the UE is unaware of spatial relationship of frequency layers. So, the network needs to indicate the UE the measurement results of which frequency can be used for the input of frequency domain prediction AI model.

Proposal 6: Regarding frequency domain prediction, for frequency to be predicted, the network can indicate the measurement results of which frequency can be used for the input of frequency domain prediction AI model.

- For measurement event prediction

For measurement event prediction, the network needs to indicate which event that the UE needs to prediction (e.g. event A3, event A5). Based on the configuration, the UE can predict whether and when the event is triggered. And since the prediction window is sliding, there are multiple time instance that the measurement event can be predicted. Considering that the prediction accuracy is improved with the decreased of the prediction window length, the prediction results (whether the event will be triggered and when the measurement event will be triggered) may be different at different time instance. For example, as shown in the figure below, at time instance 1, the UE predicts that event will be triggered at time instance 6; while at the time instance 2, the UE predicts that event will be triggered at time instance 5. With the help of actual measurement results and shorter prediction distance, the prediction accuracy at time instance 5 is higher. It is redundant and unnecessary if the UE reports predicted results every time that the event is predicted to be triggered. In our view, the network can indicate the UE when to report the predicted results. Specifically, the network can configure the UE to report the measurement results upon:

  - The measurement event is predicted to be triggered;

  - The entering condition of the measurement event is met based on the actual measurement;

  - The entry condition of the measurement event is met for a time duration based on the actual measurement.

In this way, the network implementation becomes more flexible. For example, if the network is concerned about the handover failure caused by false measurement event prediction, the network can configure the UE to report predicted results upon the entry condition of the measurement event is met for a time duration based on the actual measurement results. If the network wants to obtain the best AI gain, the network can configure the UE to report predicted results upon the measurement event is predicted to be triggered.

Figure: An example for measurement event prediction

Proposal 7: Regarding measurement event prediction with temporal domain prediction case A, the network can configure the UE when to report the predicted results.

### 2.2. Inference report

In the last meeting, we have agreed that both event-triggered measurement reporting and periodical measurement reporting are supported. In this sub-section, we’d like to discuss the content of the report in detail.

2.2.1 Periodic reporting

In the legacy periodical measurement report, the UE reports available measurement results upon the expiry of the report interval.

- Temporal domain prediction case A

For temporal domain prediction case A, both the latest actual measurement results and future predicted results are available, the issue to be discussed is that: the measurement results of which time instance the UE needs to report?

In the AI-Phy BM-Case 2, the UE supports to report inference results of N future time instance(s) in one report. In our view, in AI mobility, similar enhancement is also needed in the AI-mobility. And the motivation of the temporal domain case A is to improve handover performance, it is benefitable if the network knows the future measurement results in advance.

Besides, although the UE does not need to report the latest actual measurement results in the AI-phy BM-Case 2, in our understanding, the latest actual measurement result is still useful in the AI-mobility. This is because (1) as a network vender, before making a decision to move the UE to another cell, it is helpful that the network can know the actual measurement results. (2) considering the baseline prediction quantity is RSRP, the UE may be not able to predict future RSRQ/SINR value. With latest actual measurement result in the measurement report, the network can know the channel interference information, which is benefitable for the handover.

Proposal 8: For temporal domain prediction case A, the periodical measurement report includes the latest actual measurement results and predicted results of future time instance.

- Temporal domain prediction case B

For temporal domain prediction case B, depending on the time instance that measurement report is initiated, the type of available measurement results (e.g. historical measurement results from prediction, measurement results of future time instance) are different. For example, if the measurement report is initiated at t1, the latest historical measurement result is from prediction, and there is no future measurement result; if the measurement report is initiated at t3, the latest historical measurement is from actual measurement and the future measurement results are available.

Figure: An example for temporal domain case B

Regarding the content of the measurement report, the simplest way is to always report the latest historical measurement results, regardless that the latest historical measurement result is from prediction or actual measurement, and always not report measurement results of future time instance. While, although the motivation of temporal domain case B is to reduce measurement effort, with future measurement results available, using future measurement results to improve handover performance seems a low hanging fruit. So, in our view, for the temporal domain case B, if the results of future time instance are available, the UE shall also report future measurement results.

Proposal 9: For temporal domain prediction case B, the periodical measurement report includes the latest actual results and predicted results of future time instances (if available).

- Frequency domain prediction

For frequency domain prediction, the UE reports the latest historical results, as legacy. The only difference is that at legacy, the latest historical result is from actual measurement; but in AI-mobility, the latest historical result is from prediction.

Proposal 10: For frequency domain prediction, the measurement report include the latest actual results, as legacy.

2.2.2 Event triggered reporting

In the legacy event-triggered measurement report, the measurement report is sent when the event is triggered and the measurement report includes the latest measurement results of serving cell(s) and optional neighbor cell(s).

- Measurement event prediction with temporal domain prediction case A

For the measurement event prediction with temporal domain prediction case A, the UE can predict the measurement event and reports to the network in advance. In this case, the time instance that the measurement report is sent is earlier than the event-triggered time instance. In order to inform the network when the event is triggered, the event-triggered time needs to be included in the measurement report. Besides, with AI-assisted measurement, the UE can report the predicted measurement results to the network.

Proposal 11: For measurement event prediction with temporal domain prediction case A, the measurement report includes predicted event-triggered time, the latest actual measurement results and predicted results.

- Measurement event prediction with temporal domain prediction case B

For the measurement event prediction with temporal domain prediction case B, in the simulation, we assume that the UE reports predicted measurement event at the time instance it is to be triggered. The motivation is to decouple from Case A. However, for the spec impact study and real implementation, if a measurement event is predicted in advance, the UE can report to the network immediately, no need to wait until the event-triggered time. In this way, the network obtains the measurement report and take measures in advance, which may improve handover performance.

For the content of the measurement report, if the measurement report is sent earlier than event-triggered time (as shown in the figure (a)), the event-triggered time needs to be included in the measurement report, similar to measurement event prediction with temporal case A. If the measurement report is sent no earlier than the event-triggered time (as shown in the figure (b)), the event-triggered time is not needed. And, if future measurement results are available, the UE can include the measurement results of future time instance in the measurement report.

Figure: the example for measurement event prediction with temporal domain case B

Proposal 12: For measurement event prediction with temporal domain prediction case B, the measurement report include:

- Event-triggered time if measurement report is initiated earlier than event-triggered time;

- The latest historical measurement results;

- Measurement result of future time instance (if available)

Proposal 13: For measurement event prediction with temporal domain prediction case B, the measurement report is initiated when the measurement event is predicted to be triggered.

- Measurement event prediction with frequency domain prediction

For event-triggered measurement report with frequency domain prediction, there is no future measurement results available, the UE reports the latest measurement results to the network, as legacy.

Proposal 14: For measurement event prediction with frequency domain prediction, the measurement report includes the latest historical results, as legacy.

- Conclusion and proposals

Proposal 1: The associated id is needed for at least spatial domain measurement prediction.

Proposal 2: For AI-mobility, the network can indicate the frequency that the UE shall perform inference on and the minimum number of cell(s) on one frequency that the UE shall perform inference.

Proposal 3: For temporal domain case A, the network can configure the UE the time gap between two consecutive future time instance and the number of future time instance.

Proposal 4: For temporal domain case A, the network can configure the UE a condition to determine when to perform inference. FFS the pre-configured condition.

Proposal 5: For temporal domain case B, the network can configure the UE the skipping pattern, which is defined as the number of continuous time instance for actual measurement and the number of continuous time instance that actual measurement is skipped (i.e. for prediction).

Proposal 6: Regarding frequency domain prediction, for frequency to be predicted, the network can indicate the measurement results of which frequency can be used for the input of frequency domain prediction AI model.

Proposal 7: Regarding measurement event prediction with temporal domain prediction case A, the network can configure the UE when to report the predicted results.

Proposal 8: For temporal domain prediction case A, the periodical measurement report includes the latest actual measurement results and predicted results of future time instance.

Proposal 9: For temporal domain prediction case B, the periodical measurement report includes the latest actual results and predicted results of future time instances (if available).

Proposal 10: For frequency domain prediction, the measurement report include the latest actual results, as legacy.

Proposal 11: For measurement event prediction with temporal domain prediction case A, the measurement report includes predicted event-triggered time, the latest actual measurement results and predicted results.

Proposal 12: For measurement event prediction with temporal domain prediction case B, the measurement report include:

- Event-triggered time if measurement report is initiated earlier than event-triggered time;

- The latest historical measurement results;

- Measurement result of future time instance (if available)

Proposal 13: For measurement event prediction with temporal domain prediction case B, the measurement report is initiated when the measurement event is predicted to be triggered.

Proposal 14: For measurement event prediction with frequency domain prediction, the measurement report includes the latest historical results, as legacy.

- Reference

- Report of RAN2 129 meeting.