3GPP RAN WG2 Meeting #130	R2-2504465

# St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussion on A-IoT paging

Document for:	Discussion and Decision

1. Introduction

In RAN2, the following agreements were achieved [1]:

- FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to take this into account for their design.

In this contribution, we focus on the issues related to A-IoT paging.

2 Discussion

2.1 (issue 1-1) Subsequent A-IoT Paging procedure

In the running CR, the discussion of subsequent paging is focus on the storing of transaction ID and the paging ID comparison.

For the storing of transaction ID, in order to determine whether to respond the subsequent A-IoT paging message based on the transaction ID received in different paging messages, the device needs to store the previous transaction ID and the corresponding response state for the paging message. The response status corresponding to the transaction ID indicates whether to respond the paging message with the same ID.

Proposal 1: (issue 1-1) The device needs to store the transaction ID in paging message and the corresponding response status.

In the running CR, the rapporteur captured that the device responds the paging message by comparing transaction ID and corresponding response status. For the multi-reader case, there may be a case that the paging ID is different, but the transaction ID is same. In this case, if only considering the transaction ID, the device may respond incorrectly. Therefore, after receiving paging message, the paging ID should be a required condition for responding. If the transaction ID included in the A-IoT Paging message is the same as the stored one, and the device is selected by this paging message according to paging ID, the device will respond to the paging message if the RA for the pervious paging message was unsuccessful.

Proposal 2: (issue 1-1) If the transaction ID is the same as the stored one and the device is selected by this paging message according to paging ID (no ID case and ID available case), the device will respond to the received paging message if pervious RA procedure was unsuccessful.

2.2 (issue 1-1) End of procedure

There are two options for end of procedure:

Option A: ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure;

Option B: terminate the ongoing procedure and respond to the latest request.

The option B cannot work well in multiple readers case and is not available well for forward expansion. The option A is benefit for the procedure conducting and management. Therefore, the discussion in this Tdoc is focus on how to terminate the ongoing procedure and what is the ongoing procedure for the option A.

Proposal 3: (issue 1-1) RAN2 selects the option A for end-of procedure.

What is the ongoing procedure

After the device receives the paging message and this device is selected, the device will initiate the RA procedure and optionally execute the D2R data transmission. Initiating RA procedure means the device needs to select the access occasion and transmit the MSG1. Executing the D2R data transmission means the device receives the command request. Therefore, the ongoing procedure can be as the following:

- The device has initiated a RA procedure and the RA procedure is not completed

- The device has received a command request and the command procedure is not completed.

Proposal 4: (issue 1-1) The ongoing procedure can be as the following:

- The device has initiated a RA procedure and the RA procedure is not completed;

- The device has received a command request and the command procedure is not completed.

End-of the procedure

For the proposal 3, the end-of procedure can be summarised as end-of RA procedure and end-of command procedure. For the inventory and command procedure, the main reason to initiate the RA procedure is to send the command. So, the end-of RA procedure cannot be applied for this scenario. Therefore, the end-of RA procedure is applied for the inventory only case and end-of command procedure is applied for the inventory and command case.

For end-of RA procedure of CBRA, the following R2D message after the Msg3 can be used to end the RA procedure. If the NACK message of Msg3 is received, the RA procedure should be assumed end when the reader explicitly indicates. If the reader is not explicitly indicated, the device needs to wait for the paging message including same transaction ID due to the same service request is not complete.

Proposal 5: (issue 1-1) The RA procedure is assumed completion as the following for CBRA:

- Successful case: The device receives the following R2D message after Msg3 and no NACK is received;

- Unsuccessful case: The device receives an explicit indication for end-of procedure when NACK message has been received.

For end-of command procedure, since there is no NACK for the command response, the following R2D message after the command response can be used to end the command procedure. However, the device may not know whether there is a command request after paging. The RAN3/SA has agreed that the command indication is included in the inventory request. Therefore, it is possible the reader includes this indication in the paging message.

Proposal 6: (issue 1-1) A indication to indicate whether there is a command request is introduced in paging message.

Proposal 7: (issue 1-1) The command procedure is assumed as completion if the device receives the following R2D message after command response.

2.3 (issue 1-3/issue 1-4/issue 1-5) The content in paging message

Paging identifier related

The WID restricts a paging message contains one identifier, and a paging message contains no identifier. RAN2 confirmed “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”. Therefore, one indication can be included in paging message to inform whether the paging ID is included.

Proposal 8: (issue 1-3) The paging message provides the information to indicate whether the paging ID is present.

For the proposal 7, this information can be the agreed paging ID length information. The paging ID length information is always included together with the paging ID field. If the paging ID length is as small as enough, this filed can be used to indicate no ID case. In this case, the paging ID length field will be always present in paging message for ID available case and no ID case.

Considering the length of filter information may be variable, the paging ID length may be 5-8 bits to indicate the actual length of paging ID. In this case, a separate bit used for indicating no ID case as well as indicating the presence of paging ID length field may be less bit overhead.

Proposal 9a: (issue 1-3) If the paging ID length field is always present, the paging ID length filed can be used to indicate whether the paging ID is present.

Proposal 9b: (issue 1-3) If the paging ID length field is not always present, a separate bit is used for indicating the presence of paging ID as well as paging ID length field.

Resources related

For the paging message with multiple R2D messages, the device needs choose which R2D message it accesses. Therefore, the paging message needs to include the number of R2D messages or the number of access occasions. After receiving paging message, the device can randomly select in which R2D messages it accesses.

Proposal 10: (issue 1-4) The paging message includes the number of R2D trigger messages or the number of access occasions.

CFRA related

The scope of the WID confirms the case of a paging message contains one identifier, one identifier for a single device in the paging message may be one use case. In this case, the CFRA will be used, and subsequent paging is not needed. The situation is the same for multiple device IDs with CFRA. ID for service request used to avoiding duplicated responses is not needed for these cases, which may reduce the size of paging message or reduce the complexity and power consumption by avoiding storing ID information obtained in the paging message. Thus, the ID for service request is not mandatory for paging message.

Proposal 11: (issue 1-5) The transaction ID in the paging message can be optional for the CFRA.

2.4 (other open issue) A-IoT R2D messages extensible

RAN2 aims to design Rel-19 A-IoT R2D messages extensible to accommodate devices and features of future release. For the paging message, the current agreed information is as following:

- transaction ID

- paging ID

- paging ID length information

- a number of msg1 resources

- an explicit 1 bit indication to indicate CFRA or CBRA

The main considering to extend paging message is extending one paging ID to multiple paging ID. Therefore, the information to identify whether the multiple identifiers are included in paging message should be introduced.

For the detailed information to identify whether the multiple device IDs is included, the following options can be considered:

- Option 1: an indication to inform whether a further paging ID is included;

- Option 2: the number of paging IDs;

- Option 3: the paging ID and paging ID length is included in the last part of paging message.

For the option 1, each included device ID can correspond to an indication. Based on this indication, the device could derive whether a following device ID is in the paging message. The format of option 1 can be as following, the indication is the A field:

In the Rel-19, the format of paging message only include one pair of information, i.e. paging ID length, paging ID and A.

For the option 2, the device can get the number of device IDs in the paging message. The format of option 2 can be as following, the number of paging IDs is the numofID field:

In the Rel-19, the numofID can occupy 3 bits which is enough to indicate the number of paging IDs.

For the option 3, the device can decode the paging ID based the length of the paging ID. And the paging ID and paging ID length are included in the last part of the paging message.

In the Rel-19, there will be one pair of paging ID length and paging ID. If the multiple IDs case is considered in Rel-20, the paging format can be extended to include further information of the paging ID length and paging ID based the format of Rel-19.

In summary, above options can work well for covering multiple IDs case, RAN2 can discuss which option can be supported.

Proposal 12：RAN2 considers the following options to identify whether the multiple paging IDs are included in the paging message for the forward compatibility:

- Option 1: an indication to inform whether a further paging ID is included;

- Option 2: introduce the number of paging IDs;

- Option 3: the pair of information (i.e. paging ID length and paging ID) is included in the last part of the paging message.

3 Conclusions

Proposal 1: (issue 1-1) The device needs to store the transaction ID in paging message and the corresponding response status.

Proposal 2: (issue 1-1) If the transaction ID is the same as the stored one and the device is selected by this paging message according to paging ID (no ID case and ID available case), the device will respond to the received paging message if pervious RA procedure was unsuccessful.

Proposal 3: (issue 1-1) RAN2 selects the option A for end-of procedure.

Proposal 4: (issue 1-1) The ongoing procedure can be as the following:

- The device has initiated a RA procedure and the RA procedure is not completed;

- The device has received a command request and the command procedure is not completed.

Proposal 5: (issue 1-1) The RA procedure is assumed completion as the following for CBRA:

- Successful case: The device receives the following R2D message after Msg3 and no NACK is received;

- Unsuccessful case: The device receives an explicit indication for end-of procedure when NACK message has been received.

Proposal 6: (issue 1-1) A indication to indicate whether there is a command request is introduced in paging message.

Proposal 7: (issue 1-1) The command procedure is assumed as completion if the device receives the following R2D message after command response.

Proposal 8: (issue 1-3) The paging message provides the information to indicate whether the paging ID is present.

Proposal 9a: (issue 1-3) If the paging ID length field is always present, the paging ID length filed can be used to indicate whether the paging ID is present.

Proposal 9b: (issue 1-3) If the paging ID length field is not always present, a separate bit is used for indicating the presence of paging ID as well as paging ID length field.

Proposal 10: (issue 1-4) The paging message includes the number of R2D trigger messages or the number of access occasions.

Proposal 11: (issue 1-5) The transaction ID in the paging message can be optional for the CFRA.

Proposal 12：RAN2 considers the following options to identify whether the multiple paging IDs are included in the paging message for the forward compatibility:

- Option 1: an indication to inform whether a further paging ID is included;

- Option 2: introduce the number of paging IDs;

- Option 3: the pair of information (i.e. paging ID length and paging ID) is included in the last part of the paging message.

4. References

[1]	RAN2 #129bis meeting report